3GPP TSG-RAN5 Meeting #99	R5-232667

# Incheon, Korea (Republic Of), 22nd May 2023 – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0277 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to RRM applicability rules and test optimization - 38.522 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN5#98 the proposal introduced in R5-231871 was endorsed. The agreement consisted in categorizing the RRM test cases based on whether the requirements tested by the SA and/or EN-DC counterparts were similar or not. In RAN5#99 discussion paper R5-232668 is introducing an initial test case grouping list including the categorization of many Group1 and Group3 test cases. This CR is implementing this change in 38.522. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated the Additional Information section of the RRM test cases categorized as either Group1 or Group3 in the above referenced discussion paper. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM test case optimization would be missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related discussion paper R5-232668 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## 4.2	RRM conformance test cases

Table 4.2-1: Applicability of RRM EN-DC FR1 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 4.3 | RRC_CONNECTED state mobility |  |  |  |  |  |
| 4.3.2 | RRC connection mobility control |  |  |  |  |  |
| 4.3.2.2 | Random access |  |  |  |  |  |
| 4.3.2.2.1 | EN-DC FR1 contention based random access | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.3.2.2.1 has been tested. | 2Rx 4Rx |
| 4.3.2.2.2 | EN-DC FR1 non-contention based random access | Rel-15 | C030 | UEs supporting EN-DC FR1 and CSI-RS based PRACH | Test execution not necessary if the SA counterpart test 6.3.2.2.2 has been tested. | 2Rx 4Rx |
| 4.3.2.2.3 | EN-DC FR1 2-step contention based random access | Rel-16 | C157 | UEs supporting EN-DC FR1 and 2-step RACH | Test execution not necessary if the SA counterpart test 6.3.2.2.3 has been tested. | 2Rx 4Rx |
| 4.3.2.2.4 | EN-DC FR1 2-step non-contention based random access | Rel-16 | C158 | UEs supporting EN-DC FR1 and 2-step RACH | Test execution not necessary if the SA counterpart test 6.3.2.2.4 has been tested. | 2Rx 4Rx |
| 4.4 | Timing |  |  |  |  |  |
| 4.4.1 | UE Transmit Timing |  |  |  |  |  |
| 4.4.1.1 | EN-DC FR1 UE transmit timing accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.4.1.1 has been tested. | 2Rx 4Rx |
| 4.4.2 | UE timer accuracy |  |  |  |  |  |
| 4.4.3 | Timing Advance |  |  |  |  |  |
| 4.4.3.1 | EN-DC FR1 timing advance adjustment accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.4.3.1 has been tested. | 2Rx 4Rx |
| 4.5 | Signalling characteristics |  |  |  |  |  |
| 4.5.1 | Radio link monitoring |  |  |  |  |  |
| 4.5.1.1 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.5.1.1 has been tested. | 2Rx 4Rx |
| 4.5.1.2 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.5.1.2 has been tested. | 2Rx 4Rx |
| 4.5.1.3 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle | Test execution not necessary if the SA counterpart test 6.5.1.3 has been tested. | 2Rx 4Rx |
| 4.5.1.4 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle | Test execution not necessary if the SA counterpart test 6.5.1.4 has been tested. | 2Rx 4Rx |
| 4.5.1.5 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C038 | UEs supporting EN-DC FR1 and CSI-RS-based RLM | Test execution not necessary if the SA counterpart test 6.5.1.5 has been tested. | 2Rx 4Rx |
| 4.5.1.6 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C038 | UEs supporting EN-DC FR1 and CSI-RS-based RLM | Test execution not necessary if the SA counterpart test 6.5.1.6 has been tested. | 2Rx 4Rx |
| 4.5.1.7 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C038a | UEs supporting EN-DC FR1, CSI-RS-based RLM and long DRX cycle | Test execution not necessary if the SA counterpart test 6.5.1.7 has been tested. | 2Rx 4Rx |
| 4.5.1.8 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C038a | UEs supporting EN-DC FR1, CSI-RS-based RLM and long DRX cycle | Test execution not necessary if the SA counterpart test 6.5.1.8 has been tested. | 2Rx 4Rx |
| 4.5.2 | Interruption |  |  |  |  |  |
| 4.5.2.1 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.5.2.2 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.5.2.3 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |
| 4.5.2.4 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |
| 4.5.2.5 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | Rel-15 | C068 | UEs supporting EN-DC FR1 and 2DL CA in E-UTRA |  | 2Rx 4Rx |
| 4.5.2.6 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | Rel-15 | C068 | UEs supporting EN-DC FR1 and 2DL CA in E-UTRA |  | 2Rx 4Rx |
| 4.5.3 | Scell activation and deactivation delay |  |  |  |  |  |
| 4.5.3.1 | EN-DC FR1 Scell activation and deactivation of known Scell in non-DRX for 160ms Scell measurement cycle | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |
| 4.5.3.2 | EN-DC FR1 Scell activation and deactivation of known Scell in non-DRX for 640ms Scell measurement cycle | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |
| 4.5.3.3 | EN-DC FR1 Scell activation and deactivation of unknown Scell in non-DRX | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |
| 4.5.3.5 | EN-DC FR1 Direct Scell activation at Scell addition of known Scell | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |
| 4.5.4 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |
| 4.5.4.1 | EN-DC FR1 UE UL carrier RRC reconfiguration delay | Rel-15 | C032 | UEs supporting EN-DC FR1 and SUL | Test execution not necessary if the SA counterpart test 6.5.4.1 has been tested. | 2Rx 4Rx |
| 4.5.5 | Beam failure detection and link recovery procedures |  |  |  |  |  |
| 4.5.5.1 | EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C082 | UEs supporting EN-DC FR1 and link recovery | Test execution not necessary if the SA counterpart test 6.5.5.1 has been tested. | 2Rx 4Rx |
| 4.5.5.2 | EN-DC FR1 SSB-based beam failure detection and link recovery in DRX | Rel-15 | C082a | UEs supporting EN-DC FR1 and long DRX cycle and link recovery | Test execution not necessary if the SA counterpart test 6.5.5.2 has been tested. | 2Rx 4Rx |
| 4.5.5.3 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | Rel-15 | C083 | UEs supporting EN-DC FR1 and CSI-RS-based RLM and link recovery | Test execution not necessary if the SA counterpart test 6.5.5.3 has been tested. | 2Rx 4Rx |
| 4.5.5.4 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | Rel-15 | C083a | UEs supporting EN-DC FR1 and long DRX cycle and CSI-RS-based RLM and link recovery | Test execution not necessary if the SA counterpart test 6.5.5.4 has been tested. | 2Rx 4Rx |
| 4.5.5.5 | EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | Rel-16 | C175 | UEs supporting EN-DC FR1 and CSI-RS-based RLM and SSB link recovery | Test execution not necessary if the SA counterpart test 6.5.5.5 has been tested. | 2Rx 4Rx |
| 4.5.5.6 | EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | Rel-16 | C176 | UEs supporting EN-DC FR1 and long DRX cycle and CSI-RS-based RLM and SSB link recovery | Test execution not necessary if the SA counterpart test 6.5.5.6 has been tested. | 2Rx 4Rx |
| 4.5.6 | Active BWP switch delay |  |  |  |  |  |
| 4.5.6.1 | DCI-based and timer-based active BWP switch |  |  |  |  |  |
| 4.5.6.1.1 | EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | Rel-15 | C065 | UEs supporting EN-DC FR1 and (DCI and timer based active BWP switching delay Type1 or Type2) and (Support of BWP adaptation upto2 or upto4) |  | 2Rx 4Rx |
| 4.5.6.1.2 | EN-DC FR1 DCI-based DL active BWP switch with Scell in non-DRX in synchronous EN-DC | Rel-15 | C065a | UEs supporting EN-DC FR1 and (DCI and timer based active BWP switching delay Type1 or Type2) and (Support of BWP adaptation upto2 or upto4) and 2DL CA |  | 2Rx 4Rx |
| 4.5.6.2 | RRC-based active BWP switch |  |  |  |  |  |
| 4.5.6.2.1 | EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | Rel-15 | C065b | UEs supporting EN-DC FR1 and (Support of BWP adaptation upto2 or upto4) |  | 2Rx 4Rx |
| 4.5.7 | PSCell addition and release delay |  |  |  |  |  |
| 4.5.7.1 | EN-DC FR1 addition and release delay of known PSCell | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.5.8 | UL switching |  |  |  |  |  |
| 4.5.8.1 | EN-DC FR1 interruptions at switching between two uplink carriers | Rel-16 | C126a | UEs supporting EN-DC and dynamic UL Tx switching in case of inter-band EN-DC |  | 2Rx 4Rx |
| 4.6 | Measurement procedures |  |  |  |  |  |
| 4.6.1 | Intra-frequency measurements |  |  |  |  |  |
| 4.6.1.1 | EN-DC FR1 event-triggered reporting without gap in non-DRX | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.6.1.1 has been tested. | 2Rx 4Rx |
| 4.6.1.2 | EN-DC FR1 event-triggered reporting without gap in DRX | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle | Test execution not necessary if the SA counterpart test 6.6.1.2 has been tested. | 2Rx 4Rx |
| 4.6.1.3 | EN-DC FR1 event-triggered reporting with gap in non-DRX | Rel-15 | C042 | UEs supporting EN-DC FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction | Test execution not necessary if the SA counterpart test 6.6.1.3 has been tested. | 2Rx 4Rx |
| 4.6.1.4 | EN-DC FR1 event-triggered reporting with gap in DRX | Rel-15 | C042a | UEs supporting EN-DC FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle | Test execution not necessary if the SA counterpart test 6.6.1.4 has been tested. | 2Rx 4Rx |
| 4.6.1.5 | EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | Rel-15 | C021b | UEs supporting EN-DC FDD FR1 | Test execution not necessary if the SA counterpart test 6.6.1.5 has been tested. | 2Rx 4Rx |
| 4.6.1.6 | EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | Rel-15 | C042b | UEs supporting EN-DC FDD FR1 and CSI-RS based RLM and BWP operation without bandwidth restriction | Test execution not necessary if the SA counterpart test 6.6.1.6 has been tested. | 2Rx 4Rx |
| 4.6.1.7 | EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Rel-15 | C097 | UEs supporting EN-DC FR1 and long DRX cycle and measurement enhancements in HST | Test execution not necessary if the SA counterpart test 6.6.1.7 has been tested. | 2Rx 4Rx |
| 4.6.1.8 | EN-DC FR1 event triggered reporting cell without SSB time index detection in DRX for UE configured with highSpeedMeasCA-Scell-r17 | Rel-16 | C021c | UEs supporting EN-DC FR1 and CA measurement enhancements in HST | NOTE 1 Test execution not necessary if the SA counterpart test 6.6.1.8 has been tested. | 2Rx 4Rx |
| 4.6.2 | Inter-frequency measurements |  |  |  |  |  |
| 4.6.2.1 | EN-DC FR1-FR1 event-triggered reporting in non-DRX | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.6.2.1 has been tested. | 2Rx 4Rx |
| 4.6.2.2 | EN-DC FR1-FR1 event-triggered reporting in DRX | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle | Test execution not necessary if the SA counterpart test 6.6.2.2 has been tested. | 2Rx 4Rx |
| 4.6.2.5 | EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.6.2.5 has been tested. | 2Rx 4Rx |
| 4.6.2.6 | EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle | Test execution not necessary if the SA counterpart test 6.6.2.6 has been tested. | 2Rx 4Rx |
| 4.6.2.9 | EN-DC FR1-FR1 event triggered reporting without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 | Rel-16 | C021d | UEs supporting EN-DC FR1 and inter-frequency measurement enhancements in HST | NOTE 1 Test execution not necessary if the SA counterpart test 6.6.2.12 has been tested. | 2Rx 4Rx |
| 4.6.4 | L1-RSRP for beam reporting |  |  |  |  |  |
| 4.6.4.1 | EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.6.4.1 has been tested. | 2Rx 4Rx |
| 4.6.4.2 | EN-DC FR1 SSB-based L1-RSRP measurement in DRX | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle | Test execution not necessary if the SA counterpart test 6.6.4.2 has been tested. | 2Rx 4Rx |
| 4.6.4.3 | EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.6.4.3 has been tested. | 2Rx 4Rx |
| 4.6.4.4 | EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle | Test execution not necessary if the SA counterpart test 6.6.4.4 has been tested. | 2Rx 4Rx |
| 4.6.4.5 | EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Rel-15 | C098 | UEs supporting EN-DC FR1, long DRX cycle and intra-NR measurement enhancement in HST | Test execution not necessary if the SA counterpart test 6.6.4.5 has been tested. | 2Rx 4Rx |
| 4.6.7 | L1-SINR for beam reporting |  |  |  |  |  |
| 4.6.7.1 | EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX | Rel-16 | C141 | UEs supporting EN-DC FR1 and L1-SINR measurement based on CSI-RS as CMR without dedicated IMR configured |  | 2Rx 4Rx |
| 4.6.7.2 | EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX | Rel-16 | C142 | UEs supporting EN-DC FR1 and long DRX cycle and L1-SINR measurement based on SSB as CMR and dedicated CSI-IM as IMR |  | 2Rx 4Rx |
| 4.6.7.3 | EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX | Rel-16 | C143 | UEs supporting EN-DC FR1 and long DRX cycle and L1-SINR measurement based on CSI-RS as CMR and dedicated CSI-RS as IMR |  | 2Rx 4Rx |
| 4.7 | Measurement performance requirements |  |  |  |  |  |
| 4.7.1 | SS-RSRP |  |  |  |  |  |
| 4.7.1.1 | Intra-frequency measurements |  |  |  |  |  |
| 4.7.1.1.1 | EN-DC FR1 SS-RSRP absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.7.1.1.1 has been tested. | 2Rx 4Rx |
| 4.7.1.1.2 | EN-DC FR1 SS-RSRP relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.7.1.1.2 has been tested. | 2Rx 4Rx |
| 4.7.1.2 | Inter-frequency measurements |  |  |  |  |  |
| 4.7.1.2.1 | EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.7.1.2.1 has been tested. | 2Rx 4Rx |
| 4.7.1.2.2 | EN-DC FR1-FR1 SS-RSRP relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.7.1.2.2 has been tested. | 2Rx 4Rx |
| 4.7.2 | SS-RSRQ |  |  |  |  |  |
| 4.7.2.1 | EN-DC FR1 SS-RSRQ measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.7.2.1 has been tested. | 2Rx 4Rx |
| 4.7.2.2.1 | EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.7.2.2.1 has been tested. | 2Rx 4Rx |
| 4.7.2.2.2 | EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.7.2.2.2 has been tested. | 2Rx 4Rx |
| 4.7.3 | SS-SINR |  |  |  |  |  |
| 4.7.3.1 | EN-DC FR1 SS-SINR measurement accuracy | Rel-15 | C035 | UEs supporting EN-DC FR1 and SS-SINR-meas | Test execution not necessary if the SA counterpart test 6.7.3.1 has been tested. | 2Rx 4Rx |
| 4.7.3.2.1 | EN-DC FR1-FR1 SS-SINR absolute measurement accuracy | Rel-15 | C035 | UEs supporting EN-DC FR1 and SS-SINR-meas | Test execution not necessary if the SA counterpart test 6.7.3.2.1 has been tested. | 2Rx 4Rx |
| 4.7.3.2.2 | EN-DC FR1-FR1 SS-SINR relative measurement accuracy | Rel-15 | C035 | UEs supporting EN-DC FR1 and SS-SINR-meas | Test execution not necessary if the SA counterpart test 6.7.3.2.2 has been tested. | 2Rx 4Rx |
| 4.7.4 | L1-RSRP |  |  |  |  |  |
| 4.7.4.1.1 | EN-DC FR1 SSB-based L1-RSRP absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.7.4.1.1 has been tested. | 2Rx 4Rx |
| 4.7.4.1.2 | EN-DC FR1 SSB-based L1-RSRP relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.7.4.1.2 has been tested. | 2Rx 4Rx |
| 4.7.4.2.1 | EN-DC FR1 CSI-RS-based L1-RSRP absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.7.4.2.1 has been tested. | 2Rx 4Rx |
| 4.7.4.2.2 | EN-DC FR1 CSI-RS-based L1-RSRP relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if the SA counterpart test 6.7.4.2.2 has been tested. | 2Rx 4Rx |
| 4.7.5 | SFTD |  |  |  |  |  |
| 4.7.5.1 | EN-DC FR1 SFTD measurement accuracy | Rel-15 | C043 | UEs supporting EN-DC FR1 and SFTD measurements between E-UTRA Pcell and NR PSCell |  | 2Rx 4Rx |
| 4.7.7 | L1-SINR |  |  |  |  |  |
| 4.7.7.1.1 | EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy | Rel-16 | C135 | UEs supporting EN-DC FR1 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured | Test execution not necessary if the SA counterpart test 6.7.9.1.1 has been tested. | 2Rx 4Rx |
| 4.7.7.1.2 | EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy | Rel-16 | C135 | UEs supporting EN-DC FR1 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured | Test execution not necessary if the SA counterpart test 6.7.9.1.2 has been tested. | 2Rx 4Rx |
| 4.7.7.2 | EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Rel-16 | C136 | UEs supporting EN-DC FR1 and L1-SINR-measurement based on SSB as CMR and dedicated CSI-IM as IMR | Test execution not necessary if the SA counterpart test 6.7.9.2 has been tested. | 2Rx 4Rx |
| 4.7.7.3.1 | EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Rel-16 | C137 | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR | Test execution not necessary if the SA counterpart test 6.7.9.3.1 has been tested. | 2Rx 4Rx |
| 4.7.7.3.2 | EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy | Rel-16 | C137 | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR | Test execution not necessary if the SA counterpart test 6.7.9.3.2 has been tested. | 2Rx 4Rx |
| 4A | NE-DC with all NR cells in FR1 |  |  |  |  |  |
| 4A.1 | Signalling characteristics |  |  |  |  |  |
| 4A.1.1 | E-UTRA PSCell addition |  |  |  |  |  |
| 4A.1.1.1 | NE-DC FR1 addition and release delay of known PSCell | Rel-15 | FFS | FFS | NOTE 1 | 2Rx 4Rx |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Test X refers to the corresponding Sub-Test as defined in TS 38.533 [5]. |  |  |  |  |  |  |

<< Skipped sections >>

Table 4.2-3: Applicability of RRM NR SA FR1 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 6.1 | RRC_IDLE state mobility |  |  |  |  |  |
| 6.1.1 | NR cell re-selection |  |  |  |  |  |
| 6.1.1.1 | NR SA FR1 cell re-selection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.1.1.2 | NR SA FR1-FR1 cell re-selection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.1.1.3 | NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx |
| 6.1.1.4 | NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx |
| 6.1.1.5 | NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx |
| 6.1.1.6 | NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx |
| 6.1.1.7 | NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | Rel-15 | C052 | UEs supporting 5GS NR SA FR1 and measurement enhancements in HST |  | 2Rx 4Rx |
| 6.1.1.8 | NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17 | Rel-16 | C052a | UEs supporting 5GS NR SA FR1 and inter-freq measurement enhancements in HST | NOTE 1 | 2Rx 4Rx |
| 6.1.2 | NR – E-UTRA cell re-selection |  |  |  |  |  |
| 6.1.2.1 | NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 6.1.2.2 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 6.1.2.3 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C094 | UEs supporting 5GS NR SA FR1 and E-UTRA and relaxed RRM measurement |  | 2Rx 4Rx |
| 6.1.2.4 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C094 | UEs supporting 5GS NR SA FR1 and E-UTRA and relaxed RRM measurement |  | 2Rx 4Rx |
| 6.1.2.5 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 | Rel-15 | C025b | UEs supporting 5GS NR SA FR1 and E-UTRA and E-UTRA inter-RAT measurement enhancements in HST |  | 2Rx 4Rx |
| 6.2 | RRC_INACTIVE state mobility |  |  |  |  |  |
| 6.3 | RRC_CONNECTED state mobility |  |  |  |  |  |
| 6.3.1 | Handover |  |  |  |  |  |
| 6.3.1.1 | NR SA FR1 handover with known target cell | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.1.2 | NR SA FR1 handover with unknown target cell | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.1.3 | NR SA FR1-FR1 handover with unknown target cell | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.1.4 | NR SA FR1 – E-UTRA handover with known target cell | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 6.3.1.5 | NR SA FR1 – E-UTRA handover with unknown target cell | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 6.3.1.6 | NR SA FR1 – UTRAN FDD handover with known target cell | Rel-16 | C096 | UEs supporting 5GS NR SA FR1 and UTRAN FDD |  | 2Rx 4Rx |
| 6.3.1.7 | NR SA FR1 synchronous DAPS handover | Rel-16 | C101 | UEs supporting 5GS NR SA FR1 and intra-frequency DAPS handover |  | 2Rx 4Rx |
| 6.3.1.8 | NR SA FR1 asynchronous DAPS handover | Rel-16 | C102 | UEs supporting 5GS NR SA FR1 and intra-frequency async DAPS handover |  | 2Rx 4Rx |
| 6.3.1.9 | NR SA FR1 Intra-band inter-frequency synchronous DAPS handover | Rel-16 | C107 | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover |  | 2Rx 4Rx |
| 6.3.1.10 | NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover | Rel-16 | C108 | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover |  | 2Rx 4Rx |
| 6.3.1.11 | NR SA FR1 Inter-band inter-frequency synchronous DAPS handover | Rel-16 | C107 | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover | For test configuration 1, 2, 4, 5, 9 | 2Rx 4Rx |
|  |  |  | C109 | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover and supporting different SCSs in source Pcell and inter-frequency target Pcell | For test configuration 3, 6, 7, 8 | 2Rx 4Rx |
| 6.3.1.12 | NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover | Rel-16 | C108 | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover | For test configuration 1, 2, 4, 5, 9 | 2Rx 4Rx |
|  |  |  | C110 | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover and supporting different SCSs in source Pcell and inter-frequency target Pcell | For test configuration 3, 6, 7, 8 | 2Rx 4Rx |
| 6.3.2 | RRC connection mobility control |  |  |  |  |  |
| 6.3.2.1 | RRC re-establishment |  |  |  |  |  |
| 6.3.2.1.1 | NR SA FR1 RRC re-establishment | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.2.1.2 | NR SA FR1 - FR1 RRC re-establishment | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.2.1.3 | NR SA FR1 RRC re-establishment without serving cell timing | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.2.2 | Random access |  |  |  |  |  |
| 6.3.2.2.1 | NR SA FR1 contention based random access | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.3.2.2.1 has been tested. | 2Rx 4Rx |
| 6.3.2.2.2 | NR SA FR1 non-contention based random access | Rel-15 | C029 | UEs supporting 5GS NR SA FR1 and CSI-RS based PRACH | Test execution not necessary if the NSA counterpart test 4.3.2.2.2 has been tested. | 2Rx 4Rx |
| 6.3.2.2.3 | NR SA FR1 2-step contention based random access | Rel-16 | C159 | UEs supporting 5GS NR SA FR1and 2-step RACH | Test execution not necessary if the NSA counterpart test 4.3.2.2.3 has been tested. | 2Rx 4Rx |
| 6.3.2.2.4 | NR SA FR1 2-step non-contention based random access | Rel-16 | C159 | UEs supporting 5GS NR SA FR1and 2-step RACH | Test execution not necessary if the NSA counterpart test 4.3.2.2.4 has been tested. | 2Rx 4Rx |
| 6.3.2.3 | RRC connection release with redirection |  |  |  |  |  |
| 6.3.2.3.1 | NR SA FR1 RRC connection release with redirection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.2.3.2 | NR SA FR1 - E-UTRA RRC connection release with redirection | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 6.3.3 | Conditional handover |  |  |  |  |  |
| 6.3.3.1 | NR SA FR1 conditional handover | Rel-16 | C105 | UEs supporting 5GS NR SA FR1 and Conditional handover |  | 2Rx 4Rx |
| 6.3.3.2 | NR SA FR1-FR1 conditional handover | Rel-16 | C105 | UEs supporting 5GS NR SA FR1 and Conditional handover |  | 2Rx 4Rx |
| 6.4 | Timing |  |  |  |  |  |
| 6.4.1 | UE transmit timing |  |  |  |  |  |
| 6.4.1.1 | NR SA FR1 UE transmit timing accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.4.1.1 has been tested. | 2Rx 4Rx |
| 6.4.2 | UE timer accuracy |  |  |  |  |  |
| 6.4.3 | Timing advance |  |  |  |  |  |
| 6.4.3.1 | NR SA FR1 timing advance adjustment accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.4.3.1 has been tested. | 2Rx 4Rx |
| 6.5 | Signalling characteristics |  |  |  |  |  |
| 6.5.1 | Radio Link Monitoring |  |  |  |  |  |
| 6.5.1.1 | NR SA FR1 radio link monitoring out-of-sync test for Pcell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.5.1.1 has been tested. | 2Rx 4Rx |
| 6.5.1.2 | NR SA FR1 radio link monitoring in-sync test for Pcell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.5.1.2 has been tested. | 2Rx 4Rx |
| 6.5.1.3 | NR SA FR1 radio link monitoring out-of-sync test for Pcell configured with SSB-based RLM RS in DRX mode | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if the NSA counterpart test 4.5.1.3 has been tested. | 2Rx 4Rx |
| 6.5.1.4 | NR SA FR1 radio link monitoring in-sync test for Pcell configured with SSB-based RLM RS in DRX mode | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if the NSA counterpart test 4.5.1.4 has been tested. | 2Rx 4Rx |
| 6.5.1.5 | NR SA FR1 radio link monitoring out-of-sync test for Pcell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C037 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM | Test execution not necessary if the NSA counterpart test 4.5.1.5 has been tested. | 2Rx 4Rx |
| 6.5.1.6 | NR SA FR1 radio link monitoring in-sync test for Pcell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C037 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM | Test execution not necessary if the NSA counterpart test 4.5.1.6 has been tested. | 2Rx 4Rx |
| 6.5.1.7 | NR SA FR1 radio link monitoring out-of-sync test for Pcell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C037a | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM and long DRX cycle | Test execution not necessary if the NSA counterpart test 4.5.1.7 has been tested. | 2Rx 4Rx |
| 6.5.1.8 | NR SA FR1 radio link monitoring in-sync test for Pcell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C037a | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM and long DRX cycle | Test execution not necessary if the NSA counterpart test 4.5.1.8 has been tested. | 2Rx 4Rx |
| 6.5.2 | Interruption |  |  |  |  |  |
| 6.5.2.1 | NR SA FR1 interruptions during measurements on deactivated NR SCC | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |
| 6.5.3 | Scell activation and deactivation delay |  |  |  |  |  |
| 6.5.3.1 | NR SA FR1 Scell activation and deactivation of known Scell in non-DRX for 160ms Scell measurement cycle | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |
| 6.5.3.2 | NR SA FR1 Scell activation and deactivation of known Scell in non-DRX for 640ms Scell measurement cycle | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |
| 6.5.3.3 | NR SA FR1 Scell activation and deactivation of unknown Scell in non-DRX | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |
| 6.5.3.4 | NR SA FR1 Direct Scell activation of known Scell | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |
| 6.5.3.5 | NR SA FR1 Direct Scell activation of known Scell at handover | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |
| 6.5.4 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |
| 6.5.4.1 | NR SA FR1 UE UL carrier RRC reconfiguration delay | Rel-15 | C002 | UEs supporting 5GS NR SA FR1 and SUL | Test execution not necessary if the NSA counterpart test 4.5.4.1 has been tested. | 2Rx 4Rx |
| 6.5.5 | Link recovery procedures |  |  |  |  |  |
| 6.5.5.1 | NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C084 | UEs supporting 5GS NR SA FR1 and link recovery | Test execution not necessary if the NSA counterpart test 4.5.5.1 has been tested. | 2Rx 4Rx |
| 6.5.5.2 | NR SA FR1 SSB-based beam failure detection and link recovery in DRX | Rel-15 | C084a | UEs supporting 5GS NR SA FR1 and long DRX cycle and link recovery | Test execution not necessary if the NSA counterpart test 4.5.5.2 has been tested. | 2Rx 4Rx |
| 6.5.5.3 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | Rel-15 | C085 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and link recovery | Test execution not necessary if the NSA counterpart test 4.5.5.3 has been tested. | 2Rx 4Rx |
| 6.5.5.4 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | Rel-15 | C085a | UEs supporting 5GS NR SA FR1 and long DRX cycle and CSI-RS-based RLM and link recovery | Test execution not necessary if the NSA counterpart test 4.5.5.4 has been tested. | 2Rx 4Rx |
| 6.5.5.5 | NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | Rel-16 | C173 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and SSB-based link recovery on Scell | Test execution not necessary if the NSA counterpart test 4.5.5.5 has been tested. | 2Rx 4Rx |
| 6.5.5.6 | NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | Rel-16 | C174 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and SSB-based link recovery on Scell and long DRX cycle | Test execution not necessary if the NSA counterpart test 4.5.5.6 has been tested. | 2Rx 4Rx |
| 6.5.6 | Active BWP switch delay |  |  |  |  |  |
| 6.5.6.1 | DCI-based and timer-based active BWP switch |  |  |  |  |  |
| 6.5.6.1.1 | NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | Rel-15 | C066a | UEs supporting 5GS NR SA FR1 and (DCI and timer based active BWP switching delay type1 or type2) and (Support of BWP adaptation upto2 or upto4) and 2DL CA |  | 2Rx 4Rx |
| 6.5.6.1.2 | NR SA FR1 DCI-based DL active BWP switch in non-DRX | Rel-15 | C066 | UEs supporting 5GS NR SA FR1 and (DCI and timer based active BWP switching delay type1 or type2) and (Support of BWP adaptation upto2 or upto4) |  | 2Rx 4Rx |
| 6.5.6.2 | RRC-based active BWP switch |  |  |  |  |  |
| 6.5.6.2.1 | NR SA FR1 RRC-based DL active BWP switch in non-DRX | Rel-15 | C066b | UEs supporting 5GS NR SA FR1 and (Support of BWP adaptation upto2 or upto4) |  | 2Rx 4Rx |
| 6.5.7 | DL interruptions at switching between two uplink carriers |  |  |  |  |  |
| 6.5.7.1 | NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA | Rel-16 | C051 | UEs supporting 5GS NR SA FR1 and Inter-band CA (2UL CA)and dynamic UL Tx switching |  | 2Rx 4Rx |
| 6.5.7.2 | NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA | Rel-16 | C051 | UEs supporting 5GS NR SA FR1 and Inter-band CA (2UL CA)and dynamic UL Tx switching |  | 2Rx 4Rx |
| 6.5.8 | UE specific CBW change |  |  |  |  |  |
| 6.5.8.1 | UE specific CBW change on Pcell in FR1 in non-DRX | Rel-16 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.6 | Measurement procedures |  |  |  |  |  |
| 6.6.1 | Intra-frequency measurements |  |  |  |  |  |
| 6.6.1.1 | NR SA FR1 event-triggered reporting without gap in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.6.1.1 has been tested. | 2Rx 4Rx |
| 6.6.1.2 | NR SA FR1 event-triggered reporting without gap in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if the NSA counterpart test 4.6.1.2 has been tested. | 2Rx 4Rx |
| 6.6.1.3 | NR SA FR1 event-triggered reporting with gap in non-DRX | Rel-15 | C041 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction | Test execution not necessary if the NSA counterpart test 4.6.1.3 has been tested. | 2Rx 4Rx |
| 6.6.1.4 | NR SA FR1 event-triggered reporting with gap in DRX | Rel-15 | C041a | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle | Test execution not necessary if the NSA counterpart test 4.6.1.4 has been tested. | 2Rx 4Rx |
| 6.6.1.5 | NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading | Rel-15 | C024 | UEs supporting 5GS NR FDD SA FR1 | Test execution not necessary if the NSA counterpart test 4.6.1.5 has been tested. | 2Rx 4Rx |
| 6.6.1.6 | NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading | Rel-15 | C041b | UEs supporting 5GS NR FDD SA FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction | Test execution not necessary if the NSA counterpart test 4.6.1.6 has been tested. | 2Rx 4Rx |
| 6.6.1.7 | NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Rel-15 | C052 | UEs supporting 5GS NR SA FR1 and measurement enhancements in HST | Test execution not necessary if the NSA counterpart test 4.6.1.7 has been tested. | 2Rx 4Rx |
| 6.6.1.8 | NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17 | Rel-16 | C052b | UEs supporting 5GS NR SA FR1 and CA measurement enhancements in HST | NOTE 1 Test execution not necessary if the NSA counterpart test 4.6.1.8 has been tested. | 2Rx 4Rx |
| 6.6.2 | Inter-frequency measurements |  |  |  |  |  |
| 6.6.2.1 | NR SA FR1-FR1 event-triggered reporting in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.6.2.1 has been tested. | 2Rx 4Rx |
| 6.6.2.2 | NR SA FR1-FR1 event-triggered reporting in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if the NSA counterpart test 4.6.2.2 has been tested. | 2Rx 4Rx |
| 6.6.2.5 | NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.6.2.5 has been tested. | 2Rx 4Rx |
| 6.6.2.6 | NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if the NSA counterpart test 4.6.2.6 has been tested. | 2Rx 4Rx |
| 6.6.2.12 | NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 | Rel-16 | C052c | UEs supporting 5GS NR SA FR1 and inter-freq measurement enhancements in HST | NOTE 1 Test execution not necessary if the NSA counterpart test 4.6.2.9 has been tested. | 2Rx 4Rx |
| 6.6.3 | Inter-RAT measurements |  |  |  |  |  |
| 6.6.3.1 | NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRAN |  | 2Rx 4Rx |
| 6.6.3.2 | NR SA FR1 – E-UTRAN event-triggered reporting in DRX | Rel-15 | C025a | UEs supporting 5GS NR SA FR1, E-UTRAN and long DRX cycle |  | 2Rx 4Rx |
| 6.6.3.3 | NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 | Rel-15 | C025c | UEs supporting 5GS NR SA FR1 and E-UTRAN, long DRX cycle and E-UTRA inter-RAT measurement enhancements in HST |  | 2Rx 4Rx |
| 6.6.4 | L1-RSRP measurement for beam reporting |  |  |  |  |  |
| 6.6.4.1 | NR SA FR1 SSB-based L1-RSRP measurement in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.6.4.1 has been tested. | 2Rx 4Rx |
| 6.6.4.2 | NR SA FR1 SSB-based L1-RSRP measurement in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if the NSA counterpart test 4.6.4.2 has been tested. | 2Rx 4Rx |
| 6.6.4.3 | NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.6.4.3 has been tested. | 2Rx 4Rx |
| 6.6.4.4 | NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if the NSA counterpart test 4.6.4.4 has been tested. | 2Rx 4Rx |
| 6.6.4.5 | NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Rel-15 | C001f | UEs supporting 5GS NR SA FR1, long DRX cycle and intra-NR measurement enhancements in HST | Test execution not necessary if the NSA counterpart test 4.6.4.5 has been tested. | 2Rx 4Rx |
| 6.6.5 | FFS |  |  |  |  |  |
| 6.6.5.1 | NR SA FR1 – UTRAN FDD event triggered reporting in non-DRX | Rel-16 | C096 | UEs supporting 5GS NR SA FR1 and UTRAN FDD |  | 2Rx 4Rx |
| 6.6.8 | L1-SINR measurement for beam reporting |  |  |  |  |  |
| 6.6.8.1 | NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Rel-16 | C144 | UEs supporting 5GS NR SA FR1 and long DRX cycle and L1-SINR measurement based on CSI-RS as CMR without dedicated IMR configured |  | 2Rx 4Rx |
| 6.6.8.2 | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | Rel-16 | C145 | UEs supporting 5GS NR SA FR1 and L1-SINR measurement based on SSB as CMR and dedicated CSI-RS as IMR |  | 2Rx 4Rx |
| 6.6.8.3 | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Rel-16 | C146 | UEs supporting 5GS NR SA FR1 and L1-SINR measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  | 2Rx 4Rx |
| 6.6.9 | Idle Mode CA/DC Measurements |  |  |  |  |  |
| 6.6.9.1 | NR SA FR1 SA Idle mode CA/DC measurement for FR1 | Rel-16 | C031a | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and NR SSB measurements in RRC_IDLE/RRC_INACTIVE |  |  |
| 6.6.15 | Idle Mode inter-RAT CA/DC Measurements |  |  |  |  |  |
| 6.6.15.1 | NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting | Rel-16 | C031b | UEs supporting NE-DC FR1 and E-UTRA measurements in RRC_IDLE/RRC_INACTIVE |  |  |
| 6.6.18 | SA event triggered reporting tests with concurrent gaps |  |  |  |  |  |
| 6.6.18.1 | NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers | Rel-17 | FFS | FFS | NOTE 1 | 2Rx 4Rx |
| 6.6.18.2 | NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers | Rel-17 | FFS | FFS | NOTE 1 | 2Rx 4Rx |
| 6.6.18.3 | NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1 | Rel-17 | FFS | FFS | NOTE 1 | 2Rx 4Rx |
| 6.6.18.4 | NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used | Rel-17 | FFS | FFS | NOTE 1 | 2Rx 4Rx |
| 6.6.20 | UE Rx-Tx time difference measurement for propagation delay compensation |  |  |  |  |  |
| 6.6.20.1 | UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA | Rel-17 | C212 | UEs supporting 5GS NR SA FR1 and RTT-based PDC for Rx-Tx measurement with PRS | NOTE 1 | 2Rx |
| 6.6.21 | UE Rx-Tx time difference measurement for propagation delay compensation with TRS |  |  |  |  |  |
| 6.6.21.1 | UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA | Rel-17 | C213 | UEs supporting 5GS NR SA FR1 and RTT-based PDC for Rx-Tx measurement with TRS | NOTE 1 | 2Rx |
| 6.7 | Measurement performance requirements |  |  |  |  |  |
| 6.7.1 | SS-RSRP |  |  |  |  |  |
| 6.7.1.1 | Intra-frequency measurements |  |  |  |  |  |
| 6.7.1.1.1 | NR SA FR1 SS-RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.7.1.1.1 has been tested. | 2Rx 4Rx |
| 6.7.1.1.2 | NR SA FR1 SS-RSRP relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.7.1.1.2 has been tested. | 2Rx 4Rx |
| 6.7.1.2 | Inter-frequency measurements |  |  |  |  |  |
| 6.7.1.2.1 | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.7.1.2.1 has been tested. | 2Rx 4Rx |
| 6.7.1.2.2 | NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.7.1.2.2 has been tested. | 2Rx 4Rx |
| 6.7.2 | SS-RSRQ |  |  |  |  |  |
| 6.7.2.1 | NR SA FR1 SS-RSRQ measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.7.2.1 has been tested. | 2Rx 4Rx |
| 6.7.2.2.1 | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.7.2.2.1 has been tested. | 2Rx 4Rx |
| 6.7.2.2.2 | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.7.2.2.2 has been tested. | 2Rx 4Rx |
| 6.7.3 | SS-SINR |  |  |  |  |  |
| 6.7.3.1 | NR SA FR1 SS-SINR measurement accuracy | Rel-15 | C034 | UEs supporting 5GS NR SA FR1 and SS-SINR-meas | Test execution not necessary if the NSA counterpart test 4.7.3.1 has been tested. | 2Rx 4Rx |
| 6.7.3.2.1 | NR SA FR1-FR1 SS-SINR absolute measurement accuracy | Rel-15 | C034 | UEs supporting 5GS NR SA FR1 and SS-SINR-meas | Test execution not necessary if the NSA counterpart test 4.7.3.2.1 has been tested. | 2Rx 4Rx |
| 6.7.3.2.2 | NR SA FR1-FR1 SS-SINR relative measurement accuracy | Rel-15 | C034 | UEs supporting 5GS NR SA FR1 and SS-SINR-meas | Test execution not necessary if the NSA counterpart test 4.7.3.2.2 has been tested. | 2Rx 4Rx |
| 6.7.4 | L1-RSRP for beam reporting |  |  |  |  |  |
| 6.7.4.1.1 | NR SA FR1 SSB-based L1-RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.7.4.1.1 has been tested. | 2Rx 4Rx |
| 6.7.4.1.2 | NR SA FR1 SSB-based L1-RSRP relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.7.4.1.2 has been tested. | 2Rx 4Rx |
| 6.7.4.2.1 | NR SA FR1 CSI-RS-based L1-RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.7.4.2.1 has been tested. | 2Rx 4Rx |
| 6.7.4.2.2 | NR SA FR1 CSI-RS-based L1-RSRP relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if the NSA counterpart test 4.7.4.2.2 has been tested. | 2Rx 4Rx |
| 6.7.5 | E-UTRAN RSRP |  |  |  |  |  |
| 6.7.5.1 | NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.6 | E-UTRAN RSRQ |  |  |  |  |  |
| 6.7.6.1 | NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.7 | E-UTRAN RS-SINR |  |  |  |  |  |
| 6.7.7.1 | NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Rel-15 | C168 | UEs supporting 5GS NR SA FR1 and E-UTRA RS-SINR measurements |  | 2Rx 4Rx |
| 6.7.9 | L1-SINR |  |  |  |  |  |
| 6.7.9.1.1 | NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy | Rel-16 | C132 | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured | Test execution not necessary if the NSA counterpart test 4.7.7.1.1 has been tested. | 2Rx 4Rx |
| 6.7.9.1.2 | NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy | Rel-16 | C132 | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured | Test execution not necessary if the NSA counterpart test 4.7.7.1.2 has been tested. | 2Rx 4Rx |
| 6.7.9.2 | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Rel-16 | C133 | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on SSB as CMR and dedicated CSI-IM as IMR | Test execution not necessary if the NSA counterpart test 4.7.7.2 has been tested. | 2Rx 4Rx |
| 6.7.9.3.1 | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Rel-16 | C134 | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR | Test execution not necessary if the NSA counterpart test 4.7.7.3.1 has been tested. | 2Rx 4Rx |
| 6.7.9.3.2 | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy | Rel-16 | C134 | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR | Test execution not necessary if the NSA counterpart test 4.7.7.3.2 has been tested. | 2Rx 4Rx |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Test X refers to the corresponding Sub-Test as defined in TS 38.533 [5]. |  |  |  |  |  |  |

<< End of Changes >>