3GPP TSG WG RAN5 meeting#99					        R5-232687

Incheon, South Korea, 22nd May 2023 - 26th May 2023

Source:		Qualcomm Incorporated

Title:	Discussion to add coverage for the TEI15 feature - "intraFreqReselection” field in MIB message is set to "not allowed"

Document for:	Discussion and Decision

Agenda Item:	6.4.7

# Introduction

The Idle mode section of the TS38.523-1[2] does not cover the conformance requirements from TS38.304[1], clause 5.3.1, to check the UE behaviour when the field “intraFreqReselection” in MIB message is set to "not allowed”.

### [TS 38.304]

### 5.3.1	Cell status and cell reservations

…

-	If the field intraFreqReselection in MIB message is set to "not allowed":

-	If the cell is to be treated as if the cell status is "barre	d" due to being unable to acquire the SIB1:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

-	If the cell operates in licensed spectrum:

-	the UE shall not re-select to another cell on the same frequency as the barred cell and exclude such cell(s) as candidate(s) for cell selection/reselection for 300 seconds;

-	else:

-	the UE may select to another cell on the same frequency if the reselection criteria are fulfilled.

-	else:

-	If the cell operates in licensed spectrum, or if this cell belongs to a PLMN which is indicated as being equivalent to the registered PLMN or the selected PLMN of the UE, or if this cell belongs to the registered SNPN or the selected SNPN of the UE:

-	the UE shall not re-select to another cell on the same frequency as the barred cell and exclude such cell(s) as candidate(s) for cell selection/reselection for 300 seconds;

-	else:

-	the UE may select to another cell on the same frequency if the reselection criteria are fulfilled.

-	the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

# 2.	Discussion

To cover the Spec requirement described in section 1, a new test case can be defined as part of the Idle mode section “6.1.2 NG-RAN Only Cell Selection” or the existing test prose of “6.1.2.3 Cell selection / Intra NR / Serving cell becomes non-suitable (S<0, MIB Indicated barred)” can be updated to add an additional TP to cover the required conformance requirements w.r.t the UE behaviour when the field “intraFreqReselection” in MIB message is set to "not allowed”.

# 3.	Proposal

1) It is proposed that a testcase be added in TS38.523-1 as TC 6.1.2.3a. Contributions in R5-232688 address the new prose addition and R5-232689 addresses the applicability.

2) It is proposed that the existing testcase 6.1.2.3 be updated to add an additional TP (3) as below:

#### 6.1.2.3	Cell selection / Intra NR / Serving cell becomes non-suitable (S<0, MIB Indicated barred)

6.1.2.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state }

ensure that {

when { the serving cell becomes non-suitable (S<0)and there is a suitable neighbour cell (S>0) }

then { UE selects the suitable neighbour cell }

}

(2)

with { UE in NR RRC_IDLE state }

ensure that {

when { the serving cell becomes barred and there is a suitable neighbour cell }

then { UE selects the suitable neighbour cell }

}

(3)

with { UE in NR RRC_IDLE state }

ensure that {

when { a cell broadcasting intraFreqReselection in MIB message is set to "not allowed" }

then { the UE selects another suitable cell on a different frequency and camps on it }

}

# 4.	References

[1] 	TS38.304	User Equipment (UE) procedures in Idle mode and RRC Inactive state, (Release 15).

[2]	TS38.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 1: Protocol