3GPP TSG-RAN5 Meeting #99	R5-230279

# Incheon, South Korea, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2772 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update IE ServingCellConfigCommon for NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Abbreviation of NTN is necessary to be defined , and required condition needs to be added for the ntn-config-r17 IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Table 4.6.3-168: ServingCellConfigCommon to add the NTN condition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.6.3	Radio resource control information elements

#### <Tables skipped>

#### <Start of changes>

#### –	ServingCellConfigCommon

Table 4.6.3-168: ServingCellConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| physCellId | PhysCellId |  |  |
| downlinkConfigCommon | DownlinkConfigCommon |  |  |
|  | DownlinkConfigCommon with condition SCell_add |  | SCell_add |
| uplinkConfigCommon | UplinkConfigCommon |  |  |
|  | Not present |  | No_UL |
| supplementaryUplinkConfig | Not present |  |  |
| n-TimingAdvanceOffset | Not present |  |  |
| ssb-PositionsInBurst CHOICE { |  |  |  |
| shortBitmap | 0100 |  | FR1 AND SSB#1 AND (2.3GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=2.3GHZ) |
|  | 1000 |  | FR1 AND SSB#0 AND (2.3GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=2.3GHz) |
| mediumBitmap | 01000000 |  | FR1 AND SSB#1 AND (2.3GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
|  | 10000000 |  | FR1 AND SSB#0 AND (2.3GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
| longBitmap | 0100000000000000000000000000000000000000000000000000000000000000 |  | FR2 AND SSB#1 |
|  | 1000000000000000000000000000000000000000000000000000000000000000 |  | FR2 AND SSB#0 |
| } |  |  |  |
| ssb-periodicityServingCell | ms20 |  |  |
| dmrs-TypeA-Position | pos2 |  |  |
| lte-CRS-ToMatchAround | Not present |  |  |
| rateMatchPatternToAddModList | Not present |  |  |
| rateMatchPatternToReleaseList | Not present |  |  |
| ssbSubcarrierSpacing | SubcarrierSpacing | For signalling test cases see subclause 6.2.3. Otherwise, see subclause 4.3.1. Value SS block SCS. |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  | TDD |
|  | Not present |  | FDD |
| ss-PBCH-BlockPower | 0 |  |  |
| channelAccessMode-r16 | Not present |  |  |
| channelAccessMode-r16 CHOICE { |  |  | SharedSpectrum |
| dynamic | NULL |  |  |
| } |  |  |  |
| discoveryBurstWindowLength-r16 | Not present |  |  |
| ssb-PositionQCL-r16 | Not present |  |  |
|  | SSB-PositionQCL-Relation-r16 |  | SharedSpectrum |
| highSpeedConfig-r16 | Not present |  |  |
|  | HighSpeedConfig-r16 |  | R16 HST OR R17 HST FR1 |
| highSpeedConfig-v1700 | Not present |  |  |
|  | HighSpeedConfig-v1700 |  | R17 HST FR1 |
| channelAccessMode2-r17 | Not present |  |  |
| discoveryBurstWindowLength-r17 | Not present |  |  |
| ssb-PositionQCL-r17 | Not present |  |  |
| highSpeedConfigFR2-r17 | Not present |  |  |
| uplinkConfigCommon-v1700 | Not present |  |  |
| ntn-Config-r17 | Not present |  |  |
|  | NTN-Config |  | NTN |
| featurePriorities-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FREQ<=2.3GHz | Frequency range <= 2.3GHz |
| 2.3GHz<FREQ<=3GHz | Frequency range > 2.3GHz and <= 3GHz |
| FREQ>3GHz | Frequency range > 3GHz |
| No_UL | No uplink CA |
| SSB#N | Cell configured with SSB-Index set to N as defined in Table 4.4.2-2 |
| SharedSpectrum | Operation with shared spectrum channel access in FR1 |
| R16 HST | For R16 HST test |
| R17 HST FR1 | For R17 HST FR1 test |
| NTN | For NTN test cases |

#### <End of changes>

#### –	ServingCellConfigCommonSIB

Table 4.6.3-169: ServingCellConfigCommonSIB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB |  |  |
|  | DownlinkConfigCommonSIB with condition PEI |  | PEI |
| uplinkConfigCommon | UplinkConfigCommonSIB |  |  |
| supplementaryUplink | Not present |  |  |
|  | UplinkConfigCommonSIB with condition SUL_SUL |  | SUL |
| n-TimingAdvanceOffset | Not present |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ’0100 0000’B | When carrier frequency is smaller than or equal to 3 GHz, only the 4 leftmost bits are valid | SSB#1 |
|  | ’1000 0000’B |  | SSB#0 |
| groupPresence | Not present |  | FR1 |
|  | ’1000 0000’B |  | FR2 |
| } |  |  |  |
| ssb-PeriodicityServingCell | ms20 |  |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  | TDD |
|  | Not present |  | FDD |
| ss-PBCH-BlockPower | 0 |  |  |
| channelAccessMode-r16 | Not present |  |  |
| channelAccessMode-r16 CHOICE { |  |  | SharedSpectrum |
| dynamic | NULL |  |  |
| } |  |  |  |
| discoveryBurstWindowLength-r16 | Not present |  |  |
| highSpeedConfig-r16 | Not present |  |  |
|  | HighSpeedConfig-r16 |  | R16 HST OR R17 HST |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | Supplementary uplink |
| SSB#N | Cell configured with SSB-Index set to N as defined in Table 4.4.2-2 |
| SharedSpectrum | Operation with shared spectrum channel access |
| R16 HST | For R16 HST test |
| R17 HST | For R17 HST test |
| PEI | Paging Early Indication is configured in the cell. |