3GPP TSG-WG5 Meeting #99	R5-232699

# Incheon, South Korea, 22nd May – 26th May 2023

| Text Proposal |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.551 | CR | n/a | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on TS 38.551 Annex B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Electronics |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_OTA-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-25 |
| Category: | n/a |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 38.551 update on Annex B |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial clarification on MU budget reference, into the TS 38.551 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of requirements alignment between RAN4 and RAN5 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A, B, C, D, E and F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change>

Annex B (normative): MU Budget

Editor's Note: The final MU budget and TT will be defined in this specification and will update/replace the preliminary example MU values defined in RAN4 currently presented in B.1.

## B.1	MU budget of FR1 MPAC system

This clause defines the Preliminary measurement uncertainty (MU) budget for FR1 MPAC system, as shown in Table B.1-1.

Table B.1-1: Preliminary measurement uncertainty budget for FR1 MPAC system

| UID | Description of uncertainty contribution | Example value (410MHz<f≤3GHz) | Example value (3GHz <f≤7.125GHz) | Distribution of the probability | Std Uncertainty (410MHz<f≤3GHz) [dB] | Std Uncertainty (3GHz <f≤7.125GHz) [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| 1 | Mismatch for measurement process | 0 | 0 | U-Shaped | [0] | [0] |
| 2 | Measure distance uncertainty | 0 | 0 | Normal | [0] | [0] |
| 3 | Quality of quiet zone | 0.6 | 0.6 | Actual | [0.6] | [0.6] |
| 4 | Base Station simulator | 1.5dB | 2dB | Rectangular | [0.87] | [1.15] |
| 5 | Channel Emulator - absolute output power - output signal stability - output stability with temperature | 1.5dB 0.5dB 0.4dB | 1.5dB 0.5dB 0.4dB | Actual (Normal - power; rect-stability) | [0.84] | [0.84] |
| 6 | Amplifier uncertainties | 0.7dB | 0.7dB | Rectangular | [0.4] | [0.4] |
| 7 | Random uncertainty | 0.2dB | 0.2dB | Normal | [0.12] | [0.12] |
| 8 | Throughput measurement: output level step resolution | 0.25dB | 0.25dB | Rectangular | [0.14] | [0.14] |
| 9 | DUT sensitivity drift | 0.2 | 0.2 | Rectangular | [0.12] | [0.12] |
| 10 | Signal flatness | 0 | 0 | Normal | [0] | [0] |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| 11 | Mismatch for calibration process - loopback cable path - system input path - reference antenna | 0.2 | 0.2 | U-Shaped | [0.14] | [0.14] |
| 12 | Reference antenna positioning misalignment | 0 | 0 | Normal | [0] | [0] |
| 13 | Quality of quiet zone | 0.6 | 0.6 | [Rectangular] | [0.35] | [0.35] |
| 14 | Total uncertainty of the Network Analyzer | 0.5 | 0.5 | Rectangular | [0.29] | [0.29] |
| 15 | Uncertainty of an absolute gain of the calibration antenna | 1 | 1 | Normal | [0.5] | [0.5] |
| 16 | Offset of the Phase Center of the Reference Antenna | 0 | 0 | Normal | [0] | [0] |
| Total Expanded Uncertainty, U, with 95% Confidence Interval |  |  |  |  | [3.03 | [3.38] |

The detailed descriptions of each measurement uncertainty contributor are defined in Annex B.1.2 in [2].

## <End of Change>