3GPP TSG-WG5 Meeting #99	R5-232704

# Incheon, South Korea, 22nd May – 26th May 2023

| Text Proposal |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.551 | CR | n/a | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on TS 38.551 FR1 MIMO OTA Performance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Electronics |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_OTA-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-25 |
| Category: | n/a |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 38.551 update on FR1 MIMO OTA Performance |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial clarification into the TS 38.551 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of requirements alignment between RAN4 and RAN5 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change>

# 6	FR1 MIMO OTA Performance

## 6.1	General

### 6.1.1	Definition of MIMO throughput

The MIMO throughput is defined here as the time-averaged number of correctly received transport blocks in a communication system running an application, where a Transport Block is defined in the reference measurement channel. From OTA perspective, this is also called MIMO OTA throughput. It will be used as the baseline figure of merit for FR1 and FR2 MIMO OTA testing.

The MIMO OTA throughput is measured at the top of physical layer of NR system under the use of FRC, the SS transmit fixed-size payload bits to the DUT. The DUT signals back either ACK or NACK to the SS. The SS then records the following:

-	Number of ACKs,

-	Number of NACKs, and

-	Number of DTX slots

Hence the MIMO (OTA) throughput can be calculated as

Where Transmitted TBS is the Transport Block Size transmitted by the SS, which is fixed for an FRC during the measurement period. Measurement Time is the total composed of successful slots (ACK), unsuccessful slots (NACK) and DTX-symbols.

The time-averaging is to be taken over a time period sufficiently long to average out the variations due to the fading channel. Therefore, this is also called the average MIMO OTA throughput. The throughput should be measured at a time when eventual start-up transients in the system have evanesced.

### 6.1.2	Total Radiated Multi-Antenna Sensitivity (TRMS)

The average TRMS of free space data mode portrait (FS DMP), free space data mode landscape (FS DML), and free space data mode screen up (FS DMSU), is defined as the FR1 MIMO OTA requirement. The averaging shall be done in linear scale for the TRMS results at these DUT positions, according to the formula:

where

Such that MODE is one of {FS_DMP, FS_DML, FS_DMSU}, and {PMODE,70,0, …, PMODE,70,11} are the measured sensitivity values at each azimuth position at the 70% throughput outage.

If 1 azimuth position does not result in a defined measured sensitivity at 70% throughput, SMODE,70 is calculated using the 11 measured sensitivities and the maximum downlink RS-EPRE PRS-EPRE-MAX (substitution approach) for the one missing result. PRS-EPRE-MAX is the maximum downlink RS-EPRE supported by the test system, and is defined as -80dBm/15kHz (or equivalent -77dBm/30kHz) for FR1 MIMO OTA.

The TRMS shall be measured at the mid channel as specified in TS 38.508-1 subclause 4.3.1 [7]. The average TRMS shall be lower than the average TRMS requirements specified in Clause 6.2.

The additional criterion in azimuthal orientations shall be met:

-	The EUT must meet 70% throughput in 11 of total 12 azimuthal orientations. If the EUT fails to meet this criterion even under maximum downlink power condition (i.e. PRS-EPRE-MAX), the EUT shall fail the FR1 MIMO OTA test.

-	The EUT must meet 90% throughput in 10 of total 12 azimuthal orientations. If the EUT fails to meet this criterion even under maximum downlink power condition (i.e. PRS-EPRE-MAX), the EUT shall fail the FR1 MIMO OTA test.

## 6.2	Minimum requirement

FR1 TRMS minimum performance requirements for NR handheld UEs operating on SA mode in free space and the primary mechanical mode for 70% DL throughput with the corresponding measurement configurations (i.e., channel model and gNB configuration) specified in Annex C.1 and Annex E.1 are defined in Table 6.2-1.

Table 6.2-1: FR1 TRMS minimum performance requirements for NR handheld UEs operating on SA mode in free space and the primary mechanical mode

| NR bands | Bandwidth (MHz) | MIMO layer | Channel model | Reference channel | TRMSaverage,70 |
| --- | --- | --- | --- | --- | --- |
| n28 | 10 | 2x2 | FR1 UMi CDL-C | R.PDSCH.1-3.1 FDD | TBD dBm/15kHz |
| n41 | 40 | 4x4 | FR1 UMa CDL-C | R.PDSCH.2-2.4 TDD | -93.3 dBm/30kHz |
| n78 | 40 | 4x4 | FR1 UMa CDL-C | R.PDSCH.2-2.4 TDD | -94.8 dBm/30kHz |
| n79 | 40 | 4x4 | FR1 UMa CDL-C | R.PDSCH.2-2.4 TDD | TBD dBm/30kHz |

## <End of Change>