3GPP TSG-RAN5 Meeting #99	R5-232737

# Incheon, Korea, 22th– 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2248 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.5D.2.4.2_1 UTRA ACLR for SUL with UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_UL_MIMO_PC3_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For UE that supports UL MIMO on SUL bands, UTRA ACLR test case needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test case 6.5D.2.4.2_1 UTRA ACLR for SUL with UL MIMO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UTRA ACLR can’t be tested for UE that supports UL MIMO on SUL bands. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5D.2.4.2_1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 CR 2252 TS 38.522 CR 0278 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5D.2.4.2.5	Test requirement

UTRA ACLR requirement is applicable when signalled by the network with network signalling value indicated by the field additionalSpectrumEmission.

The measured UE mean total power in the channel bandwidth at all the antenna connectors, derived in step 3, shall fulfil requirements in [Clause 6.2D.3.5] as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured UTRA ACLR, derived in step 6 for each antenna connector, shall be higher than the limits in Table 6.5D.2.4.2.5-1.

Table 6.5D.2.4.2.5-1: UTRA ACLR requirement

|  | Power class 3 |
| --- | --- |
| UTRAACLR1 | 33 dB - TT |
| UTRAACLR2 | 36 dB - TT |
| NOTE 1: TT = 0.8 dB |  |

6.5D.2.4.2_1	UTRA ACLR for SUL with UL MIMO

6.5D.2.4.2_1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5D.2.4.2_1.2	Test applicability

This test case applies for NS_05U to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands.

6.5D.2.4.2_1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

For UE supporting UL MIMO, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters is defined as the sum of the emissions from both UE transmit antenna connectors.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclause 6.5.2.4.2.3 apply. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1_1.3.

The normative reference for this requirement is TS 38.101-1 [2] clauses 6.5D.2 and 6.5.2.4.2.

6.5D.2.4.2_1.4	Test description

6.5D.2.4.2_1.4.1	Initial conditions

S Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5C-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.5D.2.4.2_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.5D.2.4.2_1.4.1-1: Test Configuration Table for NS_05U

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: Use Fc as specified in test parameters NUL carrier: Mid range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | SUL carrier: 5 MHz, 10 MHz, 15 MHz, 20 MHz NUL carrier: Highest |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz for SUL and Lowest supported SCS for NUL carrier |  |  |  |  |
| AMPR Test parameters for NS_05U |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | Downlink Config. | Uplink config |  | A-MPR | SUL Configuration |  |  |
|  |  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |
| 1 | 1922.5 | 5 |  |  |  | A3 | CP-OFDM | QPSK | Outer_Full |
| 2 | 1925 | 10 |  |  |  | A1 |  | QPSK | Outer_Full |
| 3 | 1925 | 10 |  |  |  | A7 |  | QPSK | 42@10 |
| 4 | 1925 | 10 |  |  |  | A2 |  | QPSK | 6@40 |
| 5 | 1935 | 10 |  |  |  | A4 |  | QPSK | Outer_Full |
| 6 | 1927.5 | 15 |  |  |  | A1 |  | QPSK | Outer_Full |
| 7 | 1927.5 | 15 |  |  |  | A7 |  | QPSK | 60@19 |
| 8 | 1927.5 | 15 |  |  |  | A2 |  | QPSK | 6@56 |
| 9 | 1932.5 | 15 |  |  |  | A1 |  | QPSK | Outer_Full |
| 10 | 1932.5 | 15 |  |  |  | A2 |  | QPSK | 6@68 |
| 11 | 1942.5 | 15 |  |  |  | A5 |  | QPSK | Outer_Full |
| 12 | 1930 | 20 |  |  |  | A1 |  | QPSK | Outer_Full |
| 13 | 1930 | 20 |  |  |  | A7 |  | QPSK | 78@28 |
| 14 | 1930 | 20 |  |  |  | A2 |  | QPSK | 6@76 |
| 15 | 1950 | 20 |  |  |  | A6 |  | QPSK | Outer_Full |
| 16 | 1922.5 | 5 |  |  |  | A3 |  | 16 QAM | Outer_Full |
| 17 | 1925 | 10 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 18 | 1925 | 10 |  |  |  | A7 |  | 16 QAM | 42@10 |
| 19 | 1925 | 10 |  |  |  | A2 |  | 16 QAM | 6@40 |
| 20 | 1935 | 10 |  |  |  | A4 |  | 16 QAM | Outer_Full |
| 21 | 1927.5 | 15 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 22 | 1927.5 | 15 |  |  |  | A7 |  | 16 QAM | 60@19 |
| 23 | 1927.5 | 15 |  |  |  | A2 |  | 16 QAM | 6@56 |
| 24 | 1932.5 | 15 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 25 | 1932.5 | 15 |  |  |  | A2 |  | 16 QAM | 6@68 |
| 26 | 1942.5 | 15 |  |  |  | A5 |  | 16 QAM | Outer_Full |
| 27 | 1930 | 20 |  |  |  | A1 |  | 16 QAM | Outer_Full |
| 28 | 1930 | 20 |  |  |  | A7 |  | 16 QAM | 78@28 |
| 29 | 1930 | 20 |  |  |  | A2 |  | 16 QAM | 6@76 |
| 30 | 1950 | 20 |  |  |  | A6 |  | 16 QAM | Outer_Full |
| 31 | 1922.5 | 5 |  |  |  | A3 |  | 64 QAM | Outer_Full |
| 32 | 1925 | 10 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 33 | 1925 | 10 |  |  |  | A7 |  | 64 QAM | 42@10 |
| 34 | 1925 | 10 |  |  |  | A2 |  | 64 QAM | 6@40 |
| 35 | 1935 | 10 |  |  |  | A4 |  | 64 QAM | Outer_Full |
| 36 | 1927.5 | 15 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 37 | 1927.5 | 15 |  |  |  | A7 |  | 64 QAM | 60@19 |
| 38 | 1927.5 | 15 |  |  |  | A2 |  | 64 QAM | 6@56 |
| 39 | 1932.5 | 15 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 40 | 1932.5 | 15 |  |  |  | A2 |  | 64 QAM | 6@68 |
| 41 | 1942.5 | 15 |  |  |  | A5 |  | 64 QAM | Outer_Full |
| 42 | 1930 | 20 |  |  |  | A1 |  | 64 QAM | Outer_Full |
| 43 | 1930 | 20 |  |  |  | A7 |  | 64 QAM | 78@28 |
| 44 | 1930 | 20 |  |  |  | A2 |  | 64 QAM | 6@76 |
| 45 | 1950 | 20 |  |  |  | A6 |  | 64 QAM | Outer_Full |
| 46 | 1922.5 | 5 |  |  |  | A3 |  | 256 QAM | Outer_Full |
| 47 | 1925 | 10 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 48 | 1925 | 10 |  |  |  | A7 |  | 256 QAM | 42@10 |
| 49 | 1935 | 10 |  |  |  | A4 |  | 256 QAM | Outer_Full |
| 50 | 1927.5 | 15 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 51 | 1927.5 | 15 |  |  |  | A7 |  | 256 QAM | 60@19 |
| 52 | 1932.5 | 15 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 53 | 1942.5 | 15 |  |  |  | A5 |  | 256 QAM | Outer_Full |
| 54 | 1930 | 20 |  |  |  | A1 |  | 256 QAM | Outer_Full |
| 55 | 1930 | 20 |  |  |  | A7 |  | 256 QAM | 78@28 |
| 56 | 1950 | 20 |  |  |  | A6 |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL Reference Measurement channels are set according to Table 6.5D.2.4.2_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5D.2.4.2_1.4.3

6.5D.2.4.2_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the test configuration tables in clause 6.2D.3_1. 4.1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in clause 6.2D.3_1.5 as appropriate. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum of the rectangular filtered mean power for the assigned NR channel at each antenna connector of UE.

5.	Measure the sum of the RRC filtered mean power of the first and the second UTRA adjacent channel at each antenna connector of UE on both lower and upper side of the assigned NR channel, respectively.

6.	Calculate the ratio of the power between the values measured in step 4 over step 5 for UTRAACLR1, UTRAACLR2 for both lower an upper side of the assigned NR channel, respectively.

6.5D.2.4.2_1.4.3	Message contents

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Table 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL.

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO and with the exception specified in Table 6.5D.2.4.2_1.4.3-1.

Table 6.5D.2.4.2_1.4.3-1: AdditionalSpectrumEmission: UTRA ACLR test requirement for NS_05U

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission from SIB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 3 | NS_05U for band n84 |  |

6.5D.2.4.2_1.5	Test requirement

UTRA ACLR requirement is applicable when signalled by the network with network signalling value indicated by the field additionalSpectrumEmission.

The measured UE mean total power in the channel bandwidth at all the antenna connectors, derived in step 3, shall fulfil requirements in clause 6.2D.3_1.5 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured UTRA ACLR, derived in step 6, shall be higher than the limits in Table 6.5D.2.4.2_1.5-1.

Table 6.5D.2.4.2_1.5-1: UTRA ACLR requirement

|  | Power class 3 |
| --- | --- |
| UTRAACLR1 | 33 dB - TT |
| UTRAACLR2 | 36 dB - TT |
| NOTE 1: TT = 0.8 dB |  |

<End of Changes>