3GPP TSG RAN WG5 Meeting #99	R5-232745

Incheon, Korea (Republic Of), 22nd – 26th May 2023

Source:		Rohde & Schwarz

Title:	Discussion on GNSS emulation for NTN

Agenda Item:		5.3.38.9

Document for:	Approval

# Introduction

During the RAN5#98 a paper [1] was presented discussing the open points for NR NTN TC definition. In this paper we share our view on how to solve the issue of GNSS acquisition for NR-NTN and IoT-NTN.

# Discussion

Currently RAN5 is discussing two different work items with respect to NTN, one with respect to NR-NTN and the other one covering IoT-NTN. Since for both cases it is necessary to provide the UE with GNSS information, so that the UE can derive its location, it would be beneficial to have a common approach for both cases.

In the last meeting an associated action point has been raised:

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#98.23 | RF | Analysis how to address Initial condition and call setup procedure to support NR/IOT NTN satellite access test | CMCC, Qualcomm, Google, MTK, R&S | R5-230425 | RAN5#100 | Open |

As discussed in [1] there are multiple options to provide the GNSS information to the UE. The most prominent ones being discussed are either using GNSS simulator or providing the UE with one-shot GNSS information via e.g. AT commands.

From a test system vendor perspective both options are feasible, however they present different challenges.

While the usage of a GNSS simulator may provide real-time GNSS information to the UE, there are several drawbacks to this solution. GNSS emulators are designed to provide a wide range of GNSS emulation scenarios for LBS testing. For current NTN testing it seems that there is only a need to provide a fixed position to the UE, thus, using a fully-optioned GNSS simulator would unnecessarily increase the system complexity.

Observation 1: Using a GNSS simulator unnecessarily increases the test system complexity.

The UE needs to know its position in order to calculate the frequency pre-compensation. When using a GNSS simulator the UE needs to determine the position information based on its own GNSS receiver. Measurement uncertainties in this process can propagate through the pre-compensation and impact the measurement results for several TCs. This would need to be carefully considered and assessed.

Observation 2: Using a GNSS simulator introduces additional uncertainties into the TCs.

Both of these issues could be avoided by providing a one-shot GNSS fix to the UE at a defined location using AT commands.

Observation 3: Using a one-shot GNSS fix does not have the complexity and uncertainty issues of using a GNSS simulator.

Proprietary AT commands on the other hand may have the drawback that they would need to be device specific, which also leads to additional complexity for the test system development. This however could be lessened by introducing a common test function, which can be specified in TS 38.509 and/or 36.509.

Observation 4: Test system complexity issues of using proprietary AT commands can be reduced by specifying corresponding test functions.

Therefore, from a test system vendor perspective it would be beneficial to define test function which provides the GNSS information to the UE.

Proposal: RAN5 defines test functions in TS 38.509 and TS 36.509 to provide the UE with the GNSS information for NTN testing.

However, since there are already several protocol test cases existing for IoT-NTN, it may be difficult to “retroactively” introduce a test function into 36.509. For this case, further discussions may be necessary whether the usage of a test function is feasible or it is required to use proprietary AT commands.

# Proposals

In this contribution, we discussed how to provide the GNSS information to the UE for NTN testing, with the following Observations and Proposal.

Observation 1: Using a GNSS simulator unnecessarily increases the test system complexity.

Observation 2: Using a GNSS simulator introduces additional uncertainties into the TCs.

Observation 3: Using a one-shot GNSS fix does not have the complexity and uncertainty issues of using a GNSS simulator.

Observation 4: Test system complexity issues of using proprietary AT commands can be reduced by specifying corresponding test functions.

Proposal: RAN5 defines test functions in TS 38.509 and TS 36.509 to provide the UE with the GNSS information for NTN testing.

# References

- R5-231343, “Discussion on NR NTN open items”, Qualcomm Inc., RAN5#98, February 2023