3GPP TSG-RAN5 Meeting #99	R5-232784

# Incheon, Korea, May 22nd - May 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2785 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of a new combination of system information block for SIB16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_slice-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add a new combination of system information block for SIB16 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition a new combination of system information block for SIB16 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No combination of system information block for SIB16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.3.1.2, 4.4.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-1 CR 3759, 3760, 3678 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The hard number “NR-X” highlighted in yellow need to be assigned by MCC, and the “NR-X” used in 38.523-1 CR 3759, 3760 and 3678 need to be replaced by the assigned number. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

4.4.3.1.2	Combinations of system information blocks for NR standalone and NE-DC

< Unchanged sections omitted >

Combination NR-26 applies to the following test case scenarios:

-	3GPP NR FDD + eNPN single cell scenario

-	3GPP NR TDD + eNPN single cell scenario

Combination NR-27 applies to the following test case scenarios:

-	3GPP NR FDD + eNPN multi cell scenario

-	3GPP NR TDD + eNPN multi cell scenario

Combination NR-X applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi cell scenario + slice-based cell reselection in idle/inactive state

-	3GPP NR TDD inter-frequency multi cell scenario + slice-based cell reselection in idle/inactive state

Table 4.4.3.1.2-1: Combinations of system information blocks

|  |  |  | System information block type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Combination No. |  | SIB1 |  | SIB2 | SIB3 | SIB4 | SIB5 | SIB6 | SIB7 | SIB8 | SIB9 | SIB10 | SIB11 | SIB12 | SIB13 | SIB14 | SIB16 | posSIBs | SIB18 | SIB20 | SIB 21 |
| NR-1 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-2 |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-3 |  | X |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-4 |  | X |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-5 |  | X |  | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-6 |  | X |  | X |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-7 |  | X |  | X |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-8 |  | X |  |  |  |  |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-9 |  | X |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| NR-10 |  | X |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-11 |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-12 |  | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| NR-13 |  | X |  | X |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| NR-14 |  | X |  | X |  | X |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| NR-15 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| NR-16 |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| NR-17 |  | X |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| NR-18 |  | X |  |  |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| NR-19 |  | X |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| NR-19 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |
| NR-20 |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |
| NR-21 |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |
| NR-22 |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X |
| NR-23 |  | X |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |
| NR-24 |  | X |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |
| NR-25 |  | X |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X |
| NR-26 |  | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  | X |  |  |
| NR-27 |  | X |  | X |  |  |  |  |  |  |  | X |  |  |  |  |  |  | X |  |  |
| NR-X |  | X |  | X |  | X |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |

4.4.3.1.3	Scheduling of system information blocks

The scheduling configurations for combinations of system information blocks are defined in the following tables. There is no scheduling information for combination NR-1.

Table 4.4.3.1.3-1: Scheduling for combination NR-2

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |

< Unchanged sections omitted >

Table 4.4.3.1.3-27: Scheduling for combination NR-27

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB10, SIB18 |

Table 4.4.3.1.3-X: Scheduling for combination NR-X

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB1, SIB2, SIB4, SIB16 |

<End of modified section>