3GPP TSG-RAN5 Meeting #99	R5-232825

# Korea, Incheon, May 22nd - 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2266 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Sidelink configuration for PSSCH/PSCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some Sidelink configuration for PSSCH/PSCCH need to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some Sidelink configuration for PSSCH/PSCCH have been corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conformance test can not be implemented correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.1E	General

Sidelink RB allocations given in Table 6.1E-1 are used for V2X test cases, unless otherwise stated by the test case.

Table 6.1E-1: Sidelink configuration for PSSCH/PSCCH

| BWchannel (MHz) | SCS (kHz) | Parameters | RB allocation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer_Full | Outer_10RB_Left | Outer_10RB_Right | Inner_Full |
| 10MHz | 15 | PSSCH allocation | 50@0 | 10@0 | 10@40 | 25@12 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@40 | 10@12 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 42 | 12 |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | 25 |
|  |  | sl-NumSubchannel-r16 | 5 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 30 | PSSCH allocation | 24@0 | 10@0 | 10@14 | 12@6 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@14 | 10@6 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 14 | 6 |
|  |  | sl-SubchannelSize-r16 | 12 | 10 | 10 | 12 |
|  |  | sl-NumSubchannel-r16 | 2 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 60 | PSSCH allocation | 10@0 | 10@0 | 10@0 | N/A |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@0 | N/A |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 1 | N/A |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | N/A |
|  |  | sl-NumSubchannel-r16 | 1 | 1 | 1 | N/A |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
| 20MHz | 15 | PSSCH allocation | 105@0 | 10@0 | 10@95 | 50@25 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@95 | 10@25 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 96 | 25 |
|  |  | sl-SubchannelSize-r16 | 15 | 10 | 10 | 10 |
|  |  | sl-NumSubchannel-r16 | 7 | 1 | 1 | 5 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 30 | PSSCH allocation | 50@0 | 10@0 | 10@40 | 25@12 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@40 | 10@12 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 41 | 12 |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | 25 |
|  |  | sl-NumSubchannel-r16 | 5 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 60 | PSSCH allocation | 24@0 | 10@0 | 10@14 | 12@6 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@14 | 10@6 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 14 | 6 |
|  |  | sl-SubchannelSize-r16 | 12 | 10 | 10 | 12 |
|  |  | sl-NumSubchannel-r16 | 2 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |

| BWchannel (MHz) | SCS (kHz) | Parameters | RB allocation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer_Full | Outer_10RB_Left | Outer_10RB_Right | Inner_Full |
| 30MHz | 15 | PSSCH allocation | 160@0 | 10@0 | 10@150 | 80@40 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@150 | 10@40 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 150 | 40 |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-NumSubchannel-r16 | 16 | 1 | 1 | 8 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 30 | PSSCH allocation | 75@0 | 10@0 | 10@65 | 36@18 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@65 | 10@18 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 68 | 18 |
|  |  | sl-SubchannelSize-r16 | 15 | 10 | 10 | 12 |
|  |  | sl-NumSubchannel-r16 | 5 | 1 | 1 | 3 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 60 | PSSCH allocation | 36@0 | 10@0 | 10@26 | 15@7 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@26 | 10@7 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 28 | 7 |
|  |  | sl-SubchannelSize-r16 | 12 | 10 | 10 | 15 |
|  |  | sl-NumSubchannel-r16 | 3 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
| 40MHz | 15 | PSSCH allocation | 216@0 | 10@0 | 10@206 | 105@52 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@206 | 10@52 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 206 | 52 |
|  |  | sl-SubchannelSize-r16 | 12 | 10 | 10 | 15 |
|  |  | sl-NumSubchannel-r16 | 18 | 1 | 1 | 7 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 30 | PSSCH allocation | 105@0 | 10@0 | 10@95 | 50@25 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@95 | 10@25 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 96 | 25 |
|  |  | sl-SubchannelSize-r16 | 15 | 10 | 10 | 10 |
|  |  | sl-NumSubchannel-r16 | 7 | 1 | 1 | 5 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |
|  | 60 | PSSCH allocation | 50@0 | 10@0 | 10@40 | 25@12 |
|  |  | PSCCH allocation | 10@0 | 10@0 | 10@40 | 10@12 |
|  |  | sl-StartRB-Subchannel-r16 | 0 | 0 | 41 | 12 |
|  |  | sl-SubchannelSize-r16 | 10 | 10 | 10 | 25 |
|  |  | sl-NumSubchannel-r16 | 5 | 1 | 1 | 1 |
|  |  | sl-FreqResourcePSCCH-r16 | 10 | 10 | 10 | 10 |
|  |  | sl-TimeResourcePSCCH-r16 | 3 | 3 | 3 | 3 |

<< END OF CHANGES >>