3GPP TSG-RAN5 Meeting # 99	R5-232835

Incheon, KR, May 22nd – May 26th , 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0670 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add applicability rule for PDSCH with inter cell interference and CRS-IM demodulation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The applicability rules for PDSCH with inter cell interference and CRS-IM demodulation requirements are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add applicability rule for PDSCH with inter cell interference and CRS-IM demodulation requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no applicability rules for PDSCH with inter cell interference and CRS-IM demodulation requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.3 5.1.1.10 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

5.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 5.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 5.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 3-1) Clause 5.2.3.1.1 (Test 5-1) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 3-1) Clause 5.2.3.2.1 (Test 5-1) |  |
| Alternative additional DMRS position for co-existence with LTE CRS (additionalDMRS-DL-Alt) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 (Test 1-2) Clause 5.2.3.1.4 (Test 1-2) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.4 (Test 1-2) Clause 5.2.3.2.4 (Test 1-2) |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) | NR CA | SDR | Clause 5.5A.1 | 1) Up to 16 DL carriers 2) Same numerology across carrier for data/control channel at a given time |
| Enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h | FR1 FDD | PDSCH | Clause 5.2.2.1.9 (Test 1-1) Clause 5.2.3.1.9 (Test 1-1) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.9 (Test 1-1) Clause 5.2.3.2.9 (Test 1-1) |  |
| Single DCI based SDM transmission for multi-TRxP (singleDCI-SDM-scheme-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.11 Clause 5.2.3.1.11 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.11 Clause 5.2.3.2.11 |  |
| Multi DCI based multi-TRxP support (multiDCI-MultiTRP-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.12 Clause 5.2.3.1.12 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.12 Clause 5.2.3.2.12 |  |
| Single DCI based FDM Scheme-A for multi-TRxP(supportFDM-SchemeA-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.13 Clause 5.2.3.1.13 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.13 Clause 5.2.3.2.13 |  |
| Single DCI based inter-slot TDM for multi-TRxP (supportInter-slotTDM-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 Clause 5.2.3.1.14 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.14 Clause 5.2.3.2.14 |  |
| Maximum number of TCI states in Single-DCI based inter-slot TDM (maxNumberTCI-states-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 Clause 5.2.3.1.14 | The requirements apply only when maxNumberTCI-states-r16 = 2. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.14 Clause 5.2.3.2.14 |  |
| Alternative 64QAM MCS table for PDSCHNew 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt) | FR1 FDD | PDSCH | Clause 5.2.2.1.5 Clause 5.2.3.1.5 Clause 5.2.2.1.6 Clause 5.2.3.1.6 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.5 Clause 5.2.3.2.5 Clause 5.2.2.2.6 Clause 5.2.3.2.6 |  |
| CQI table with target BLER of 10^-5New CQI table (cqi-TableAlt) | FR1 FDD | PDSCH | Clause 5.2.2.1.5 Clause 5.2.3.1.5 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.5 Clause 5.2.3.2.5 |  |
| PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots) | FR1 FDD | PDSCH | Clause 5.2.2.1.6 Clause 5.2.3.1.6 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.6 Clause 5.2.3.2.6 |  |
| UE PDSCH processing capability #2 (pdsch-ProcessingType2) | FR1 FDD | PDSCH | Clause 5.2.2.1.7 Clause 5.2.3.1.7 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.7 Clause 5.2.3.2.7 |  |
| Pre-emption indication for DL (pre-EmptIndication-DL) | FR1 FDD | PDSCH | Clause 5.2.2.1.8 Clause 5.2.3.1.8 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.8 Clause 5.2.3.2.8 |  |
| Support of neighboring LTE cell CRS-IM in DSS scenario with NR 15 kHz SCS ( CRS-IM-DSS-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.18 Clause 5.2.3.1.17 | UE can support the feature on the CC(s) in a band only if the UE indicates support of rateMatchingLTE-CRS on that band. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.19 Clause 5.2.3.2.18 |  |
| Support of neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth (CRS-IM-nonDSS-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.19 (Test 2-1) Clause 5.2.3.1.18 (Test 2-1) | The UE can perform CRS-IM when MeasObjectEUTRA IE is configured, and the configured measurement gaps overlap with neighbour LTE cell PBCH position. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 2-1) Clause 5.2.3.2.19 (Test 2-1) |  |
| Support of neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth (CRS-IM-nonDSS-NWA-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.19 (Test 1-1) Clause 5.2.3.1.18 (Test 1-1) | If the Test 2-1 in Clause 5.2.2.1.19 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.2.1.19. If the Test 2-1 in Clause 5.2.3.1.18 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.3.1.18. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 1-1) Clause 5.2.3.2.19 (Test 1-1) | If the Test 2-1 in Clause 5.2.2.2.20 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.2.2.20. If the Test 2-1 in Clause 5.2.3.2.19 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.3.2.19. |
| CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth (crs-IM-nonDSS-30kHzSCS-r17) | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 2-2) Clause 5.2.3.2.19 (Test 2-2) | The UE can perform CRS-IM when MeasObjectEUTRA IE is configured, and the configured measurement gaps overlap with neighbour LTE cell PBCH position. |
| CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth (crs-IM-nonDSS-NWA-30kHzSCS-r17) | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 1-2) Clause 5.2.3.2.19 (Test 1-2) | If the Test 2-2 in Clause 5.2.2.2.20 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.2.2.2.20. If the Test 2-2 in Clause 5.2.3.2.19 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.2.3.2.19. |

<Unchanged part skipped>

5.1.1.9	Applicability of requirements for PDSCH on bands with shared spectrum access

| Tests | Applicability notes |
| --- | --- |
| All tests in Clause 5.2.2.2.15 and 5.2.3.2.15 | Only test the supported largest channel bandwidth. |
| All tests in Clause 5.2A.2.3 and 5.2A.3.3 | Only test the supported largest channel bandwidth on SCell. |

5.1.1.10	Applicability of requirements for PDSCH with inter cell interference

| Tests | Applicability notes |
| --- | --- |
| All tests in Clause 5.2.2.1.15, 5.2.3.1.15, 5.2.2.2.16 and 5.2.3.2.16 | If UE supporting both duplex mode TDD and FDD with 2RX, only test 1-1 in clause 5.2.2.1.15 and test 1-2 in clause 5.2.2.2.16 will be applied. If UE supporting both duplex mode TDD and FDD with 4RX, only test 1-1 in clause 5.2.3.1.15 and test 1-2 in clause 5.2.3.2.16 will be applied. |

<End of change>