3GPP TSG-RAN5 Meeting #99	R5-232933

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0543 | rev | - | Current version: | 16.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS testcase 10.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR includes several modifications ranging from editorial corrections to changes to modify the test case behavior. This testcase is run with preconditions as per cl 10.4.3.1. Hence the Table 10.4.3.3-1 needs to be updated to include requires-option tag and set it to “preconditions”. In pre-test conditions, removed the reference to (Item 3 in Table A.4.3-2 in TS 36.523-2 [XX]).Simialar text exists in 34.229-1 cl 19.3.1.4. This is a editorial clean up. In current Prose, the Steps 10-12 (which follow 38.508-1 Table 4.9.15.2.2-1) happen before step 15 (Step 5 of A.4.1), which is incorrect. This needs to be called after Step 15. UPDATE, as specified, will be only sent when TS 24.229 clause 5.1.6.10 conditions are met. There are 3 clauses where GRUU and secure port in contact header will always be met as in case contrary test case will fail at earlier stages (INVITE reception). This means that with actual implementation, UPDATE will be only sent in case UE supports geolocation services (in this case UE will include location services information in SDP as it was not sent in INVITE as well as related headers): - if the UE has its geographical location information available, or a URI that points to the location information, and the geographical location information or URI has not been included in the initial request for a dialog; Currently UPDATE is a mandatory step, however UPDATE will be mandatory to be sent by UE at step 16 and inclusion of location information in UPDATE body will depend on UE capabilities In current Prose in Table 10.4.3.3-1, the Content-type has incorrect condition setting for condition A1. Current prose indicates that : If condition A1 applies: multipart/mixed If condition A1 application/sdp This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Table 10.4.3.3-1 to include requires-option tag and set it to “preconditions”. Removed the reference to (Item 3 in Table A.4.3-2 in TS 36.523-2 [XX]) in pre-test conditions. Voided Steps 10-12, and introduced new Steps 15A-15C which refer to Steps 10-12 of 38.508-1 Table 4.9.15.2.2-1. Updated Table 10.4.3.3-2 and introduced not A1 condition, changing it to : If condition A1 applies: multipart/mixed If condition not A1 applies : application/sdp (This is also inline with 34.229-1 cl 19.3.1.4 Step 7) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.4	Non-UE detectable emergency call / IM CN sends a 1xx response / UE geographical location information available or not / 5GS

10.4.1	Test Purpose (TP)

(1)

with { UE being registered to IMS}

ensure that {

when { UE is being made to initiate a voice call it does not recognize as an emergency call }

then { UE sends a correctly composed INVITE request }

}

(2)

with { UE having sent INVITE for a non-UE detectable emergency call }

ensure that {

when { UE receives 183 Session Progress containing a P-Asserted-Identity header }

then { UE sends (after PRACK/200 OK) a correctly composed UPDATE request and completes the call setup }

}

10.4.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229 clause 5.1.6.10]

If the UE receives a 1xx or 200 (OK) response to an initial request for a dialog, the response containing a P-Asserted-Identity header field set to an emergency number as specified in 3GPP TS 22.101 [1A], and:

-	if a public GRUU value (pub-gruu) has been saved associated with the public user identity, the public GRUU value has not been included in the Contact header field of the initial request for a dialog as specified in RFC 5627 [93];

-	if a public GRUU value (pub-gruu) has not been saved and a protected server port was not included in the address in the Contact header field of the initial request for a dialog; or

-	if the UE has its geographical location information available, or a URI that points to the location information, and the geographical location information or URI has not been included in the initial request for a dialog;

then the UE shall send an UPDATE request according to RFC 3311 [29]; and:

1)	if available to the UE, and if defined for the access type as specified in subclause 7.2A.4, the UE shall include in the UPDATE request a P-Access-Network-Info header field and it shall contain a location identifier such as the cell id or the identity of the WLAN access node;

2)	if the UE has its geographical location information available, or a URI that points to the location information, then the UE shall include it in the UPDATE request in the following way:

I)	if the UE is aware of the URI that points to where the UE's location is stored, include the URI as the Geolocation header field value, as described in RFC 6442 [89]; or

II)	if the UE is aware of its location information, include the location information in a PIDF location object, in accordance with RFC 4119 [90], include the location object in a message body with the content type application/pidf+xml, and include a Content ID URL, referring to the message body, as the Geolocation header field value, as described RFC 6442 [89], and include a Content-Disposition header field with a disposition type "render" value and a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

3)	if the UE includes a Geolocation header field, the UE shall also include a Geolocation-Routing header field with a "yes" header field value, which indicates that the location of the UE can be used by other entities to make routing decisions, as described in RFC 6442 [89];

4)	if the UE has neither geographical location information available, nor a URI that points to the location information, the UE shall not insert a Geolocation header field in the UPDATE request; and

5)	if a public GRUU value ("pub-gruu" header field parameter) has been saved associated with the public user identity, then the UE shall insert the public GRUU ("pub-gruu" header field parameter) value in the Contact header field of the UPDATE request as specified in RFC 5627 [93]; otherwise the UE shall include the address in the Contact header field set in accordance with subclause 5.1.6.8.4, item 8.

NOTE 1:	The IMS emergency specification in 3GPP TS 23.167 [4B] describes several methods how the UE can get its location information from the access network or from a server. Such methods are not in the scope of this specification.

NOTE 2:	It is suggested that UEs only use the option of providing a URI when the domain part belongs to the current P-CSCF or S-CSCF provider. This is an issue on which the network operator needs to provide guidance to the end user. A URI that is only resolvable to the UE which is making the emergency call is not desirable.

NOTE 3:	During the dialog, the points of attachment to the IP-CAN of the UE can change (e.g. UE connects to different cells). The UE will populate the P-Access-Network-Info header field in any request (except CANCEL requests) or response (except CANCEL responses) within a dialog with the current point of attachment to the IP-CAN (e.g. the current cell information).

NOTE 4:	In this version of the specification, only requests creating a dialog can request emergency services.

10.4.3	Test description

10.4.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	UE is configured to use preconditions.

-	The test environment shall be set up to provide the needed input to the UE, in order for the UE to derive its location, if the UE is capable of obtaining location information. This shall be done by use of the test function Update UE Location Information defined in TS 38.509 [48], if supported by the UE according to pc_UpdateUE_LocationInformation. Otherwise, or in addition any other suitable method may also be used.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

10.4.3.2	Test procedure sequence

Table 10.4.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE attempt an IMS voice call. |  |  | - | - |
| 2-7 | Steps 2-7 of test procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to step 8 below, step 8 of test procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] takes place. | - | - | - | - |
| 8 | Check: Does the UE send an INVITE (Step 1 of Annex A.4.1)? | --> | INVITE | 1 | P |
| 9 | SS sends a 100 Trying provisional response (Step 2 of Annex A.4.1) | <-- | 100 Trying | - | - |
| 10-12 | Void | - | - | - | - |
| 13 | SS sends an SDP answer (Step 3 of Annex A.4.1) | <-- | 183 Session Progress | - | - |
| 14 | UE acks 183 Session Progress (Step 4 of Annex A.4.1) | --> | PRACK | - | - |
| 15 | SS responds to PRACK (Step 5 of Annex A.4.1) | <-- | 200 OK | - | - |
| 15A-15C | Steps 10 -12 of test procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. |  |  |  |  |
| 16 | Check: Does the UE send an UPDATE? | --> | UPDATE | 2 | P |
| 17 | SS responds to UPDATE (Step 7 of Annex A.4.1) | <-- | 200 OK | - | - |
| 17A | SS sends 180 Ringing reliably. (Step 8 of Annex A.4.1) | <-- | 180 Ringing | - | - |
| 17B | UE acknowledges reception of 180 Ringing. (Step 9 of Annex A.4.1) | --> | PRACK | - | - |
| 17C | SS responds to PRACK. (Step10 of Annex A.4.1) | <-- | 200 OK | - | - |
| 17D | SS responds to INVITE. (Step 11 of Annex A.4.1) | <-- | 200 OK | - | - |
| 18 | UE acks 200 OK for INVITE (Step 12 of Annex A.4.1) | --> | ACK | - | - |
| 19-20 | Steps 1-2 of the procedure A.8 are executed for MT release of speech call | - | - | - | - |
| 21-23 | Steps 3-5 of test procedure specified in Table 4.9.18.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |

10.4.3.3	Specific message contents

Table 10.4.3.3-1: 183 Session Progress (step 13, table 10.4.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in annex A.2.3, Conditions A1, A5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Require option-tag |  | precondition |  |  |
| Message-body |  | The following SDP types and values. Session description: v=0 o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) s=- c=IN (addrtype) (connection-address for SS) b=AS:65 Time description: t=0 0 Media description: m=audio (transport port) RTP/AVP (fmt) [Note 1, 2] b=AS:65 b=RS: (bandwidth-value) [Note 3] b=RR: (bandwidth-value) [Note 3] Attributes for media: a=rtpmap: (payload type) EVS/16000/1 [Note 1, 8] a=fmtp: (format) br=13.2; bw=swb; mode-set=0,1,2; max-red=220 [Note 8] a=rtpmap: (payload type) EVS/16000/1 [Note 1, 9] a=fmtp: (format) br=5.9-13.2; bw=nb-swb; mode-set=0,1,2; max-red=220 [Note 9] a=ecn-capable-rtp: leap ect=0 [Note 6] a=rtcp-fb:* nack ecn [Note 6] a=rtcp-xr:ecn-sum [Note 6] a=ptime:20 a=maxptime:240 Attributes for media security mechanism: a=3ge2ae: requested [Note 7] a=crypto:1 AES_CM_128_HMAC_SHA1_80inline:PS1uQCVeeCFCanVmcjkpPywjNWhcYD0mXXtxaVBR¦2^20¦1:4 [Note 7] Attributes for preconditions: a=curr:qos local none a=curr:qos remote none a=des:qos mandatory local sendrecv a=des:qos mandatory remote sendrecv a=conf:qos remote sendrecv Note 1: The values for fmt, payload type and format are copied from step 1. Note 2: Transport port is the port number of the SS (see RFC 3264 clause 6). Note 3: The bandwidth-value is copied from step 1. Note 4: Void Note 5: Void Note 6: Attributes for ECN Capability are present if the UE supports Explicit Congestion Notification. Note 7: Attributes for media plane security are present if the use of end-to-access-edge security is supported by UE. Note 8: This EVS configuration is sent if UE sent it as the first of its EVS configurations in INVITE. Note 9: This EVS configuration is sent if UE did not send "br=13.2; bw=swb" as the first of its EVS configurations in INVITE. |  |  |

Table 10.4.3.3-2: UPDATE (step 16, table 10.4.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in annex A.2.5, Conditions A1, A6 |  |  |
| --- | --- | --- |
| Header/param | Cond | Value/Remark |
| Geolocation locationURI | B1 | cid-url indicating the Content-Id of the PIDF-LO within the multipart MIME body of INVITE request. (Note that location-by-reference URI is not allowed as the SS does not provide any external storage for location info for the UE to refer.) |
| Geolocation-Routing | B1 | “yes” |
| Contact addr-spec |  | SIP URI with IP address or FQDN and protected server port of UE |
| Content-Type media-type |  | If condition A1 applies: multipart/mixed If condition not A1 applies: application/sdp |
| Message-body |  | If condition A1 applies, the multipart-mime body shall also contain a PIDF-LO element mapped to the same Content-ID which can be found from the Geolocation header The PIDF-LO shall contain at least the following elements: - One or more ‘geopriv’ elements, each containing: - One ‘location-info’ element describing the location of the UE; and - One ‘usage-rules’ element describing the limitations of the usage of the location info. |

| Condition | Explanation |
| --- | --- |
| B1 | UE is capable of obtaining location information |

Table 10.4.3.3-3: Void

Table 10.4.3.3-4: 200 OK (step 17, table 10.4.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.3.1, Condition A6.