3GPP TSG-RAN5 Meeting #99	R5-232940

# Incheon, Korea, 22th May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3767 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new RedCap TC 7.1.1.1.15-SI request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | if initialUplinkBWP-RedCap is not configured, RedCap UE uses the PRACH preambles and PRACH resources in si-RequestConfig to initiate the Random Access for System information request. That is covered by legacy test case 7.1.1.1.3. if initialUplinkBWP-RedCap is configured, RedCap UE uses the PRACH preambles and PRACH resources in si-RequestConfigRedcap to initiate the Random Access for System information request. That is covered by any test case.Therefore 7.1.1.1.15 is introduced to cover this senario based on TF160's comment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of RedCap TC 7.1.1.1.15 Random access procedure / RedCap UE / SI request. The test procedure for 7.1.1.1.15 is same as 7.1.1.1.3, but the message contain for SIB1 is different. InitialUplinkBWP-RedCap and si-RequestConfig and si-RequestConfigRedcap are configured in SIB1, and test case check UE using the PRACH resources configured in si-RequestConfigRedcap. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_redcap_plus_ARCH-UEConTest will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.15 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0355 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.1.15	Random access procedure / RedCap UE / SI request

7.1.1.1.15.1	Test Purpose (TP)

(1)

with { UE in RRC_Idle State and initialUplinkBWP-RedCap and si-RequestConfigRedCap is configured in SIB1 of Serving Cell}

ensure that {

when { one System information,which si-BroadcastStatus is set to notBroadcasting, is updated }

then { UE transmitted PRACH preamble for System information request using the PRACH preamble(s) and PRACH resource(s) in si-RequestConfigRedcap and retransmitted PRACH preamble when ra-ResponseWindow has expired }

}

(2)

with { UE in RRC_Idle State and transmitted PRACH preamble for System information request }

ensure that {

when { UE received a RAR message addressed to RA-RNTI and including matching RAPID only }

then { UE considers the RACH procedure to be successfully completed and informs the upper layer }

}

7.1.1.1.15.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.321, clauses 5.1.1 and 5.1.2, TS38.331, clauses 5.2.2.3.3 and 6.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.1.1]

-	ra-PreambleStartIndex: the starting index of Random Access Preamble(s) for on-demand SI request;

[TS 38.321, clause 5.1.2]

If the selected RA_TYPE is set to 4-stepRA, the MAC entity shall:

…

1>	else if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]); and

1>	if the Random Access Resources for SI request have been explicitly provided by RRC:

2>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above rsrp-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	select a Random Access Preamble corresponding to the selected SSB, from the Random Access Preamble(s) determined according to ra-PreambleStartIndex as specified in TS 38.331 [5];

2>	set the PREAMBLE_INDEX to selected Random Access Preamble.

[TS 38.331, clause 5.2.2.3.3]

The UE shall, while SDT procedure is not ongoing:

…

1>	else if the UE is a RedCap UE and if initialUplinkBWP-RedCap is configured in UplinkConfigCommonSIB and if SIB1 includes si-SchedulingInfo containing si-RequestConfigRedCap and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met:

2>	trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in si-RequestConfigRedcap corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting;

2>	if acknowledgement for SI request is received from lower layers:

3>	acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

1>	else:

2>	if the UE is not a RedCap UE and if SIB1 includes si-SchedulingInfo containing si-RequestConfig and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met; or

2>	if the UE is a RedCap UE and if initialUplinkBWP-RedCap is not configured in UplinkConfigCommonSIB and if SIB1 includes si-SchedulingInfo containing si-RequestConfig and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met:

3>	trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in si-RequestConfig corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting;

3>	if acknowledgement for SI request is received from lower layers:

4>	acquire the requested SI message(s) as defined in clause 5.2.2.3.2, immediately;

[TS 38.331, clause 6.3.2]

si-RequestConfigRedCap

Configuration of Msg1 resources for initialUplinkBWP-RedCap that the RedCap UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting.

7.1.1.1.15.3	Test description

7.1.1.1.15.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 11.

-	System information combination NR-3 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in NR RRC_Idle mode (state 1N-A) on NR Cell 1 according to 38.508-1 [4] Table 4.4A.2-1.

7.1.1.1.15.3.2	Test procedure sequence

Same test procedure sequence as in clause 7.1.1.1.3.3.3 with the following differences:

Table 7.1.1.1.15.3.2-3 is the same as Table 7.1.1.1.3.3.2-3 with the exception that the preamble is indicated by ra-PreambleStartIndex defined in SI-RequestConfigRedCap in SIB1 in Table 7.1.1.1.15.3.3-1 in step 5/6/7/8.

7.1.1.1.15.3.3	Specific message contents

Table 7.1.1.1.15.3.3-1: SIB1 on NR Cell 1 (Step 4, Table 7.1.1.1.15.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| si-SchedulingInfo SEQUENCE { |  |  |  |  |
| schedulingInfoList SEQUENCE { |  | 2 entries |  |  |
| si-BroadcastStatus[1] |  | Broadcasting |  |  |
| si-Periodicity[1] |  | rf32 |  |  |
| sib-MappingInfo[1] SEQUENCE { |  |  |  |  |
| type |  | SibType2 |  |  |
| valueTag |  | 0 |  |  |
| areaScope |  | Not present |  |  |
| } |  |  |  |  |
| si-BroadcastStatus[2] |  | notBroadcasting |  |  |
| si-Periodicity[2] |  | rf64 |  |  |
| sib-MappingInfo[2] SEQUENCE { |  |  |  |  |
| type |  | SibType3 |  |  |
| valueTag |  | 1 |  |  |
| areaScope |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| si-RequestConfig SEQUENCE { |  |  |  |  |
| rach-OccasionsSI SEQUENCE { |  |  |  |  |
| rach-ConfigSI |  | RACH-ConfigGeneric | TS 38.508-1 [4], Table 4.6.3-130 |  |
| ssb-perRACH-Occasion |  | one |  |  |
| } |  |  |  |  |
| si-RequestPeriod |  | two |  |  |
| si-RequestResources SEQUENCE { |  | 1 entry |  |  |
| ra-PreambleStartIndex[1] |  | 52 |  |  |
| ra-AssociationPeriodIndex[1] |  | 0 |  |  |
| ra-ssb-OccasionMaskIndex[1] |  | 0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| si-RequestConfigSUL |  | Not present |  |  |
| } |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 7.1.1.1.15.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| si-SchedulingInfo-v1740 SEQUENCE { |  |  |  |  |
| si-RequestConfigRedCap-r17 SEQUENCE { |  |  |  |  |
| rach-OccasionsSI SEQUENCE { |  |  |  |  |
| rach-ConfigSI |  | RACH-ConfigGeneric | TS 38.508-1 [4], Table 4.6.3-130 |  |
| ssb-perRACH-Occasion |  | one |  |  |
| } |  |  |  |  |
| si-RequestPeriod |  | two |  |  |
| si-RequestResources SEQUENCE { |  | 1 entry |  |  |
| ra-PreambleStartIndex[1] |  | 53 | Different from the ra-PreambleStartIndex in si-RequestConfig |  |
| ra-AssociationPeriodIndex[1] |  | 0 |  |  |
| ra-ssb-OccasionMaskIndex[1] |  | 0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.1.15.3.3-2: ServingCellConfigCommonSIB (Table 7.1.1.1.15.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| uplinkConfigCommon-v1700 ::= SEQUENCE { |  |  |  |
| initialUplinkBWP-RedCap-r17 | BWP-UplinkCommon | Table 7.1.1.1.15.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.15.3.3-3: BWP-UplinkCommon (Table 7.1.1.1.15.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| pucch-ConfigCommon CHOICE { |  |  |  |
| setup | PUCCH-ConfigCommon | Table 7.1.1.1.15.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.15.3.3-4: PUCCH-ConfigCommon (Table 7.1.1.1.15.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-113 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pucch-ResourceCommon | Not present |  |  |
| pucch-ResourceCommonRedCap-r17 | 0 |  |  |
| } |  |  |  |

Table 7.1.1.1.15.3.3-5: SIB3 on NR Cell 1 (Preamble and Step 9A, Table 7.1.1.1.15.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB3 ::= SEQUENCE { |  |  |  |  |
| intraFreqNeighCellList SEQUENCE { |  |  |  |  |
| physCellId |  | The cell identity of NR Cell 11 defined in 38.508-1 [4] clause 4.4.2 |  |  |
| q-OffsetCell |  | 16 | Preamble |  |
|  |  | -10 | Step 9A |  |
| } |  |  |  |  |
| } |  |  |  |  |

7.1.1.1.16	Random access procedure / RedCap UE identification / Msg3-based / CCCH1

7.1.1.1.16.1	Test Purpose (TP)

<End of modified section 1>

<Start of modified section 2>

<End of modified section 2>