3GPP TSG-RAN5 Meeting #99	R5-232978

# Incheon, South Korea, 22 – 26 May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2273 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding additional spectrum emission mask requirement for new NR band n13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Missing additional spectrum emission mask requirements for NR band n13 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add additional spectrum emission mask requirements for NR band n13 - remove editor’s notes from clause 6.5.2.3 - add NS_07 into clauses 6.5.2.3.3.4, 6.5.2.3.4.3.5 and 6.5.2.3.5.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Additional spectrum emission mask requirements for NR band n13 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TR 38.905... CR 0778 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP Analysis in R5-232976 (CR 0778) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

#### 6.5.2.3	Additional spectrum emission mask

6.5.2.3.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

6.5.2.3.2	Test applicability

This test case applies to all types of NR Power Class 1 UE release 15 and forward.

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

6.5.2.3.3	Minimum conformance requirements

-------- Skipped unchanged clauses --------

6.5.2.3.3.4	Requirements for network signalling value "NS_06" or “NS_07”

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_06" or "NS_07" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.3.4-1.

Table 6.5.2.3.3.4-1: Additional requirements for "NS_06" or "NS_07"

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 |  |
| ± 0 – 0.1 | -15 | -18 | -20 | 30 kHz |
| ± 0.1 – 1 | -13 | -13 | -13 | 100 kHz |
| ± 1 – 6 | -13 | -13 | -13 | 1 MHz |
| ± 6 – 10 | -25 |  |  |  |
| ± 10 – 15 |  | -25 |  |  |
| ± 15 – 20 |  |  | -25 |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.2.3.4.

-------- Skipped unchanged clauses --------

6.5.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6, with the following exceptions for each network signalling value.

6.5.2.3.4.3.1	Message contents exceptions (network signalling value "NS_35")

For "NS_35" see A-MPR test case in table 6.2.3.4.3.2-1.

6.5.2.3.4.3.2	Message contents exceptions (network signalling value "NS_04")

For "NS_04" see A-MPR test case in table 6.2.3.4.3.4-1.

6.5.2.3.4.3.3	Message contents exceptions (network signalling value "NS_03")

For "NS_03" see A-MPR test case in table 6.2.3.4.3.1-1.

6.5.2.3.4.3.4	Message contents exceptions (network signalling value "NS_03U")

For "NS_03U" see A-MPR test case in table 6.2.3.4.3.3-1.

6.5.2.3.4.3.5	Message contents exceptions (network signalling value "NS_06" or “NS_07”)

For "NS_06" see A-MPR test case in table 6.2.3.4.3.7-1. For "NS_07" see A-MPR test case in table 6.2.3.4.3.35-1

6.5.2.3.4.3.6	Message contents exceptions (network signalling value "NS_21")

For "NS_21" see A-MPR test case in table 6.2.3.4.3.20-1.

6.5.2.3.4.3.7	Message contents exceptions (network signalling value "NS_27")

For "NS_27" see A-MPR test case in table 6.2.3.4.3.22-1.

6.5.2.3.5	Test requirement

Table 6.5.2.3.5-1: Test Tolerance (Additional spectrum emission mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 1.5 dB | 1.8 dB | 1.8 dB |

-------- Skipped unchanged clauses --------

6.5.2.3.5.4	Test requirements (network signalling value "NS_06" or “NS_07)

When "NS_06" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in table 6.2.3.5-34 as appropriate for a NR UE

When “NS_07 is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in table 6.2.3.5-37 as appropriate for a NR UE

and

-	the power of any UE emission shall fulfil requirements in table 6.5.2.3.5.4-1, as applicable.

Table 6.5.2.3.5.4-1: Additional requirements for "NS_06" or “NS_07”

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | Measurement bandwidth |
| ± 0 – 0.1 | -15 + TT | -18 + TT | -20 + TT | 30 kHz |
| ± 0.1 – 1 | -13 + TT | -13 + TT | -13 + TT | 100 kHz |
| ± 1 – 6 | -13 + TT | -13 + TT | -13 + TT | 1 MHz |
| ± 6 – 10 | -25 + TT | -13 + TT | -13 + TT | 1 MHz |
| ± 10 – 15 |  | -25 + TT | -13 + TT | 1 MHz |
| ± 15 – 20 |  |  | -25 + TT | 1 MHz |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5.2.3.5-1. |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

-------- End of changes --------