3GPP TSG-RAN5 Meeting #99 	R5-232992

# Incheon, South Korea, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2798 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RRM TRS CSI-ResourceConfig |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on TS 38.508-1 Table 7.3.1-12, if second set of TRS is configured, the number of NZP-CSI-RS-ResourceSet in CSI-ResourceConfig for TRS is 2. However, based on TS 38.331 and TS 38.214, for periodic CSI Resource Setting and groupBasedBeamReporting-v1710 is not configured, the number of CSI-RS Resource Sets configured is limited to 1. <TS 38.331> <TS 38.214> |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 7.3.1-6 and Table 7.3.1-12 were updated such that separate CSI-ResourceConfig is configured per TRS resource set. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance test cannot be performed correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3.1	Radio resource control information elements for RRM

As defined in clause 4.6.3 with the following exceptions:

<<Unchanged tables skipped>>

-	CSI-MeasConfig

Table 7.3.1-6: CSI-MeasConfig for RRM

| Derivation Path: Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | n1+n2+n3+n4+n5 entries | n1=1 if CSI-RS for CSI is configured in test case, n1=0 otherwise; n2=0 if CSI-RS for BM is not configured in the test case. If CSI-RS for BM is configured in the test case, n2 = 2 if 2SSB, n2 = 1 otherwise n3=4 if TRS is configured in test case, n3=0 otherwise; n4=4 if second resource set of TRS is configured in test case, n4=0 otherwise; n5=2 if aperiodic CSI-RS for BM is configured in test case, n5=0 otherwise; |  |
| NZP-CSI-RS-Resource[k, k=1..n1] | NZP-CSI-RS-Resource for CSI | entry 1 | n1>0 |
| NZP-CSI-RS-Resource[k, k=n1+1.. n1+n2] | NZP-CSI-RS-Resource for BM (k-n1-1) | entry ... | n2>0 |
| NZP-CSI-RS-Resource[k, k= n1+n2+1.. n1+n2+n3] | NZP-CSI-RS-Resource for TRS (k-n1-n2) | entry ... | n3>0 |
| NZP-CSI-RS-Resource[k, k= n1+n2+n3+1.. n1+n2+n3+n4] | NZP-CSI-RS-Resource for TRS (k-n1-n2-n3) with condition SECOND_SET | entry ... | n4>0 |
| NZP-CSI-RS-Resource[k, k= n1+n2+n3+n4+1.. n1+n2+n3+n4+n5] | NZP-CSI-RS-Resource for BM (k-n1-n2-n3-n4) with condition APERIODIC | entry ... | n5>0 |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | m1+m2+m3+m4+m5 entries | mi=1 if ni>0, mi=0 otherwise |  |
| NZP-CSI-RS-ResourceSet[k, k=1..m1] | NZP-CSI-RS-ResourceSet for CSI | entry 1 | n1>0 |
| NZP-CSI-RS-ResourceSet[k, k=m1+1.. m1+m2] | NZP-CSI-RS-ResourceSet for BM | entry ... | n2>0 |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+1.. m1+m2+m3] | NZP-CSI-RS-ResourceSet for TRS | entry ... | n3>0 |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+m3+1.. m1+m2+m3+m4] | NZP-CSI-RS-ResourceSet for TRS with condition SECOND_SET | entry ... | n4>0 |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+m3+m4+1.. m1+m2+m3+m4+m5] | NZP-CSI-RS-ResourceSet for BM with condition APERIODIC | entry ... | n5>0 |
| } |  |  |  |
| csi-IM-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource { | 1 entry |  | n1>0 |
| CSI-IM-Resource[1] | CSI-IM-Resource-RRM | entry 1 |  |
| } |  |  |  |
| csi-IM-ResourceToAddModList | Not present |  | n1=0 |
| csi-IM-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet { | 1 entry |  | n1>0 |
| CSI-IM-ResourceSet[1] | CSI-IM-ResourceSet-RRM | entry 1 |  |
| } |  |  |  |
| csi-IM-ResourceSetToAddModList | Not present |  | n1=0 |
| csi-SSB-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet { |  |  |  |
| CSI-SSB-ResourceSet[1] | CSI-SSB-ResourceSet-RRM | entry 1 |  |
| } |  |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | p1+m2+m3+m4+m5 entries | p1=m1+1 if n1>0, p1=0 otherwise. |  |
| CSI-ResourceConfig[k, k=1..m1] | CSI-ResourceConfig for CSI | entry 1 | n1>0 |
| CSI-ResourceConfig[k, k=p1] | CSI-ResourceConfig for CSI-IM | entry ... | n1>0 |
| CSI-ResourceConfig[k, k=p1+1.. p1+m2] | CSI-ResourceConfig for BM | entry ... | n2>0 |
| CSI-ResourceConfig[k, k= p1+m2+1.. p1+m2+m3] | CSI-ResourceConfig for TRS | entry ... | n3>0 |
| CSI-ResourceConfig[k, k= p1+m2+m3+1.. p1+m2+m3+m4] | CSI-ResourceConfig for TRS with condition SECOND_SET | entry ... | n4>0 |
| CSI-ResourceConfig[k, k= p1+m2+m3+m4+1.. p1+m2+m3+m4+m5] | CSI-ResourceConfig for BM with condition APERIODIC | entry ... | n5>0 |
| } |  |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { | r1+r2+r5+s1 entries | r1 = 1 if CSI Reporting for CSI is configured in test case, r1 = 0 otherwise; r2 = 1 if CSI Reporting for BM is configured in test case, r2 = 0 otherwise; r5 = 1 if aperiodic CSI Reporting for BM is configured in test case, r5 = 0 otherwise; s1 = 1 if: - this CSI-MeasConfig is configured on NR SpCell, and, - CSI-RS for CSI is configured on SCell in TC, and, - SCell is not a PUCCH-SCell Otherwise s1 = 0. |  |
| CSI-ReportConfig[k, k=1..r1] | CSI-ReportConfig for CSI | entry 1 | r1>0 |
| CSI-ReportConfig[k, k=r1+1.. r1+r2] | CSI-ReportConfig for BM | entry ... | r2>0 |
| CSI-ReportConfig[k, k=r1+r2+1.. r1+r2+r5] | CSI-ReportConfig for BM with condition APERIODIC | entry ... | r5>0 |
| CSI-ReportConfig[k, k=r1+r2+r5+1.. r1+r2+r5+s1+1] | CSI-ReportConfig for CSI with condition SCELL_CSI_ON_SPCELL | entry ... | s1>0 |
| } |  |  |  |
| reportTriggerSize | Not present |  |  |
|  | 1 |  | n5>0 |
| aperiodicTriggerStateList CHOICE { | Not present |  |  |
| setup | CSI-AperiodicTriggerStateList |  | n5>0 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2SSB | For configuration with 2 SS Blocks |

<<Unchanged tables skipped>>

–	CSI-ResourceConfig for TRS

Table 7.3.1-12: CSI-ResourceConfig for TRS

| Derivation Path: TS 38.508-1 Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId-TRS |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | NZP-CSI-RS-ResourceSetId-TRS |  |  |
|  | NZP-CSI-RS-ResourceSetId-TRS with condition SECOND_SET |  | SECOND_SET |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SECOND_SET | For the second resource set for TRS configured in test, only applies to FR2 test |

<<End of change>>