3GPP TSG-RAN5 Meeting #99 	R5-233006

# Incheon, South Korea, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0694 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to K1 settings in 5.5A.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_perf_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current default K1 settings of 5.5A.1.1 are not applicable for CA band combinations FDD15kHz (PCell) - TDD30kHz (SCell) and TDD30kHz (PCell) - FDD15kHz (SCell). - For FDD15kHz-TDD30kHz, default K1 settings of TDD Scell lead current HARQ Process (=8) to be not enough. - For TDD30kHz-FDD15kHz, default K1 settings of FDD Scell do not fall into TDD Pcell's UL slot. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new message exception Table 5.5A.1.1.4.3-4: Physical layer parameters for DCI format 1_1 - For FDD15-TDD30, proposed K1 values = {3} (cannot be similar to Demod CA {2} because has UL RMC with K2=2. As per TS 38.213 cl.9.1.2.2 "A UE sets to NACK value in the HARQ-ACK codebook any HARQ-ACK information corresponding to PDSCH reception or SPS PDSCH release or TCI state update that the UE detects in a PDCCH monitoring occasion that starts after a PDCCH monitoring occasion where the UE detects a DCI format scheduling the PUSCH transmission.") - For TDD30-FDD15, proposed K1 values = {7,5,4,11,9} (similar to Demod CA) Added new message exception Table 5.5A.1.1.4.3-5: PDSCH-ServingCellConfig - For TDD30-FDD15, proposed nrofHARQ-ProcessesForPDSCH = {8} for FDD SCell due to changes in K1 values |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect K1 values will be used. ACK-NACK might not be transmitted/received accordingly. SDR test would not be carried out with the proper messages. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

5.5A.1.1	FR1 SDR performance for CA (2DL CA)

5.5A.1.1.1	Test Purpose

The purpose of the test is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. The sustained downlink data rate shall be verified in terms of the success rate of delivered PDCP SDU(s) by Layer 2. The test case below specifies the RF conditions and the required success rate of delivered TB by Layer 1 to meet the sustained data rate requirement.

5.5A.1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supports 2DL CA

5.5A.1.1.3	Minimum conformance requirements

The Sustained Data Rate (SDR) requirements in this clause are applicable to the FR1 CA.

The TB success rate shall be higher than 85% when PDSCH is scheduled with MCS defined for the selected CA bandwidth combination and with the downlink physical channel setup according to Annex C.2.1.

The TB success rate is defined as 100%*NDL_correct_rx / (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks.

The common test parameters are specified in Table 5.5A.1.1.3-1. The parameters specified in Table 5.5A.1.1.3-2 are applicable for tests on FDD CCs and parameters specified in Table 5.5A.1.1.3-3 are applicable for tests on TDD CCs.

Unless otherwise stated, no user data is scheduled on slot #0, 10 and 11 within 20 ms for SCS 15 kHz.

Unless otherwise stated, no user data is scheduled on slot #0, 20 and 21 within 20 ms for SCS 30 kHz.

Table 5.5A.1.1.3-1: Common test parameters for FDD and TDD component carriers

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| EPRE ratio of PTRS to PDSCH |  |  | dB | N/A |
| Channel bandwidth |  |  | MHz | Channel bandwidth from selected CA bandwidth combination |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
| Cross carrier scheduling |  |  |  | Not configured |
| Active DL BWP index |  |  |  | 1 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
| DL BWP configuration #1 | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  |  | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
|  | Cyclic prefix |  |  | Normal |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  |  | Symbols #0 |
|  | Number of PRBs in CORESET |  |  | Table 5.5A-4 |
|  | Number of PDCCH candidates and aggregation levels |  |  | 1/AL 1 for 30 kHz / 5 MHz 1/AL4 for 15 kHz / 5 MHz, 30 kHz / 10 MHz and 30 kHz / 15 MHz 1/AL 8 for other combinations |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI State |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | For number of Tx=1: No precoding;For 2: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of precoder indices 0 and 2 For number of Tx=4: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of i_1,1 in {1,2,3,5,6,7} and i_2 in {0,2} |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | wideband |
|  | Resource allocation type |  |  | Type 0 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 1 |
|  | Length |  |  | 1 |
|  | Antenna ports indexes |  |  | {1000} for 1 Layer CCs {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for 1 layer and 2 layers CCs 2 for 4 Layers CCs |
| PTRS configuration |  |  |  | PTRS is not configured |
| CSI-RS for tracking | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 3 for CSI-RS resource 1,2,3,4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size /4)*4 |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size /4)*4 |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | For number of Tx=1: No precoding; For number of Tx>1: Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12] |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 OP.1 TDD as defined in Annex A.5.2.1 |
| Propagation condition |  |  |  | Static propagation condition No external noise sources are applied |
| Antenna configuration | 1 layer CCs |  |  | 1x2 or 1x4 |
|  | 2 layers CCs |  |  | 2x2 or 2x4 |
|  | 4 layers CCs |  |  | 4x4 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing |  |  |  |  |

Table 5.5A.1.1.3-2: Additional test parameters for FDD CC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| PDSCH configuration | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
| Number of HARQ Processes |  |  | 4 |
| K1 value |  |  | 2 |

Table 5.5A.1.1.3-3: Additional test parameters for TDD CC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| PDSCH configuration | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
| Number of HARQ Processes |  |  | 8 |
| K1 value |  |  | Specific to each UL-DL pattern |
| TDD UL-DL pattern |  |  | 15 kHz SCS: FR1.15-1 30 kHz SCS: FR1.30-1 |
| Note 1: PDSCH is scheduled only on full DL slots |  |  |  |

Table 5.5A.1.1.3-4: Number of PRBs in CORESET

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 24 | 48 | 78 | 102 | 132 | 156 | 186 | 216 | 240 | 270 | N/A | N/A | N/A |
| 30 | 6 | 24 | 36 | 48 | 60 | 78 | 90 | 102 | 114 | 132 | 162 | 216 | 270 |

Table 5.5A.1.1.3-5: MCS indexes for indicated UE capabilities

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 1 | 8 | 1 | 26 |
| 1 | 8 | 0.8 | 21 |
| 1 | 8 | 0.75 | 20 |
| 1 | 8 | 0.4 | 11 |
| 1 | 6 | 1 | 27 |
| 1 | 6 | 0.8 | 23 |
| 1 | 6 | 0.75 | 22 |
| 1 | 6 | 0.4 | 14 |
| 1 | 4 | 1 | 16 |
| 1 | 4 | 0.8 | 16 |
| 1 | 4 | 0.75 | 16 |
| 1 | 4 | 0.4 | 10 |
| 1 | 2 | 1 | 9 |
| 1 | 2 | 0.8 | 9 |
| 1 | 2 | 0.75 | 9 |
| 1 | 2 | 0.4 | 4 |
| 2 | 8 | 1 | 26 |
| 2 | 8 | 0.8 | 21 |
| 2 | 8 | 0.75 | 20 |
| 2 | 8 | 0.4 | 11 |
| 2 | 6 | 1 | 27 |
| 2 | 6 | 0.8 | 23 |
| 2 | 6 | 0.75 | 22 |
| 2 | 6 | 0.4 | 14 |
| 2 | 4 | 1 | 16 |
| 2 | 4 | 0.8 | 16 |
| 2 | 4 | 0.75 | 16 |
| 2 | 4 | 0.4 | 10 |
| 2 | 2 | 1 | 9 |
| 2 | 2 | 0.8 | 9 |
| 2 | 2 | 0.75 | 9 |
| 2 | 2 | 0.4 | 4 |
| 4 | 8 | 1 | 26 |
| 4 | 8 | 0.8 | 23 |
| 4 | 8 | 0.75 | 22 |
| 4 | 8 | 0.4 | 12 |
| 4 | 6 | 1 | 27 |
| 4 | 6 | 0.8 | 24 |
| 4 | 6 | 0.75 | 23 |
| 4 | 6 | 0.4 | 14 |
| 4 | 4 | 1 | 16 |
| 4 | 4 | 0.8 | 16 |
| 4 | 4 | 0.75 | 16 |
| 4 | 4 | 0.4 | 11 |
| 4 | 2 | 1 | 9 |
| 4 | 2 | 0.8 | 9 |
| 4 | 2 | 0.75 | 9 |
| 4 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

5.5A.1.1.3.1	Procedure for test parameter selection

The test parameters are determined by the following procedure:

-	Select one CA bandwidth combination among all supported CA configurations and set of per component carrier (CC) UE capabilities among all supported UE capabilities that provides the largest data rate in accordance with clause 4.1.2 of TS 38.306 [14].

-	Set of per CC UE capabilities includes channel bandwidth, subcarrier spacing, number of PDSCH MIMO layers, modulation format and scaling factor in accordance with clause 4.1.2 of TS 38.306 [14].

-	When there are multiple sets of CA bandwidth combinations and UE capabilities (channel bandwidth, subcarrier spacing, number of MIMO layer, modulation format, scaling factor) with same largest data rate, select one among sets with the smallest aggregated channel bandwidth.

-	For each CC in CA bandwidth combination, use Table 5.5A.1.1.3-5 to determine MCS based on test parameters and indicated UE capabilities.

The normative reference for this requirement is TS 38.101-4 [5], clause 5.5A.1.

5.5A.1.1.4	Test description

5.5A.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of TS 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 5.2.2.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

1.	Connect the SS  to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.2A, A.3.1.7.6, and A.3.1.7.7 for TE diagram (without fader and AWGN) for 2Rx and 4Rx CC(s) respectively and clause A.3.2.6 for UE diagram.

2.	The parameter settings for the NR cell are initially set up according to Table 5.5.1.3-1 as appropriate.

3.	Downlink signals for the NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions for the NR cell are set according to Annex B.1.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR with Connected without release On, Test Loop Function On with UE Test Loop Mode A with UL_PDCP_SDU_SIZE = 0 according to TS 38.508-1 [6] clause 4.5.4. Message content are defined in clause 5.5 A.1.1.4.3.

6.	Configure SCC(s) as applicable  according to Annex C.0, C.1 and C.2 for all downlink physical channels.

7.	The SS shall configure SCC (s) as applicable as per TS 38.508-1 [6] clause 5.5.1.

8.	SS activates SCC(s) as applicable by sending the activation MAC-CE (Refer TS 38.321 [24], clauses 5.9, 6.1.3.10). Wait for at least 1 second (Refer TS 38.133[25], clause9.3).

9.	SS shall transmit UECapabilityEnquiry message.

10.	The UE shall transmit UECapabilityInformation message.

11.	Using the UE capabilities advertised in the UE-CapabilityRAT-Container of the type UE-NR-Capability, and the procedure outlined in 5.5A.1.1.3.1 determine one set of parameters that would provide the largest data rate.

12.	Setup up the NR cells using these parameters for the test.

13.	Configure the TBsize, DL RMC, UL RMC, PDCP size from Annex A.3.2_1 and Annex A.2.2 for UL as appropriate.

5.5A.1.1.4.2	Test procedure

1.	SS configures T-reordering timer to be infinity.

2.	SS sends a PDCP reestablishment via RRC Reconfiguration message requesting for PDCP Status Report.

3.	SS sets the counters NDL_newtx NDL_retx to 0.

4.	For each new DL HARQ transmission the SS generates sufficient PDCP SDUs (max PDCP SDU size and minimum  number of consecutive PDCP SDUs) to fill up the TB on both PCC and SCC(s) as applicable in accordance with Annex A.3.2_1. The SS ciphers the PDCP SDUs, concatenates the resultant PDCP PDUs to form an RLC PDU and then a MAC PDU. The SS transmits the MAC PDU. The SS increments then NDL_newtx by one

5.	If PHY requests a DL HARQ retransmission, the SS performs a HARQ retransmission and increments NDL_retx by one.

6.	Steps 5 to 6 are repeated at every TTI for at least 300 frames and the SS waits for 300ms to let any HARQ retransmissions and RLC retransmissions to finish.

7.	SS sends a PDCP reestablishment via RRC Reconfiguration message requesting for PDCP Status Report.

8.	The SS calculates the TB success rate as A = 100%* NDL_correct_rx / (NDL_newtx + NDL_retx).

9.	SS computes the PDCP SDU loss by looking into the FMC and Bitmap field in the PDCP Status Report. PDCP SDU loss B = COUNT reported in the Bitmap field of PDCP Stata Report.

10.	The UE passes the test if A ≥  85% TB success rate on both PCC and SCC and B = 0.

Note 1:	In case of RLC PDU retransmission, the number of new required PDCP SDUs is as many as to fill the rest of TB.

5.5A.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 5.4.2 with the following exceptions:

Table 5.5A.1.1.4.3-1: CLOSE UE TEST LOOP (in the preamble)

| Derivation Path: TS 38.509 [9] clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 0 0 0 0 0 0 |  |  |
| UE test loop mode |  | 0 0 0 0 0 0 0 0 | UE test loop mode A |  |
| UE test loop mode A LB setup |  |  |  |  |
| Length of UE test loop mode A LB setup list in bytes |  | 0 0 0 0 0 0 1 1 | Length of one LB setup DRB (3 bytes) |  |
| LB setup DRB |  | 0 0 0 0 0 0 0 0, 0 0 0 0 0 0 0 0, 0 0 Q5 Q4 Q3 Q2 Q1 Q0 | UL PDCP SDU size = 0 Q5 = 1 (for NR Data Radio Bearers) Q4..Q0 = Data Radio Bearer identity number -1 for the radio bearer. See 38.509 clause 6.3.1 |  |
| UE test loop mode B LB setup |  | Not present |  |  |

Table 5.5A.1.1.4.3-2: RadioBearerConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { | 1 entry |  | DRB1 |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB1 |  |  |
| reestablishPDCP | true |  | DRB1 AND Re-establish_PDCP |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |

Table 5.5A.1.1.4.3-3: PDCP-Config

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| discardTimer | infinity |  |  |
| pdcp-SN-Size-UL | len18bits |  |  |
| pdcp-SN-Size-DL | len18bits |  |  |
| headerCompression CHOICE { |  |  |  |
| notUsed | Null |  |  |
| } |  |  |  |
| integrityProtection | Not present |  |  |
| statusReportRequired | true |  |  |
| outOfOrderDelivery | Not present |  |  |
| } |  |  |  |
| t-Reordering | Not present |  |  |
| } |  |  |  |

Table 5.5A.1.1.4.3-4: Physical layer parameters for DCI format 1_1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PDSCH-to-HARQ_feedback timing indicator | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 For FDD 15kHz SCell: K1 = 7 if mod(i,5) = 0 K1 = 5 if mod(i,5) = 1 K1 = 4 if mod(i,5) = 2 K1 = 11 if mod(i,5) = 3 K1 = 9 if mod(i,5) = 4 where i is slot index per frame; i = {0,…,19} | - | (For FDD CC if configured as SCell with TDD PCell) |
|  | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 For TDD30kHz SCell: K1 = 3 |  | (For TDD CC if configured as SCell with FDD PCell) |

Table 5.5A.1.1.4.3-5: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | Not present | n8 | (For FDD CC if configured as SCell with TDD PCell) |
| } |  |  |  |

5.5A.1.1.5	Test requirement

The TB success rate of greater than 85% with no PDCP SDU loss shall be sustained during at least 300 frames on each CC.

<<End of change>>