3GPP TSG-RAN5 Meeting #99	R5-233052

# Incheon, Korea, May 22nd – 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0710 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MU and TT for HST scheme A and B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Demode Test cases for HST scheme A and B are introduced in this meeting. The relevante MU and TT could reuse those of existing HST test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding MU/TT for HST scheme A and B test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

F.1.1.2	Measurement of Demod Performance requirements

This clause defines the maximum test system uncertainty for Demod Performance requirements. The maximum test system uncertainty allowed for the measurement uncertainty contributors are defined in Table F.1.1.2-1.

Table F.1.1.2-1: Maximum measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Channel BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWconfig | dB | ±2.0 | Same as in LTE |
| Signal to noise ratio uncertainty | dB | ±0.3 | Same as in LTE |
| Signal to noise ratio variation | dB | ±0.5 | Same as in LTE |
| Fading profile power uncertainty for 1Tx | dB | ±0.5 | Same as in LTE |
| Fading profile power uncertainty for 2Tx | dB | ±0.7 | Same as in LTE |
| Fading profile power uncertainty for 4Tx | dB | ±0.7 | Same as in LTE |

The maximum test system uncertainty for test cases defined in section 5 is defined in Table F.1.1.2-2.

Table F.1.1.2-2: Maximum test system uncertainty for FR1 demodulation performance test cases

| Subclause |  |  |  |  | Maximum Test System Uncertainty |  |  |  |  | Derivation of Test System Uncertainty |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.2.1.1.1 1 Rx FDD FR1 PDSCH performance for RedCap |  |  |  |  | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.1_1 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler ± 0.6 dB for test 1-6 ± 0.9 dB for test 1-7 |  |  |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) + SNR uncertainty due to finite test time2 Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.3 dB for 10Hz Doppler, otherwise ±0.0 dB For test point 1-6, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.6 dB For test point 1-7, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.9 dB |  |  |  |  |
| 5.2.2.1.1_2 2Rx FDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.2_1 2Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.3_1 2Rx FDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.4_1 2Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.5_1 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | [± 0.6 dB] |  |  |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |  |  |
| 5.2.2.1.6_1 2Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | [0.7dB] |  |  |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness 3. SNR uncertainty due to finite test time Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2+ SNR uncertainty due to finite test time2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±[0.4] dB for 1% residual BLER |  |  |  |  |
| 5.2.2.1.7_1 2Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.8_1 2Rx FDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.9_1 2Rx FDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | ± 0.6 dB |  |  |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty, ±0.3 dB 2. Effect of AWGN flatness and signal flatness, ±2.0 dB 3. SNR uncertainty due to finite test time, ±0.0 dB for >10Hz Doppler. Items 1, 2, 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.6 dB |  |  |  |  |
| 5.2.2.1.10_1 2Rx FDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | ± 0.6 dB |  |  |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty, ±0.3 dB 2. Effect of AWGN flatness and signal flatness, ±2.0 dB 3. SNR uncertainty due to finite test time, ±0.0 dB for >10Hz Doppler. Items 1, 2, 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.6 dB |  |  |  |  |
| 5.2.2.1.11_1 2Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.12_1 2Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.13_1 2Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.14_1 2Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  |
| 5.2.2.1.20 2Rx FDD FR1 PDSCH HST-SFN Scheme A performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  |
| 5.2.2.1.21 2Rx FDD FR1 PDSCH HST-SFN Scheme B performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  |
| 5.2.2.2.1_1 2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | ± 0.9 dB for test 1-10 ± 0.6 dB for test 1-11 For other TPs, same as 5.2.2.1.1_1 |  |  |  |  | For test point 1-10, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.9 dB For test point 1-11, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.6 dB For other TPs, same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.1_2 2Rx TDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.2_1 2Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.3_1 2Rx TDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.4_1 2Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.5_1 2Rx TDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.5_1 |  |  |  |  | Same as 5.2.2.1.5_1 |  |  |  |  |
| 5.2.2.2.6_1 2Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  |
| 5.2.2.2.7_1 2Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.8_1 2Rx TDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.9_1 2Rx TDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  |
| 5.2.2.2.10_1 2Rx TDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.10_1 |  |  |  |  | Same as 5.2.2.1.10_1 |  |  |  |  |
| 5.2.2.2.11_1 2Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.12_1 2Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.13_1 2Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.14_1 2Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  |
| 5.2.2.2.21 2Rx TDD FR1 PDSCH HST-SFN Scheme A performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  |
| 5.2.2.2.21 2Rx TDD FR1 PDSCH HST-SFN Scheme B performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  |
| 5.2.3.1.1_1 4Rx FDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | ± 0.9 dB for > 10Hz doppler ± 1.0 dB for 10Hz doppler |  |  |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.3 dB for 10Hz Doppler, otherwise ±0.0 dB |  |  |  |  |
| 5.2.3.1.1_2 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.1_4 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with enhanced receiver type 1 for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.2_1 4Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 4x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.3_1 4Rx FDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.4_1 4Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.5_1 4Rx FDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.5_1 |  |  |  |  | Same as 5.2.2.1.5_1 |  |  |  |  |
| 5.2.3.1.6_1 4Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  |
| 5.2.3.1.7_1 4Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.8_1 4Rx FDD FR1 PDSCH pre-emption performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.9_1 4Rx FDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  |
| 5.2.3.1.10_1 4Rx FDD FR1 HST-DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.10_1 |  |  |  |  | Same as 5.2.2.1.10_1 |  |  |  |  |
| 5.2.3.1.11_1 4Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x4 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.12_1 4Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x4 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.13_1 4Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x4 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.14_1 4Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x4 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.6_1 |  |  |  |  | Same as 5.2.3.1.6_1 |  |  |  |  |
| 5.2.3.1.19 4Rx FDD FR1 PDSCH HST-SFN Scheme A performance - 2x4 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.9_1 |  |  |  |  | Same as 5.2.3.1.9_1 |  |  |  |  |
| 5.2.3.1.20 4Rx FDD FR1 PDSCH HST-SFN Scheme B performance - 2x4 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.9_1 |  |  |  |  | Same as 5.2.3.1.9_1 |  |  |  |  |
| 5.2.3.2.1_1 4Rx TDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.1_2 4Rx TDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.1_4 4Rx TDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with enhanced receiver type 1 for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.2_1 4Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.3_1 4Rx TDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.4_1 4Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.5_1 4Rx TDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.5_1 |  |  |  |  | Same as 5.2.2.1.5_1 |  |  |  |  |
| 5.2.3.2.6_1 4Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  |
| 5.2.3.2.7_1 4Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.9_1 4Rx TDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.9_1 |  |  |  |  | Same as 5.2.3.1.9_1 |  |  |  |  |
| 5.2.3.2.10_1 4Rx TDD FR1 HST DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.10_1 |  |  |  |  | 5.2.3.1.10_1 |  |  |  |  |
| 5.2.3.2.11_1 4Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.12_1 4Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.13_1 4Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.14_1 4Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.6_1 |  |  |  |  | Same as 5.2.3.1.6_1 |  |  |  |  |
| 5.2.3.2.20 4Rx TDD FR1 PDSCH HST-SFN Scheme A performance - 2x4 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.2.9_1 |  |  |  |  | Same as 5.2.3.2.9_1 |  |  |  |  |
| 5.2.3.2.21 4Rx TDD FR1 PDSCH HST-SFN Scheme B performance - 2x4 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.2.9_1 |  |  |  |  | Same as 5.2.3.2.9_1 |  |  |  |  |
| 5.2A.2.1.1 2Rx Normal PDSCH Demodulation Performance for CA (2DL CA) |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2A.2.1.2 2Rx Normal PDSCH Demodulation Performance for CA (3DL CA) |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2A.2.1.3 2Rx Normal PDSCH Demodulation Performance for CA (4DL CA) |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2A.2.2.1 2Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA) |  |  |  |  | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Downlink EVM ≤ 6% |  |  |  |  | 6% EVM is equivalent to a Test system downlink SNR of 24.4dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |  |  |  |  |
| 5.2A.2.4.1 2Rx PDSCH Demodulation Performance for HST-SFN CA |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  |
| 5.2A.2.5.1 2RX PDSCH Demodulation Performance for HST-DPS CA |  |  |  |  | Same as 5.2.2.1.10_1 |  |  |  |  | Same as 5.2.2.1.10_1 |  |  |  |  |
| 5.2A.3.1.1 4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2A.3.1.2 4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2A.3.1.3 4Rx Normal PDSCH Demodulation Performance for CA (4DL CA) |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2A.3.2.1 4Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA) |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2A.3.4.1 4Rx PDSCH Demodulation Performance for HST-SFN CA |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  |
| 5.2A.3.5.1 4Rx PDSCH Demodulation Performance for HST-DPS CA |  |  |  |  | Same as 5.2.2.1.10_1 |  |  |  |  | Same as 5.2.2.1.10_1 |  |  |  |  |
| 5.2A.3A.1.1 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) |  |  |  |  | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC |  |  |  |  | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC |  |  |  |  |
| 5.2A.3A.1.2 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) |  |  |  |  | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC |  |  |  |  | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC |  |  |  |  |
| 5.2A.3A.1.3 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) |  |  |  |  | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC |  |  |  |  | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC |  |  |  |  |
| 5.3.2.1.1 2Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA |  |  |  |  | ± 0.9 dB |  |  |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2+ SNR uncertainty due to finite test time2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for 1Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.4 dB |  |  |  |  |
| 5.3.2.1.2 2Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA |  |  |  |  | ± 1.0 dB |  |  |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2+ SNR uncertainty due to finite test time2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.4 dB |  |  |  |  |
| 5.3.2.1.3 2Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  |
| 5.3.2.2.1 2Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  |
| 5.3.2.2.2 2Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  |
| 5.3.2.2.3 2Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  |
| 5.3.3.1.1 4Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  |
| 5.3.3.1.2 4Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  |
| 5.3.3.1.3 4Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  |
| 5.3.3.2.1 4Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  |
| 5.3.3.2.2 4Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  |
| 5.3.3.2.3 4Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  |
| 5.5A.1.1 FR1 SDR performance for CA (2DL CA) |  |  |  |  | Same as 5.5.1 |  |  |  |  | Same as 5.5.1 |  |  |  |  |
| 5.5A.1.2 FR1 SDR performance for CA (3DL CA) |  |  |  |  | Same as 5.5.1 |  |  |  |  | Same as 5.5.1 |  |  |  |  |
| 5.5A.1.3 FR1 SDR performance for CA (4DL CA) |  |  |  |  | Same as 5.5.1 |  |  |  |  | Same as 5.5.1 |  |  |  |  |
| 5.5A.1.4 FR1 SDR performance for CA (5DL CA) |  |  |  |  | Same as 5.5.1 |  |  |  |  | Same as 5.5.1 |  |  |  |  |
| 5.5.1 FR1 Sustained downlink data rate performance for single carrier |  |  |  |  | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Downlink EVM ≤ 3% |  |  |  |  | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |  |  |  |  |
| 9.4B.1.1 Sustained downlink data rate performance for EN-DC within FR1 |  |  |  |  | E-UTRA CC: ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz NR CC: Same as 5.5.1 |  |  |  |  | Same as 5.5.1 |  |  |  |  |
| 11.1.2.1.1_1 2Rx FR1 PSSCH performance - single active PSSCH link |  |  |  |  | ± 0.8 dB |  |  |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |  |  |  |  |
| 11.1.3.1.1_1 2Rx FR1 PSCCH performance - single active PSSCH link |  |  |  |  | Same as 11.1.2.1.1_1 |  |  |  |  | Same as 11.1.2.1.1_1 |  |  |  |  |
| 11.1.5.1.1_1 2Rx FR1 PSCCH performance - single active PSSCH link |  |  |  |  | Same as 11.1.2.1.1_1 |  |  |  |  | Same as 11.1.2.1.1_1 |  |  |  |  |
| 11.1.6.1.1_1 2Rx FR1 Power imbalance performance - two active PSSCH link |  |  |  |  | ± 0.6 dB |  |  |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1, and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |  |  |
| 11.1.7.1.1_1 2Rx FR1 HARQ buffer soft combining performance - maximum number of HARQ processes |  |  |  |  | Same as 11.1.2.1.1_1 |  |  |  |  | Same as 11.1.2.1.1_1 |  |  |  |  |
| 11.1.8.1.1_1 2Rx FR1 PSCCH decoding capability - maximum number of received PSCCHs |  |  |  |  | Downlink absolute power uncertainty, averaged over BWConfig ±1.0 dB Downlink EVM ≤ 3% |  |  |  |  | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |  |  |  |  |
| 11.1.9.1.1_1 2Rx FR1 PSFCH decoding capability - maximum number of received PSFCHs |  |  |  |  | Same as 11.1.8.1.1_1 |  |  |  |  | Same as 11.1.8.1.1_1 |  |  |  |  |

<Unchanged Text Skipped>

F.1.3.2	Measurement of Demod Performance requirements

The derivation of the test requirements for the test cases in section 5 is defined in Table F.1.3.2-1.

Table F.1.3.2-1: Derivation of Test Requirements (FR1 demodulation performance tests)

| Test |  |  | Minimum Requirement in TS 38.101-4 |  | Test Tolerance (TT) |  | Test Requirement in TS 38.521-4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.2.1.1.1 1 Rx FDD FR1 PDSCH performance for RedCap |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.1.2.1 1Rx TDD FR1 PDSCH performance for RedCap |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.1_1 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler 0.6 dB for test 1-6 0.9 dB for test 1-7 |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.1_2 2Rx FDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.2_1 2Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.3_1 2Rx FDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.4_1 2Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.5_1 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.6_1 2Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified |  | [0.7] |  | Formula: SNR + TT |  |
| 5.2.2.1.7 2Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.8_1 2Rx FDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.9_1 2Rx FDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.10_1 2Rx FDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.11_1 2Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.12_1 2Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.13_1 2Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.14_1 2Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | [0.7] |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.17 2Rx FDD FR1 PDSCH performance for RedCap |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.20 2Rx FDD FR1 PDSCH HST-SFN Scheme A performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.21 2Rx FDD FR1 PDSCH HST-SFN Scheme B performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.1_1 2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler 0.9 dB for test 1-10 0.6 dB for test 1-11 |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.1_2 2Rx TDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.2_1 2Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.3_1 2Rx TDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.4_1 2Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.5_1 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | [0.6 dB] |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.6_1 2Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | [0.6 dB] |  | Formula: SNR + TT |  |
| 5.2.2.2.7_1 2Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.8_1 2Rx TDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.9_1 2Rx TDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.10_1 2Rx TDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.11_1 2Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.12_1 2Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.13_1 2Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.14_1 2Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | [0.7dB] |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.18 2Rx TDD FR1 PDSCH performance for RedCap |  |  | SNRs as specified |  | 0.9 dB for > 10Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.21 2Rx TDD FR1 PDSCH HST-SFN Scheme A performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.22 2Rx TDD FR1 PDSCH HST-SFN Scheme B performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.1_1 4Rx FDD FR1 PDSCH mapping Type A performance - 2x4 MIMO baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.1_2 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.1_4 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with enhanced receiver type 1 for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.2_1 4Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 4x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.3_1 4Rx FDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.4_1 4Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.5_1 4Rx FDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | [0.6 dB] |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.6_1 4Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | [0.7dB] |  | Formula: SNR + TT |  |
| 5.2.3.1.7_1 4Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.8_1 4Rx FDD FR1 PDSCH pre-emption performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.9_1 4Rx FDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.10_1 4Rx FDD FR1 HST-DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.11_1 4Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x4 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.12_1 4Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x4 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.13_1 4Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x4 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.14_1 4Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x4 MIMO for both SA and NSA |  |  | SNRs as specified |  | [0.7dB] |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.19 4Rx FDD FR1 PDSCH HST-SFN Scheme A performance - 2x4 MIMO for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.20 4Rx FDD FR1 PDSCH HST-SFN Scheme B performance - 2x4 MIMO for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.2_1 4Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.3_1 4Rx TDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.4_1 4Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.5_1 4Rx TDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | [0.6 dB] |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.6_1 4Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | [0.7dB] |  | Formula: SNR + TT |  |
| 5.2.3.2.7_1 4Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.8_1 4Rx TDD FR1 PDSCH pre-emption performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.9_1 4Rx TDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.10_1, 4Rx TDD FR1 HST DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.11_1 4Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.12_1 4Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.13_1 4Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.14_1 4Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA |  |  | SNRs as specified |  | [0.7dB] |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.20 4Rx TDD FR1 PDSCH HST-SFN Scheme A performance - 2x4 MIMO for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.21 4Rx TDD FR1 PDSCH HST-SFN Scheme B performance - 2x4 MIMO for both SA and NSA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.2.1.1 2Rx Normal PDSCH Demodulation Performance for CA (2DL CA) |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.2.1.2 2Rx Normal PDSCH Demodulation Performance for CA (3DL CA) |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.2.1.3 2Rx Normal PDSCH Demodulation Performance for CA (4DL CA) |  |  | SNRs as specified |  | 1.0 dB for 10Hz doppler |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.2.2.1 2Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA) |  |  | Power level as specified |  | No TT added |  | T-put limit unchanged |  |
| 5.2A.2.4.1 2RX PDSCH Demodulation Performance for HST-SFN CA |  |  | SNR as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.2.5.1 2RX PDSCH Demodulation Performance for HST-DPS CA |  |  | SNR as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.1.1 4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.1.2 4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.1.3 4Rx Normal PDSCH Demodulation Performance for CA (4DL CA) |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.2.1 4Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA) |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.4.1 4Rx PDSCH Demodulation Performance for HST-SFN CA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.5.1 4Rx PDSCH Demodulation Performance for HST-DPS CA |  |  | SNRs as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3A.1.1 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) |  |  | SNRs as specified |  | 2Rx CC: 1.0 dB for 10Hz doppler 4Rx CC: 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3A.1.2 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) |  |  | SNRs as specified |  | 2Rx CC: 1.0 dB for 10Hz doppler 4Rx CC: 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3A.1.3 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) |  |  | SNRs as specified |  | 2Rx CC: 1.0 dB for 10Hz doppler 4Rx CC: 0.9dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.1.1 2Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.1.2 2Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.1.3 2Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.2.1 2Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.2.2 2Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.2.3 2Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.1.1 4Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.1.2 4Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.1.3 4Rx FDD FR1 PDCCH 1 Tx antenna performance power saving |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.2.1 4Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA |  |  | SNRs as specified |  | 0.9 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.2.2 4Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA |  |  | SNRs as specified |  | 1.0 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 11.1.2.1.1_1 2Rx FR1 PSSCH performance - single active PSSCH link |  |  | SNR as specified |  | 0.8 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 11.1.3.1.1_1 2Rx FR1 PSCCH performance - single active PSSCH link |  |  | SNR as specified |  | 0.8 dB |  | Formula: SNR + TT missing detection probability limit unchanged |  |
| 11.1.5.1.1_1 2Rx FR1 PSCCH performance - single active PSSCH link |  |  | SNR as specified |  | 0.8 dB |  | Formula: SNR + TT missing detection probability limit unchanged |  |
| 11.1.6.1.1_1 2Rx FR1 Power imbalance performance - two active PSSCH link |  |  | SNR as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 11.1.7.1.1_1 2Rx FR1 HARQ buffer soft combining performance - maximum number of HARQ processes |  |  | SNR as specified |  | 0.6 dB |  | Formula: SNR + TT T-put limit unchanged |  |
| 11.1.8.1.1_1 2Rx FR1 PSCCH decoding capability - maximum number of received PSCCHs |  |  | sidelink power |  | 0 dB |  | sidelink power unchanged missing detection probability limit unchanged |  |
| 11.1.9.1.1_1 2Rx FR1 PSFCH decoding capability - maximum number of received PSFCHs |  |  | sidelink power |  | 0 dB |  | sidelink power unchanged missing detection probability limit unchanged |  |
| 5.5.1 FR1 Sustained downlink data rate performance for single carrier |  |  | Downlink power -112 dBm/Hz |  | 0 dB |  | Formula: Downlink power + TT T-put limit unchanged |  |
| 5.5A.1.1 FR1 SDR performance for CA (2DL CA) | Same as 5.5.1 |  | Same as 5.5.1 |  | Same as 5.5.1 |  |
| 5.5A.1.2 FR1 SDR performance for CA (3DL CA) | Same as 5.5.1 |  | Same as 5.5.1 |  | Same as 5.5.1 |  |
| 5.5A.1.3 FR1 SDR performance for CA (4DL CA) | Same as 5.5.1 |  | Same as 5.5.1 |  | Same as 5.5.1 |  |
| 5.5A.1.4 FR1 SDR performance for CA (5DL CA) | Same as 5.5.1 |  | Same as 5.5.1 |  | Same as 5.5.1 |  |

< End of Changes >