3GPP TSG-RAN5 Meeting #99	R5-233090

# Incheon , Korea, 22nd – 26th May. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2292 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NS_04 test configuration for Additional spurious emissions for UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test configuration table for test case A-SE for UL MIMO for NS_04 is inconsistent with the A-MPR for UL MIMO and needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified test configuration table in Table 6.5D.3.3.4.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some test points cannot be correctly executed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5D.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5D.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in Table 6.5D.3.3.4.1-1 through Table 6.5D.3.3.4.1-4. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5D.3.3.4.1-1: Test Configuration Table (network signalling value "NS_04")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (See Freq column) |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest |  |  |
| Additional spurious emissions test parameters for NS_04 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |  |  |  |
| Test ID |  | Freq |  |  |  |  |  | N/A for A-MPR testing |  |  | Modulation |  |  | RB allocation (NOTE 1) |  |  |
|  |  |  |  |  |  |  | (NOTE 2) |  |  |  |
| 1 |  | Low |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  | Edge_1RB_Left |  |  |
| 2 |  | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  | Edge_1RB_Left |  |  |
| 3 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |  |
| 4 | Low |  |  |  |  |  | CP-OFDM QPSK |  | Outer Full |  |
| 5 | High |  |  |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |  |
| 6 | High |  |  |  |  |  | CP-OFDM QPSK |  | Outer Full |  |
| 7 | Low |  |  |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |  |
| 8 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |  |
| 9 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |  |
| 10 | Low |  |  |  |  |  | CP-OFDM 16 QAM |  | Outer Full |  |
| 11 | High |  |  |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |  |
| 12 | High |  |  |  |  |  | CP-OFDM 16 QAM |  | Outer Full |  |
| 13 | Low |  |  |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |  |
| 14 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |  |
| 15 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |  |
| 16 | Low |  |  |  |  |  | CP-OFDM 64 QAM |  | Outer Full |  |
| 17 | High |  |  |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |  |
| 18 | High |  |  |  |  |  | CP-OFDM 64 QAM |  | Outer Full |  |
| 19 | Low |  |  |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |  |
| 20 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |  |
| 21 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |  |
| 22 | Low |  |  |  |  |  | CP-OFDM 256 QAM |  | Outer Full |  |
| 23 | High |  |  |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |  |
| 24 | High |  |  |  |  |  | CP-OFDM 256 QAM |  | Outer Full |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of changes>