3GPP TSG-RAN5 Meeting #99	R5-233168

# Incheon, South Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1618 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Additional Spurious Emissions FR2 CA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Additional spurious emissions test cases are missing for EN-DC with FR2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional spurious emissions test cases for 2 NR CCs, 3 NR CCs and 4 NR CCs have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Definition of additional spurious emissions CA TCs for FR2 EN-DC devices will not progress. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.4.4_1 (new), F1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0293 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | associated applicability update in CR R5-233176 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

6.5B.4.4a.5	Test requirement

Same test requirement as in clause 6.5.3.3_1.5 in TS 38.521-2 [9] for the NR carrier.

#### 6.5B.4.4_1	Additional Spurious Emissions for Inter-band including FR2 (>1 NR CC)

##### 6.5B.4.4_1.1	Additional Spurious Emissions for Inter-band including FR2 (2 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Test procedure only includes the testing of smartphone and is FFS for laptop and FWA.

-	For a transition period until RAN#99, the stability and repeatability of test procedure with PHR (variant b) for Rel-15 UEs is under evaluation.

6.5B.4.4_1.1.1	Test purpose

Same test purpose as in clause 6.5A.3.3.1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.4.4_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR UL CCs.

6.5B.4.4_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.4.5.

6.5B.4.4_1.1.4	Test description

6.5B.4.4_1.1.4.1	Initial condition

Same test description as in clause 6.5A.3.3.1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.3.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.3.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.3.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.4.4_1.1.5	Test Requirements

Same test requirement as in clause 6.5A.3.3.1.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.4.4_1.2	Additional Spurious Emissions for Inter-band including FR2 (3 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Test procedure only includes the testing of smartphone and is FFS for laptop and FWA.

-	For a transition period until RAN#99, the stability and repeatability of test procedure with PHR (variant b) for Rel-15 UEs is under evaluation.

6.5B.4.4_1.2.1	Test purpose

Same test purpose as in clause 6.5A.3.3.2.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.4.4_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 3 NR UL CCs.

6.5B.4.4_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.4.5.

6.5B.4.4_1.2.4	Test description

6.5B.4.4_1.2.4.1	Initial condition

Same test description as in clause 6.5A.3.3.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.3.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.3.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.3.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.4.4_1.2.5	Test Requirements

Same test requirement as in clause 6.5A.3.3.2.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.5B.4.4_1.3	Additional Spurious Emissions for Inter-band including FR2 (4 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Test procedure only includes the testing of smartphone and is FFS for laptop and FWA.

-	For a transition period until RAN#99, the stability and repeatability of test procedure with PHR (variant b) for Rel-15 UEs is under evaluation.

6.5B.4.4_1.3.1	Test purpose

Same test purpose as in clause 6.5A.3.3.3.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.4.4_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 NR UL CCs.

6.5B.4.4_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.4.5.

6.5B.4.4_1.3.4	Test description

6.5B.4.4_1.3.4.1	Initial condition

Same test description as in clause 6.5A.3.3.3.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1.

For Initial conditions as in clause 6.5A.3.3.3.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5A.3.3.3.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5A.3.3.3.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.4.4_1.3.5	Test Requirements

Same test requirement as in clause 6.5A.3.3.3.5 in TS 38.521-2 [9] for the NR carrier.

< Unchanged sections omitted >

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2B.1.1 UE Maximum Output Power for Intra-Band Contiguous EN-DC | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| < Unchanged rows omitted > |  |  |
| 6.5B.4.1 Additional Spurious Emissions for Intra-band contiguous EN-DC | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.4.2 Additional Spurious Emissions for Intra-band non-contiguous EN-DC | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.4.3 Additional Spurious Emissions for Inter-band EN-DC within FR1 | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.4.4 Additional Spurious Emissions for Inter-band including FR2 (1 NR CC) | Same as clause 6.5.3.3 in TS 38.521-2 [9] |  |
| 6.5B.4.4a Additional Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) | Same as clause 6.5.3.3_1 in TS 38.521-2 [9] |  |
| 6.5B.4.4_1.1 Additional Spurious Emissions for Inter-band including FR2 (2 NR CC) | Same as clause 6.5A.3.3.1 in TS 38.521-2 [9] |  |
| 6.5B.4.4_1.2 Additional Spurious Emissions for Inter-band including FR2 (3 NR CC) | Same as clause 6.5A.3.3.2 in TS 38.521-2 [9] |  |
| 6.5B.4.4_1.3 Additional Spurious Emissions for Inter-band including FR2 (4 NR CC) | Same as clause 6.5A.3.3.3 in TS 38.521-2 [9] |  |
| 6.5B.5.3 Transmit intermodulation for Inter-band EN-DC within FR1 | Same as clause 6.5.4.3 in TS 38.521-1 [8] |  |
| 6.6B.4 Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP | Same as clause 6.6.1 in TS 38.521-2 [9] |  |
| 6.6B.5 Enhanced Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP | Same as clause 6.6.2 in TS 38.521-2 [9] |  |

< Unchanged sections omitted >

## F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.2B.1.1 UE Maximum Output Power for Intra-Band Contiguous EN-DC | Same as 6.2.1 in TS 38.521-1 [8] |  |
| < Unchanged rows omitted > |  |  |
| 6.5B.4.1 Additional Spurious Emissions for Intra-band contiguous EN-DC | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.4.2 Additional Spurious Emissions for Intra-band non-contiguous EN-DC | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.4.3 Additional Spurious Emissions for Inter-band EN-DC within FR1 | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.4.4 Additional Spurious Emissions for Inter-band including FR2 (1 NR CC) | Same as clause 6.5.3.3 in TS 38.521-2 [9] |  |
| 6.5B.4.4a Additional Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) | Same as clause 6.5.3.3_1 in TS 38.521-2 [9] |  |
| 6.5B.4.4_1.1 Additional Spurious Emissions for Inter-band including FR2 (2 NR CC) | Same as clause 6.5A.3.3.1 in TS 38.521-2 [9] |  |
| 6.5B.4.4_1.2 Additional Spurious Emissions for Inter-band including FR2 (3 NR CC) | Same as clause 6.5A.3.3.2 in TS 38.521-2 [9] |  |
| 6.5B.4.4_1.3 Additional Spurious Emissions for Inter-band including FR2 (4 NR CC) | Same as clause 6.5A.3.3.3 in TS 38.521-2 [9] |  |
| 6.5B.5.3 Transmit intermodulation for Inter-band EN-DC within FR1 | Same as 6.5.4 in TS 38.521-1 [8] |  |
| 6.6B.4 Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP | Same as clause 6.6.1 in TS 38.521-2 [9] |  |
| 6.6B.5 Enhanced Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP | Same as clause 6.6.2 in TS 38.521-2 [9] |  |

< End of changes >