Title:	Discussion on test point selection for CA_NS_202 A-MPR in FR2

Source:	Rohde & Schwarz

# Introduction

The purpose of this paper is to provide test point selection proposal for 38.101-2 A-MPR requirements when CA_NS_202 is signalled (band n257 and n258). Additional RF emission requirements apply in this scenario in the form of Spurious emissions, meaning this proposal applies for test cases 6.2A.3 (A-MPR for CA) and 6.5A.3.3 (Additional Spurious emission for CA).

Test point selection is considered for PC1, PC2, PC3 and PC4.

# Discussion

Since A-MPR for CA_NS_202 in RAN4 is defined together with A-Spurious requirements, a combined analysis is required. In general, the following non-compliant UE behaviors need to be checked:

- UE apply too much A-MPR (more than RAN4 allow)

- UE apply to little A-MPR (causing too much RF emissions)

Case A can be verified in A-MPR test case.

Case B can be verified in A-Spurious test case and does not require check of UE output power if it is ensured that the same test point is tested inside A-MPR test.

A-MPR for CA_NS_202 is defined in 38.101-2 [1] for intra-band contiguous CA as:

For power class 1, A-MPR for CA_NS_202 shall be 11.0 dB

For power class 2, A-MPR for CA_NS_202 shall be 2.0 dB.

For power class 3, A-MPR for CA_NS_202 shall be 2.0 dB.

For power class 4, A-MPR for CA_NS_202 shall be 2.0 dB.

A-Spurious for CA_NS_202 is defined in 38.101-2 [2] as:

When "CA_NS_202" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in

Table 6.5.3.2.3-1.

Table 6.5.3.2.3-1: Additional requirements (NS_202)

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 dBm | 100 MHz |  |
| 23.6 GHz f 24.0 GHz | +1 dBm | 200 MHz | 1 |
| NOTE 1: This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth. The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |  |

## Power classes

For PC2, PC3 and PC4 and for all UL modulations and cumulative aggregated bandwidths, A-MPR is smaller than MPRC_CA and hence does not need to be considered in the test point selection. There is another conformance requirement valid in the case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration. However, this does not fulfil the purpose of UL CA test where more than one UL CC shall be allocated. Therefore, this test point selection analysis focuses on PC1.

## Test Environment

RF emission requirements for CA_NS_202 are defined as Spurious emissions. Regular spurious emissions for FR2 is tested in Normal conditions, and the same selection is appropriate also for Additional Spurious emission.

## Test Frequencies

Protected range for CA_NS_202 is both below and above band n257 and n258. It is judged that the worst case is when the carrier frequency is as close as possible to the protected range, which implies Low and High frequency range.

## Test Channel Bandwidth

The A-MPR requirements are the same for a particular power class for any channel bandwidth. Therefore, test bandwidth can be decided based on what is most demanding for Spurious emission. Selection can therefore be reused from normal spurious test case which means Highest aggregated bandwidth.

## Test Subcarrier Spacing

RF emission requirements for CA_NS_202 are defined as Spurious emissions. Regular spurious emissions for FR2 is tested with 120 kHz. Selection can therefore be reused from normal spurious test case which means 120 kHz.

## UL Modulation

RF emission requirements for CA_NS_202 are defined as Spurious emissions. Regular spurious emissions for FR2 is tested with QPSK DFT-s-OFDM modulation to maximize UE output power (lowest power backoff). Testing at a high output power is considered more important than testing a more demanding modulation scheme for Spurious testing.

For PC1 where A-MPR is higher than MPR regardless of modulation for a cumulative aggregated channel bandwidth < 800 MHz also 64QAM is tested. The reason is that in this case 64QAM does not give the UE more allowed MPR and should therefore be added as a worst case.

The same UL modulation is applied on all CCs.

## Uplink RB allocation

RF emission requirements for CA_NS_202 are defined as Spurious emissions.

For PC1, A-MPR is always 11 dB meaning the UE output power may be the same regardless of test parameters. A worst case to test will then be an outer allocation closest to the band edge with high order modulation.

Inner_Partial allocation is appropriate due to high MPRnarrow values applicable to Inner_1RB waveform. In order to avoid possible testability issue caused by large MPR value, it can be considered to use partial allocation of 0 dB MPR

The same allocation is applied to the PCC and SCC(s).

## Check of alignment of test parameters between power measurement test case (A-MPR) and RF emissions test case (A-Spur)

Now that test point proposal is completed, it is required to check that all test points selected in RF emissions test case (Spurious in this case) is covered in A-MPR test case.

Result:

The test points in this paper matches the test points in A-Spurious and A-MPR test cases in TS38.521-2 for all power classes.

# Proposal

Following test points are proposed for CA_NS_202 A-MPR and A-Spur test cases (PC1)

- Test environment: Normal Conditions

- Test frequencies: Low and High range

- Test Channel Bandwidth: Highest aggregated bandwidth

- Test SCS: 120 kHz

- RB allocation and modulation: Inner_Partial (DFT-s-OFDM QPSK ), Outer_Full (64QAM)

No test points are proposed for CA_NS_202 A-MPR and A-Spur test cases (PC2, PC3 and PC4).

# Number of Test Points

The number of test points for NS_202 A-MPR is 4

The number of test points for NS_202 A-Spurious is 4, same as in Tx Spurious emissions test

# References

- TS 38.101-2 v17.8.0 User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone (Release 17)