3GPP TSG-RAN5 Meeting #99	R5-233186

# Incheon, Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2302 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to clauses using void table 5.5A.3-x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas ADT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | All the tables in subclause 5.5A.3 has been set to Void, however, there still many test cases and/or note are refer to the void tables, need to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated to sections which reference to Table 5.5A.3-x with proper sections and/or tables. - Updated NOTE which refer to 5.5A.3-x to “Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band.” - Some 3GPP style/format updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reference information in specification will stay incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1.1.4.1, 6.2A.2.1.4.1, 6.2A.3.1.4.1, 6.3A.1.1.4.1, 6.3A.3.1.4.1, 6.3A.3.1_1.4.1, 6.4A.1.1.4.1, 6.4A.2.1.1.4.1, 6.4A.2.2.1.4.1, 6.4A.2.3.1.4.1, 6.5A.1.1.4.1, 6.5A.2.2.1.4.1, 6.5A.3.1.1.4.1, 6.5A.3.2.1.4.1, 6.5A.4.1.4.1, 7.9A.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## {Start of changes}

#### 6.2A.1.1	UE maximum output power for CA (2UL CA)

Editor’s Note:

No test points are defined for intra-band contiguous UL CA and intra-band non-contiguous UL CA since there is no configuration satisfying MPR=0dB requirements in RAN4. Testing with lowest value of MPR will be covered in 6.2A.2.

<Unchanged Sections Skipped>

6.2A.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2A.1.1.4.1-1 and table 6.2A.1.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2A.1.1.4.1-1: Inter-band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  | (NOTE 2) | PCC | SCC |
| 1 | N/A for this test | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 2 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Left | Inner 1RB Left |
| 3 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Right | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

<Unchanged Sections Skipped>

#### 6.2A.2.1	UE maximum output power reduction for CA (2UL CA)

Editor’s note:	Testing for Intra-band non-contiguous UL CA is not applicable for UE not supporting IE dualPA-Architecture due to the lack of core requirement.

<Unchanged Sections Skipped>

6.2A.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR CA configuration specified in table 5.5A.3.1-1 for inter-band UL CA, table 5.5A.1-1 for intra-band contiguous UL CA and table 5.5A.2-1 for intra-band non-contiguous UL CA. All of these configurations shall be tested with applicable test parameters for each CA configuration of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2A.2.1.4.1-1 to table 6.2A.2.1.4.1-3b. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

<Unchanged Sections Skipped>

Table 6.2A.2.1.4.1-3a: Intra-band non-contiguous CA Test Configuration Table (MPRIM3 to meet -13dBm/MHz) when UE supporting IE dualPA-Architecture

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  |  |  |  | Low range High range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL config for PCC & SCC | UL configuration |  |  |  |  |  |  |  |  |
|  |  | Modulations for all CCs (NOTE 2) |  | RB allocation LCRB@RBSTART (NOTE 2) |  |  |  |  |  |  |
|  |  |  |  | SCS 15 kHz |  |  | SCS 30 kHz |  | SCS 60 kHz |  |
|  |  |  |  | PCC | SCC |  | PCC | SCC | PCC | SCC |
| 1 | N/A |  | Pi/2 BPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 2 |  |  | Pi/2 BPSK | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 3 |  |  | Pi/2 BPSK | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 4 |  |  | Pi/2 BPSK | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 53 |  |  | Pi/2 BPSK | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 64 |  |  | Pi/2 BPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 75 |  |  | Pi/2 BPSK | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 85 |  |  | Pi/2 BPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 9 |  |  | QPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 10 |  |  | QPSK | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 11 |  |  | QPSK | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 12 |  | D | QPSK | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 133 |  | F | QPSK | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 144 |  | T | QPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 155 |  | - | QPSK | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 165 |  | s | QPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 17 |  | - | 16QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 18 |  | O | 16QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 19 |  | F | 16QAM | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 20 |  | D | 16QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 213 |  | M | 16QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 224 |  |  | 16QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 235 |  |  | 16QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 245 |  |  | 16QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 25 |  |  | 64QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 26 |  |  | 64QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 27 |  |  | 64QAM | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 28 |  |  | 64QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 293 |  |  | 64QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 304 |  |  | 64QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 315 |  |  | 64QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 325 |  |  | 64QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 33 |  |  | 256QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 34 |  |  | 256QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 35 |  |  | 256QAM | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 36 |  |  | 256QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 373 |  |  | 256QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 384 |  |  | 256QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 395 |  |  | 256QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 405 |  |  | 256QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 41 |  |  | QPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 42 |  |  | QPSK | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 43 |  |  | QPSK | 23@0 | 23@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 44 |  |  | QPSK | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 453 |  |  | QPSK | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 464 |  |  | QPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 475 |  |  | QPSK | 150@0 | 150@0 |  | 75@0 | 75@0 | 38@0 | 38@0 |
| 485 |  |  | QPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 49 |  |  | 16QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 50 |  | C | 16QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 51 |  | P | 16QAM | 23@0 | 23@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 52 |  | - | 16QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 533 |  | O | 16QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 544 |  | F | 16QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 555 |  | D | 16QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 38@0 | 38@0 |
| 565 |  | M | 16QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 57 |  |  | 64QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 58 |  |  | 64QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 59 |  |  | 64QAM | 23@0 | 23@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 60 |  |  | 64QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 613 |  |  | 64QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 624 |  |  | 64QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 635 |  |  | 64QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 38@0 | 38@0 |
| 645 |  |  | 64QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 65 |  |  | 256QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 66 |  |  | 256QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 67 |  |  | 256QAM | 23@0 | 23@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 68 |  |  | 256QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 693 |  |  | 256QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 704 |  |  | 256QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 715 |  |  | 256QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 38@0 | 38@0 |
| 725 |  |  | 256QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1. NOTE 2: Edge_1RB_Left, Edge_1RB_Right and Outer_Full RB allocation are specified in table 6.1-1 for each component carrier. NOTE 3: Only being tested when the channel bandwidth of both carriers are larger than 10 MHz. NOTE 4: Only being tested when the channel bandwidth of both carriers are larger than 20 MHz. NOTE 5: Only being tested when the channel bandwidth of both carriers are larger than 25 MHz. NOTE 6: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.4.1-3b: Intra-band non-contiguous CA Test Configuration Table (MPRIM3 to meet -30dBm/MHz) when UE supporting IE dualPA-Architecture

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  |  |  |  | Low range High range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL config for PCC & SCC | UL configuration |  |  |  |  |  |  |  |  |
|  |  | Modulations for all CCs (NOTE 2) |  | RB allocation LCRB@RBSTART (NOTE 2) |  |  |  |  |  |  |
|  |  |  |  | SCS 15 kHz |  |  | SCS 30 kHz |  | SCS 60 kHz |  |
|  |  |  |  | PCC | SCC |  | PCC | SCC | PCC | SCC |
| 1 | N/A |  | Pi/2 BPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 2 |  |  | Pi/2 BPSK | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 3 |  |  | Pi/2 BPSK | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 43 |  |  | Pi/2 BPSK | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 54 |  |  | Pi/2 BPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 65 |  |  | Pi/2 BPSK | 216@0 | 216@0 |  | 108@0 | 108@0 | 54@0 | 54@0 |
| 75 |  |  | Pi/2 BPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 8 |  |  | QPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 9 |  |  | QPSK | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 10 |  |  | QPSK | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 113 |  |  | QPSK | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 124 |  | D | QPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 135 |  | F | QPSK | 216@0 | 216@0 |  | 108@0 | 108@0 | 54@0 | 54@0 |
| 145 |  | T | QPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 15 |  | - | 16QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 16 |  | s | 16QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 17 |  | - | 16QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 183 |  | O | 16QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 194 |  | F | 16QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 205 |  | D | 16QAM | 216@0 | 216@0 |  | 108@0 | 108@0 | 54@0 | 54@0 |
| 215 |  | M | 16QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 22 |  |  | 64QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 23 |  |  | 64QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 24 |  |  | 64QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 253 |  |  | 64QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 264 |  |  | 64QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 275 |  |  | 64QAM | 216@0 | 216@0 |  | 108@0 | 108@0 | 54@0 | 54@0 |
| 285 |  |  | 64QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 29 |  |  | 256QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 30 |  |  | 256QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 31 |  |  | 256QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 323 |  |  | 256QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 334 |  |  | 256QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 345 |  |  | 256QAM | 216@0 | 216@0 |  | 108@0 | 108@0 | 54@0 | 54@0 |
| 355 |  |  | 256QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 36 |  |  | QPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 37 |  |  | QPSK | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 38 |  |  | QPSK | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 393 |  |  | QPSK | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 404 |  |  | QPSK | 115@0 | 115@0 |  | 58@0 | 58@0 | 29@0 | 29@0 |
| 415 |  | C | QPSK | 210@0 | 210@0 |  | 105@0 | 105@0 | 53@0 | 53@0 |
| 425 |  | P | QPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 43 |  | - | 16QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 44 |  | O | 16QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 45 |  | F | 16QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 463 |  | D | 16QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 474 |  | M | 16QAM | 115@0 | 115@0 |  | 58@0 | 58@0 | 29@0 | 29@0 |
| 485 |  |  | 16QAM | 210@0 | 210@0 |  | 105@0 | 105@0 | 53@0 | 53@0 |
| 495 |  |  | 16QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 50 |  |  | 64QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 51 |  |  | 64QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 52 |  |  | 64QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 533 |  |  | 64QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 544 |  |  | 64QAM | 115@0 | 115@0 |  | 58@0 | 58@0 | 29@0 | 29@0 |
| 555 |  |  | 64QAM | 210@0 | 210@0 |  | 105@0 | 105@0 | 53@0 | 53@0 |
| 565 |  |  | 64QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 57 |  |  | 256QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 58 |  |  | 256QAM | 15@0 | 15@0 |  | 8@0 | 8@0 | 4@0 | 4@0 |
| 59 |  |  | 256QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 603 |  |  | 256QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 614 |  |  | 256QAM | 115@0 | 115@0 |  | 58@0 | 58@0 | 29@0 | 29@0 |
| 625 |  |  | 256QAM | 210@0 | 210@0 |  | 105@0 | 105@0 | 53@0 | 53@0 |
| 635 |  |  | 256QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1. NOTE 2: Edge_1RB_Left, Edge_1RB_Right and Outer_Full RB allocation are specified in table 6.1-1 for each component carrier. NOTE 3: Only being tested when the channel bandwidth of both carriers are larger than 10 MHz. NOTE 4: Only being tested when the channel bandwidth of both carriers are larger than 20 MHz. NOTE 5: Only being tested when the channel bandwidth of both carriers are larger than 25 MHz. NOTE 6: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |

<Unchanged Sections Skipped>

#### 6.2A.3.1	UE additional maximum output power reduction for CA (2UL CA)

<Unchanged Sections Skipped>

6.2A.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For the UE maximum output power modified by A-MPR specified in table 6.2A.3.0.3-1, the power limits specified in subclause 6.2A.4.1.3 apply.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing and are shown in table 6.2A.3.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

<Unchanged Sections Skipped>

Table 6.2A.3.1.4.1-4: Test Configuration Table for inter-band CA (network signalling value NS_56/NS_04)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  | Low range for PCC and Low and High range for SCC as specified in test parameters |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest and Highest for PCC as specified in test parameters Lowest and Highest for SCC as specified in test parameters |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest as specified in test parameters |  |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  |  | SCC |  |  |  |  |
|  |  | FC (MHz) | SCS | Ch BW (MHz) | Modulation | RB allocation |  | FC (MHz) | Ch BW (MHz) | Modulation | RB allocation |  |
|  |  |  |  |  |  |  |  |  |  |  | SCS (15 kHz) | SCS (60 kHz) |
| 1 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right |  | Low | 10 | DFT-s-OFDM QPSK | 20@20 | 4@4 |
| 2 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 3 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | DFT-s-OFDM QPSK | 20@20 | 4@4 |
| 4 | N/A | Low | 30 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right |  | Low | 10 | DFT-s-OFDM QPSK | 20@20 | 4@4 |
| 5 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 6 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | DFT-s-OFDM QPSK | 20@20 | 4@4 |
| 7 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | 18@20 |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 8 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 9 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 10 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | 4@5 |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 11 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 12 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |

Table 6.2A.3.1.4.1-5: Test Configuration Table for inter-band CA (network signalling value NS_56/NS_27)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  | Low range for PCC and Low and High range for SCC as specified in test parameters |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest and Highest for PCC as specified in test parameters Lowest and Highest for SCC as specified in test parameters |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest as specified in test parameters |  |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  |  | SCC |  |  |  |  |
|  |  | FC (MHz) | SCS | Ch BW (MHz) | Modulation | RB allocation |  | FC (MHz) | Ch BW (MHz) | Modulation | RB allocation |  |
|  |  |  |  |  |  |  |  |  |  |  | Low SCS | High SCS |
| 1 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right |  | Low | 15 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 2 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 15 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 3 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 15 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 4 | N/A | Low | 30 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right |  | Low | 15 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 5 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 15 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 6 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 15 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 7 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | 18@20 |  | High | 40 | DFT-s-OFDM QPSK | 15@10 | 5@3 |
| 8 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 40 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 9 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 40 | DFT-s-OFDM QPSK | 15@10 | 5@3 |
| 10 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | 4@5 |  | High | 40 | DFT-s-OFDM QPSK | 15@10 | 5@3 |
| 11 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 40 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 12 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 40 | DFT-s-OFDM QPSK | 15@10 | 5@3 |

Table 6.2A.3.1.4.1-6: Test Configuration Table for inter-band CA (network signalling value NS_56/NS_55)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  | Low range for PCC and Low and High range for SCC as specified in test parameters |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest and Highest for PCC as specified in test parameters Lowest and Highest for SCC as specified in test parameters |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest as specified in test parameters |  |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  |  | SCC |  |  |  |  |
|  |  | FC (MHz) | SCS | Ch BW (MHz) | Modulation | RB allocation |  | FC (MHz) | Ch BW (MHz) | Modulation | RB allocation |  |
|  |  |  |  |  |  |  |  |  |  |  | SCS (15 kHz) | SCS (60 kHz) |
| 1 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right |  | Low | 10 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 2 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | CP-OFDM 256QAM | Outer Full | Outer Full |
| 3 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 4 | N/A | Low | 30 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right |  | Low | 10 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 5 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | CP-OFDM 256QAM | Outer Full | Outer Full |
| 6 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 7 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | 18@20 |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 8 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | CP-OFDM 256QAM | Outer Full | Outer Full |
| 9 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 10 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | 4@5 |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 11 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | CP-OFDM 256QAM | Outer Full | Outer Full |
| 12 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |

Table 6.2A.3.1.4.1-7: Test Configuration Table for intra-band contiguous CA for CA_NS_04 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  | Refering to Test Frequency Column |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg Highest NRB_agg |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |
| Test ID | Test Frequency | Downlink Configuration | Uplink Configuration |  |
|  |  |  | Modulation | RB allocation for PCC&SCC (NOTE 1) |
| 1 | High range |  | CP-OFDM QPSK | Outer_Full |
| 2 | High range |  | CP-OFDM QPSK | Edge_1RB_Right |
| 3 | Low range |  | CP-OFDM QPSK | Outer_Full |
| 4 | Low range |  | CP-OFDM QPSK | Edge_1RB_Left |
| 5 | Low range |  | DFT-s-OFDM Pi/2 BPSK | Inner_Full |
| 6 | Low range |  | DFT-s-OFDM Pi/2 QPSK | Inner_Full |
| 7 | Low range |  | DFT-s-OFDM Pi/2 16QAM | Inner_Full |
| 8 | Low range |  | DFT-s-OFDM Pi/2 64QAM | Inner_Full |
| 9 | Low range |  | DFT-s-OFDM Pi/2 256QAM | Inner_Full |
| 10 | Low range |  | CP-OFDM QPSK | Inner_Full |
| 11 | Low range |  | CP-OFDM 16QAM | Inner_Full |
| 12 | Low range |  | CP-OFDM 64QAM | Inner_Full |
| 13 | Low range |  | CP-OFDM 256QAM | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

<Unchanged Sections Skipped>

#### 6.3A.1.1	Minimum output power for CA (2UL CA)

6.3A.1.1.1	Test purpose

To verify the UE's ability to transmit with a broadband output power for 2UL CA below the value specified in the test requirement when the power is set to a minimum value.

6.3A.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support 2UL CA.

6.3A.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3A.1.0.

6.3A.1.1.4	Test description

6.3A.1.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in table 6.3A.1.1.4.1-1 or 6.3A.1.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3A.1.1.4.1-1: Test Configuration Table for inter-band CA

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A for this test | DFT-s-OFDM QPSK | Outer Full | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

Table 6.3A.1.1.4.1-2: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A for this test | DFT-s-OFDM QPSK | Outer Full | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in 6.1A-1a NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

<Unchanged Sections Skipped>

#### 6.3A.3.1	Transmit ON/OFF time mask for CA (2UL CA)

6.3A.3.1.1	Test purpose

To verify that the general ON/OFF time mask for CA (2UL CA) meets the requirements given in 6.3A.3.1.5

The transmit power time mask for transmit ON/OFF defines the transient period(s) allowed between transmit OFF power and transmit ON power symbols for CA.

Transmission of the wrong power increases interference to other channels, or increases transmission errors in the uplink channel.

6.3A.3.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support 2UL CA.

6.3A.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3A.3.0.

6.3A.3.1.4	Test description

6.3A.3.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in table 6.3A.3.1.4.1-1 or 6.3A.3.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3A.3.1.4.1-1: Test Configuration Table for inter-band CA

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A for this test | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

Table 6.3A.3.1.4.1-2: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A for this test | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in 6.1A-1a NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

<Unchanged Sections Skipped>

#### 6.3A.3.1_1	Time mask for switching between two uplink carriers

Editor’s Note: The improvement for test procedure is FFS

6.3A.3.1_1.1	Test purpose

To verify that the time mask for switching between two uplink carriers meets the requirements given in 6.3A.3.0.3.2.

The time mask for switching between two uplink carriers defines the transient period(s) allowed between two uplink carriers for an uplink band pair of an inter-band UL CA configuration when the capability uplinkTxSwitchingPeriod is present.

6.3A.3.1_1.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support 2UL inter-band CA and dynamic UL Tx switching.

6.3A.3.1_1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3A.3.0.3.2.

6.3A.3.1_1.4	Test description

6.3A.3.1_1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in table 6.3A.3.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3A.3.1_1.4.1-1: Test Configuration Table for inter-band CA Uplink switching

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.1.3 for inter band CA in FR1 |  | Mid range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 2) |  |
|  |  |  | PCC | SCC |
| 1 | N/A for this test | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| NOTE 1: PCC is the component carrier with lower center frequency between two component carriers. PCC is configured as Carrier 1 and SCC is configured as Carrier2. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

<Unchanged Sections Skipped>

#### 6.4A.1.1	Frequency error for CA (2UL CA)

6.4A.1.1.1	Test purpose

This test verifies the ability of both, the receiver and the transmitter, to process frequency for 2UL CA correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

6.4A.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA.

6.4A.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.4A.1.0.

6.4A.1.1.4	Test description

6.4A.1.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in Table 6.4A.1.1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.1.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  | Mid range for PCC and SCC (NOTE 3) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest NRB_agg for both PCC and SCC |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | Modulation | RB allocation | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band belongs to inter-band CA combination. NOTE 3: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |

<Unchanged Sections Skipped>

##### 6.4A.2.1.1	Error Vector Magnitude for CA (2UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

- The minimum requirements for intra-band contiguous CA and intra-band non-contiguous CA have not been defined.

6.4A.2.1.1.1	Test Purpose

For 2UL carrier aggregation, the Error Vector Magnitude requirement should be defined for each component carrier. Requirement applies for the allocated component carrier, when all other component carriers are activated, but not allocated.

Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in clause 6.4.2.1.

6.4A.2.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA.

6.4A.2.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.4A.2.1.0.

6.4A.2.1.1.4	Test description

6.4A.2.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in Table 6.4A.2.1.1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.2.1.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg for both PCC and SCC Highest NRB_agg for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Smallest and biggest supported SCS per Channel Bandwidth |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation (NOTE 3) | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 13 | N/A | DFT-s-OFDM PI/2 BPSK | Inner Full | 0 |
| 23 |  | DFT-s-OFDM PI/2 BPSK | Outer Full | 0 |
| 3 |  | DFT-s-OFDM QPSK | Inner Full | 0 |
| 4 |  | DFT-s-OFDM QPSK | Outer Full | 0 |
| 5 |  | DFT-s-OFDM 16 QAM | Inner Full | 0 |
| 6 |  | DFT-s-OFDM 16 QAM | Outer Full | 0 |
| 7 |  | DFT-s-OFDM 64 QAM | Outer Full | 0 |
| 8 |  | DFT-s-OFDM 256 QAM | Outer Full | 0 |
| 9 |  | CP-OFDM QPSK | Inner Full | 0 |
| 10 |  | CP-OFDM QPSK | Outer Full | 0 |
| 11 |  | CP-OFDM 16 QAM | Inner Full | 0 |
| 12 |  | CP-OFDM 16 QAM | Outer Full | 0 |
| 13 |  | CP-OFDM 64 QAM | Outer Full | 0 |
| 14 |  | CP-OFDM 256 QAM | Outer Full | 0 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |

<Unchanged Sections Skipped>

##### 6.4A.2.2.1	Carrier leakage for CA (2UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

- The minimum requirements for intra-band contiguous CA and intra-band non-contiguous CA have not been defined.

6.4A.2.2.1.1	Test purpose

Carrier leakage is an additive sinusoid waveform that is confined within the aggregated transmission bandwidth configuration. The carrier leakage requirement for 2UL CA is defined for each component carrier and is measured on the component carrier with PRBs allocated.

6.4A.2.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA.

6.4A.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.4A.2.2.0.

6.4A.2.2.1.4	Test description

6.4A.2.2.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in Table 6.4A.2.2.1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.2.2.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid NRB_agg for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Smallest supported SCS per Channel Bandwidth |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | RB allocation (NOTE 1, 3) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | DFT-s-OFDM QPSK | Inner_1RB_Left | 0 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. NOTE 3: When the signalled DC carrier position is at Inner_1RB_Left, use Inner_1RB_Right for UL RB allocation. NOTE 4: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |

<Unchanged Sections Skipped>

##### 6.4A.2.3.1	In-band emissions for CA (2UL CA)

6.4A.2.3.1.1	Test purpose

The in-band emissions are a measure of the interference falling into the non-allocated resources blocks.

For an allocated component carrier, the in-band emission is defined as the average across 12 sub-carrier and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB. The basic in-band emissions measurement interval is defined over one slot in the time domain, however, the minimum requirement applies when the in-band emission measurement is averaged over 10 sub-frames. When the PUSCH or PUCCH transmission slot is shortened due to multiplexing with SRS, the in-band emissions measurement interval is reduced by one or more symbols, accordingly.

For a non allocated component carrier a spectral measurement is specified.

6.4A.2.3.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA.

6.4A.2.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.4A.2.3.0.

6.4A.2.3.1.4	Test description

6.4A.2.3.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in Table 6.4A.2.3.1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.2.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg for both PCC and SCC Highest NRB_agg for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Smallest supported SCS per Channel Bandwidth |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | DFT-s-OFDM QPSK | Inner_1RB_Left | 0 |
| 2 |  | DFT-s-OFDM QPSK | Inner_1RB_Right | 0 |
| 3 |  | CP-OFDM QPSK | Inner_1RB_Left | 0 |
| 4 |  | CP-OFDM QPSK | Inner_1RB_Right | 0 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each NR band, which applicable channel bandwidths are specified in Table 5.5A.3.1-1. NOTE 3: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |

<Unchanged Sections Skipped>

#### 6.5A.1.1	Occupied bandwidth for CA (2UL CA)

Editor’s Note:

- Due to lack of MPR requirements in core specification, this test case is incomplete for intra-band contiguous UL CA for power class 2 UEs indicating IE dualPA-Architecture supported, and incomplete for intra-band non-contiguous UL CA for power class 2 UEs, and power class 3 UEs when signalling is absent for dualPA-Architecture.

- MU needs to be reassessed.

6.5A.1.1.1	Test purpose

To verify that the UE occupied bandwidth for all transmission bandwidth configurations supported by the UE are less than their specific limits for 2 UL CA

6.5A.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA

6.5A.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.1.0.

6.5A.1.1.4	Test description

6.5A.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.1.1.4.1-1 for inter-band UL CA, table 6.5A.1.1.4.1-2 for intra-band contiguous CA and table 6.5A.1.1.4.1-3 for intra-band non-contiguous CA. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.1.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range for both PCC and SCC (NOTE 4) |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg for both PCC and SCC Highest NRB_agg for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Smallest supported SCS per Channel Bandwidth |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  | (NOTE 2) | PCC | SCC |
| 1 | N/A for this test | CP-OFDM QPSK | Outer_Full | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.3A.4-1. NOTE 3: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |

<Unchanged Sections Skipped>

##### 6.5A.2.2.1	Spectrum emission mask for CA (2UL CA)

6.5A.2.2.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth for 2UL CA.

6.5A.2.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA.

6.5A.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause6.5A.2.2.0.

6.5A.2.2.1.4	Test description

6.5A.2.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Tables 6.5A.2.2.1.4.1-1 through 6.5A.2.2.1.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.2.2.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg for both PCC and SCC Highest NRB_agg for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Smallest and biggest supported SCS per Channel Bandwidth |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  | Modulation (NOTE 3) | RB allocation (NOTE 1) |  |
|  |  |  |  | PCC | SCC |
| 13 | Low | N/A | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left | Edge_1RB_Left |
| 23 | High |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right | Edge_1RB_Right |
| 3 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 4 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 5 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 6 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 7 | Default |  | DFT-s-OFDM 64 QAM | Outer_Full | Outer_Full |
| 8 | Default |  | DFT-s-OFDM 256 QAM | Outer_Full | Outer_Full |
| 9 | Low |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 10 | High |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 11 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 12 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 13 | Default |  | CP-OFDM 64 QAM | Outer_Full | Outer_Full |
| 14 | Default |  | CP-OFDM 256 QAM | Outer_Full | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1 and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. |  |  |  |  |  |

Table 6.5A.2.2.1.4.1-2: Intra-band contiguous CA Test Configuration Table for PC3 and PC2

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |  |
| Test Parameters for CA bandwidth class B and C |  |  |  |  |  |
| Test ID | DL configuration for PCC & SCC | UL configuration |  |  |  |
|  |  | Modulations for all CCs (NOTE 2) |  |  | RB allocation (NOTE 3) |
| 1 |  | DFT-s-OFDM |  | Pi/2 BPSK | Outer Full |
| 2 |  |  |  | QPSK | Outer Full |
| 3 |  |  |  | 16QAM | Outer Full |
| 4 | N/A |  |  | 64QAM | Outer Full |
| 5 |  |  |  | 256QAM | Outer Full |
| 6 |  | CP-OFDM |  | QPSK | Outer Full |
| 7 |  |  |  | 16QAM | Outer Full |
| 8 |  |  |  | 64QAM | Outer Full |
| 9 |  |  |  | 256QAM | Outer Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

Table 6.5A.2.2.1.4.1-3: Intra-band non-contiguous CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest for both PCC and SCC Highest for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Smallest and biggest supported SCS per Channel Bandwidth |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation (NOTE 4) | RB allocation (NOTE 2) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left | Edge_1RB_Right |
| 2 |  | DFT-s-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Right |
| 3 |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Right |
| 4 |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left | Edge_1RB_Right |
| 5 |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left | Edge_1RB_Right |
| 6 |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Right |
| 7 |  | CP-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Right |
| 8 |  | CP-OFDM 64 QAM | Edge_1RB_Left | Edge_1RB_Right |
| 9 |  | CP-OFDM 256 QAM | Edge_1RB_Left | Edge_1RB_Right |
| NOTE 1: This test configuration is only applicable for UEs indicating IE dualPA-Architecture supported NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. NOTE 4: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |

<Unchanged Sections Skipped>

##### 6.5A.3.1.1	General spurious emissions for CA (2UL CA)

6.5A.3.1.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5A.3.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA.

6.5A.3.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.3.1.0.

6.5A.3.1.1.4	Test description

6.5A.3.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.3.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.1.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg for both PCC and SCC Highest NRB_agg for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | CP-OFDM QPSK | Outer_Full | Outer_Full |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

Table 6.5A.3.1.1.4.1-2: Intra-band contiguous CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | CP-OFDM QPSK | Outer_Full | N/A |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left | N/A |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right | N/A |
| 4 |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Right |
| 5 |  | CP-OFDM QPSK | Outer_Full | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

<Unchanged Sections Skipped>

##### 6.5A.3.2.1	Spurious emissions for UE co-existence for CA (2UL CA)

6.5A.3.2.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions for 2UL CA.

6.5A.3.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support 2UL CA.

6.5A.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.3.2.0.

6.5A.3.2.1.4	Test description

6.5A.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.3.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.2.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  | Normal |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  | Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  | Lowest SCS for default test points. CA configuration specific test points: Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  |  |  |  |  |  |  | DL Allocation |  |  |  |  |  | UL Allocation |  |  |  |  |  |
|  | CA Configuration |  |  |  |  |  | PCC NRB @SCS | SCC NRB@SCS | CC MOD | PCC & SCC RB allocation |  |  |  |  | CC MOD |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
|  | PCC |  | SCC |  |  |  |  |  |  | PCC |  |  | SCC |  |  |  |  |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n1A-n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | Low | n8 | Low |  | 25@15kHz |  | 25@15kHz | CP-OFDM QPSK | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| Test Settings for CA_n1A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | Low | n78 | Low | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@105 |  | 1@272 |
| 2 | n1 | Low | n78 | 3421,14 MHz (NOTE 2) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| 3 | n1 | Low | n78 | High | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| Test Settings for CA_n1A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | LOW | n79 | MID | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@181 |
| 2 | n1 | LOW | n79 | 4618,41 MHz (NOTE 3) | 79@15kHz |  |  | 216@15kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| 3 | n1 | LOW | n79 | MID | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@105 |  | 1@245 |
| 4 | n1 | LOW | n79 | 4785,54 MHz (NOTE 4) | 106@15kHz |  |  | 216@15kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| Test Settings for CA_n2A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | Low | n5 | low | 20/106 |  |  | 20/106 | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| 2 | n2 | High | n5 | High | 20/106 |  |  | 20/106 | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@105 |  | 1@105 |
| Test Settings for CA_n2A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N/A (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n2A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | Low | n77 | Mid | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@272 |
| 2 | n2 | High | n77 | 3850,23 MHz (NOTE 5) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| 3 | n2 | High | n77 | 3965,25 MHz (NOTE 6) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| Test Settings for CA_n3A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | HIGH | n41 | HIGH | 160@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@0 |  | 1@151 |
| 2 | n3 | LOW | n41 | HIGH | 160@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@149 |  | 1@18 |
| 3 | n3 | LOW | n41 | HIGH | 133@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@132 |  | 1@155 |
| 4 | n3 | HIGH | n41 | LOW | 160@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@0 |  | 1@151 |
| 5 (NOTE 7) | n3 | HIGH | n41 | HIGH | 160@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@159 |  | 1@105 |
| 6 (NOTE 7) | n3 | LOW | n41 | HIGH | 79@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@78 |  | 1@153 |
| Test Settings for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | LOW | n78 | HIGH | 160@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@159 |  |  | 1@272 |  |
| 2 | n3 | HIGH | n78 | LOW | 160@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@159 |  |  | 1@0 |  |
| 3 | n3 | LOW | n78 | MID | 133@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@262 |  |
| 4 | n3 | LOW | n78 | MID | 160@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@41 |  |
| Test Settings for CA_n5A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | HIGH | n77 | MID | 79@15kHz |  |  | 162@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@161 |  |
| 2 | n5 | HIGH | n77 | MID | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@17 |  |
| 3 | n5 | LOW | n77 | 3458,79 MHz (NOTE 8) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 4 | n5 | HIGH | n77 | 3593,79 MHz (NOTE 9) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 5 | n5 | LOW | n77 | 4021,89 MHz (NOTE 10) | 52@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@51 |  |  | 1@0 |  |
| 6 | n5 | HIGH | n77 | HIGH | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@272 |  |
| 7 | n5 | LOW | n77 | 3538,29 MHz (NOTE 11) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n8 | HIGH | n78 | LOW | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 2 | n8 | LOW | n78 | MID | 79@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@192 |  |
| 3 | n8 | HIGH | n78 | HIGH | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@35 |  |
| 4 | n8 | LOW | n78 | LOW | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@0 |  |
| 5 | n8 | LOW | n78 | HIGH | 106@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@139 |  |
| Test Settings for CA_n24A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n24 | High | n41 | Low | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@136 |  |
| 2 | n24 | Low | n41 | Low | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@52 |  |  | 1@108 |  |
| 3 | n24 | High | n41 | Low | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@136 |  |
| 4 | n24 | Low | n41 | Low | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@94 |  |
| 5 | n24 | High | n41 | High | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@192 |  |
| Test Settings for CA_n24A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n24 | High | n48 | Mid | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@136 |  |
| 2 | n24 | High | n48 | High | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n24A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n24 | Low | n77 | Mid | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@136 |  |
| 2 | n24 | Low | n77 | 3930 MHz | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@273 |  |
| 3 | n24 | Low | n77 | 3930 MHz | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@262 |  |
| 4 | n24 | Low | n77 | 3930 MHz | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@52 |  |  | 1@196 |  |
| Test Settings for CA_n26A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n26 | High | n66 | High | 106@15kHz |  |  | 216@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@0 |  |
| Test Settings for CA_n26A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n26 | Low | n70 | Low | 106@15kHz |  |  | 79@15kHz | QPSK/CP-OFDM QPSK |  | NA |  |  | QPSK / CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n28A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | LOW | n41 | HIGH | 79@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@78 |  |  | 1@157 |  |
| 2 | n28 | HIGH | n41 | HIGH | 79@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@78 |  |  | 1@115 |  |
| 3 | n28 | HIGH | n41 | LOW | 79@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@194 |  |
| Test Settings for CA_n39A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n39 | LOW | n41 | HIGH | 216@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@137 |  |
| Test Settings for CA_n41A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | LOW | n79 | LOW | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@272 |  |  | 1@0 |  |
| 2 | n41 | HIGH | n79 | LOW | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@272 |  |  | 1@259 |  |
| 3 | n41 | LOW | n79 | MID | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@272 |  |  | 1@150 |  |
| 4 | n41 | HIGH | n79 | LOW | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 5 | n41 | HIGH | n79 | LOW | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@129 |  |  | 1@0 |  |
| 6 | n41 | HIGH | n79 | LOW | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 7 | n41 | HIGH | n79 | HIGH | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@272 |  |  | 1@272 |  |
| 8 | n41 | HIGH | n79 | MID | 217@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@216 |  |  | 1@133 |  |
| Test Settings for CA_n48A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | High | n66 | Low | 216@15kHz |  |  | 216@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n48A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | High | n70 | High | 216@15kHz |  |  | 79@15kHz | QPSK |  | NA |  |  | QPSK / CP-OFDM QPSK |  | 1@215 |  |  | 1@78 |  |
| 2 | n48 | High | n70 | Low | 216@15kHz |  |  | 79@15kHz | QPSK |  | NA |  |  | QPSK / CP-OFDM QPSK |  | 1@215 |  |  | 1@0 |  |
| Test Settings for CA_n48A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | HIGH | n71 | LOW | 106@15kHz |  |  | 106@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 2 | n48 | LOW | n71 | HIGH | 216@15kHz |  |  | 106@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@105 |  |
| Test Settings for CA_n66A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | Low | n71 | Low | 216@15kHz |  |  | 216@15kHz | QPSK/CP-OFDM QPSK |  | NA |  |  | QPSK / CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n66A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | LOW | n77 | 3825,24 MHz (NOTE 12) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 2 | n66 | LOW | n77 | HIGH | 216@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@151 |  |
| 3 | n66 | LOW | n77 | 4065,18 MHz (NOTE 13) | 79@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 4 | n66 | LOW | n77 | HIGH | 216@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@48 |  |
| 5 | n66 | LOW | n77 | HIGH | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@217 |  |
| 6 (NOTE 14) | n66 | LOW | n77 | HIGH | 79@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@214 |  |
| 7 | n66 | LOW | n77 | HIGH | 216@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@211 |  |
| 8 | n66 | LOW | n77 | MID | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@13 |  |
| 9 | n66 | HIGH | n77 | MID | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@24 |  |
| Test Settings for CA_n70A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n70 | Low | n71 | Low | 79@15kHz |  |  | 106@15kHz | QPSK/CP-OFDM QPSK |  | NA |  |  | QPSK / CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| NOTE 1: No impact on the protected bands by 2nd and 3rd order IM products caused by the NR CA band combination. NOTE 2: Test Point ID 2 for CA_n1A-n78A and Band n78 is defined as a carrier with CBW=10MHz and centre frequency at 3421.14MHz (NR ARFCN=628067). NOTE 3: Test Point ID 2 for CA_n1A-n79A and Band n79 is defined as a carrier with CBW=40MHz and centre frequency at 4618.41MHz (NR ARFCN=707867). NOTE 4: Test Point ID 4 for CA_n1A-n79A and Band n79 is defined as a carrier with CBW=40MHz and centre frequency at 4785.54MHz (NR ARFCN=719000). NOTE 5: Test Point ID 2 for CA_n2A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3850.23MHz (NR ARFCN=656667). NOTE 6: Test Point ID 3 for CA_n2A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3965.25MHz (NR ARFCN=664333). NOTE 7: Test point ID 5 and 6 for CA_n3A-n41A test protected NR band 100 introduced in Rel-17. NOTE 8: Test Point ID 3 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3458.79MHz (NR ARFCN=630533). NOTE 9: Test Point ID 4 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3593.79MHz (NR ARFCN=639533). NOTE 10: Test Point ID 5 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 4021.89MHz (NR ARFCN=668067). NOTE 11: Test Point ID 7 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3538.29MHz (NR ARFCN=635867). NOTE 12: Test Point ID 1 for CA_n66A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3825.24MHz (NR ARFCN=655000). NOTE 13: Test Point ID 3 for CA_n66A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 4065.18MHz (NR ARFCN=671000). NOTE 14: Test point ID 6 for CA_n66A-n77A tests protected E-UTRA band 103 introduced in Rel-17. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] subclause4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.3.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.3.2.1.4.3.

<Unchanged Sections Skipped>

#### 6.5A.4.1	Transmit intermodulation for CA (2UL CA)

6.5A.4.1.1	Test purpose

To verify that the UE transmit intermodulation does not exceed the described value in the test requirement.

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.

6.5A.4.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA.

6.5A.4.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.4.0.

6.5A.4.1.4	Test description

6.5A.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.4.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.4.1.4.1-1: Inter-band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Mid range for PCC and SCC (NOTE 4) |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg for both PCC and SCC Highest NRB_agg for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Smallest and biggest supported SCS per Channel Bandwidth |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation(NOTE 3) | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 13 | N/A | DFT-s-OFDM PI/2 BPSK | Inner Full | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |

<Unchanged Sections Skipped>

### 7.9A.1	Spurious emissions for CA (2DL CA)

<Unchanged Sections Skipped>

7.9A.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR CA bands specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in Table 7.9A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

## {End of changes}