3GPP TSG-RAN5 Meeting #99	R5-233190

# Incheon, Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1623 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional editors note to Power Boost relevant test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas ADT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some of power boost test which test requirement are reference to TS38.521-2 which still have remaining incomplete part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added editor’s note to TC6.2B.4.1.4_1 to align with referred test requirement of TS38.521-2 TC6.2.4_1. - Added editor’s note to 6.5B.2.4.1a to align with referred test requirement of TS38.521-2 TC6.5.2.1_1. - Added editor’s note to 6.5B.3.4.1a to align with referred test requirement of TS38.521-2 TC6.5.3.1_1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases completion level will keep misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.4.1.4_1, 6.5B.2.4.1a, 6.5B.3.4.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## {Start of changes}

##### 6.2B.4.1.4_1	Configured Output Power with Power Boost for Inter-Band EN-DC including FR2 (1 NR CC)

Editor’s note: The following aspects are either missing or not yet determined:

- The referred test case 6.2.4_1 in TS 38.521-2 is incomplete for power class 1, 2 and 4.

- The referred test case 6.2.4_1 in TS 38.521-2 is incomplete for band n259.

6.2B.4.1.4_1.1	Test purpose

Same test purpose as in clause 6.2.4_1.1 in TS 38.521-2 [9] for the NR carrier.

6.2B.4.1.4_1.2	Test applicability

This test case applies to all types of E-UTRA UE release 16 and forward, supporting inter-band EN-DC including FR2 with 1 NR UL CC and supporting mpr-PowerBoost-FR2-r16 UE capability.

6.2B.4.1.4_1.3	Minimum conformance requirements

UE configured output power requirement for E-UTRA single carrier and CA operation specified in subclauses 6.2.5 and 6.2.5A of [10] and for NR single carrier specified in subclause 6.2.4_1 of [9] apply.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.4.1.4.

6.2B.4.1.4_1.4	Test description

Same test description as in clause 6.2.4_1.4 in TS 38.521-2 [9] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.2.4_1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 4.6-1.

3.1.	Downlink E-UTRA signals are initially set up according to TS 36.521-1 [10] Annex C, clauses C.0, C.1 and C.3.0, and uplink signals according to Annex H, clauses H.1 and H.3.0.

4.1.	The E-UTRA UL Reference Measurement channels are set according to Table 4.6-1.

Step 6 of Initial conditions as in clause 6.2.4_1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508 [6] clause 4.5.

Same test procedure as in clause 6.2.4_1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.2B.4.1.4_1.5	Test requirement

Same test requirement as in clause 6.2.4_1.5 in TS 38.521-2 [9] for the NR carrier.

<Unchanged Sections Skipped>

##### 6.5B.2.4.1a	Spectrum emissions mask with Power Boost for Inter-band EN-DC including FR2 (1 NR CC)

Editor’s note: The following aspects are either missing or not yet determined:

- The referred test case 6.5.2.1_1 in TS 38.521-2 is incomplete for power class 1, 2 and 4.

6.5B.2.4.1a.1	Test purpose

Same test purpose as in clause 6.5.2.1_1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.2.4.1a.2	Test applicability

This test case applies to all types of NR UE release 16 and forward supporting mpr-PowerBoost-FR2-r16 UE capability, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.5B.2.4.1a.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.1_1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

6.5B.2.4.1a.4	Test description

Same test description as in clause 6.5.2.1_1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.7-1. For Initial conditions as in clause 6.5.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2.4_1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5.2.1_1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.2.4.1a.5	Test requirement

Same test requirement as in clause 6.5.2.1_1.5 in TS 38.521-2 [9] for the NR carrier.

<Unchanged Sections Skipped>

##### 6.5B.3.4.1a	General Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC)

Editor’s note: This clause is complete for Band n257, n258, n260 and n261 and PC3. The following aspects of the clause are for future consideration:

- The referred test case 6.5.3.1_1 in TS 38.521-2 [9] is incomplete for frequency above 80 GHz.

6.5B.3.4.1a.1	Test purpose

Same test purpose as in clause 6.5.3.1_1.1 in TS 38.521-2 [9] for the NR carrier.

6.5B.3.4.1a.2	Test applicability

This test case applies to all types of NR UE release 16 and forward supporting mpr-PowerBoost-FR2-r16 UE capability, supporting inter-band EN-DC including FR2 with 1 NR UL CC.

6.5B.3.4.1a.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.3.1_1.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.3.4.

6.5B.3.4.1a.4	Test description

Same test description as in clause 6.5.3.1_1.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 6.5.3.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.2.4_1.4 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 6.5.3.1.4.1 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

6.5B.3.4.1a.5	Test requirement

Same test requirement as in clause 6.5.3.1_1.5 in TS 38.521-2 [9] for the NR carrier.

## {End of changes}