3GPP TSG WG RAN5 meeting#99						R5-233201

Incheon, South Korea, 22nd May 2023 - 26th May 2023

Source: 	Qualcomm CDMA Technologies

Title: 	Discussion to add coverage for TEI16 feature - Redirection with MPS Indication

Document for:	Discussion and Decision

Agenda Item: 	6.4.7

# 1	Introduction

TEI_16 RAN2 features includes Introduction of Redirection with MPS Indication which was approved in RAN#92-e as CR2413 [2] for TS38.331 cl 5.3.3.3, 5.3.13.2, 5.3.14.5 for NR and CR4579 [1] for TS36.331 cl 5.3.3.3, 5.3.3.3a, 5.3.3.11, 5.3.16.5 for LTE.

RAN5 has no test coverage for these features in TS 36.523-1 and TS 38.523-1.

# 2	Discussion

When the network performs a release with redirection, for UEs with ongoing MPS priority session(s), the network may include an MPS priority indicator in the RRCRelease message based on UE’s capability to support it. When connecting to the target network and the connection establishment is the result of release with redirection with mpsPriorityIndication, the UE sets the RRC Establishment Cause to mps-PriorityAccess if the target RAN is NR and to highPriorityAccess if the target is E-UTRA. This applies to UEs without a MPS subscription that initiate a session and MPS is established for the session or to a terminating UE with an incoming MPS session.

Redirection with MPS Indication in Rel-16 versions of TS 38.331 applies to Redirection from –

- NR to: NR

- NR to LTE(EPC),

- NR to LTE(5GC)

Redirection with MPS Indication in Rel-16 versions of TS 36.331 applies to Redirection from -

- LTE(5GC) to NR or

- LTE(EPC) to NR

- LTE(EPC) to LTE(EPC)

- LTE(5GC) to LTE (EPC)

Along with MPSRelease to Redirection check there are some access barring requirements in respective specifications - cl 5.3.14.5 of TS38.331 and cl 5.3.3.11 of TS36.331.

Initial Proposed test case list for NR and LTE (EPC) combinations:

| TS | Clause | Release | Title | Test purpose / Description |
| --- | --- | --- | --- | --- |
| 38.523-1 | 8.1.5.5.x | Rel-16 | Redirection from NR to E-UTRA / MPS Priority Indication | To verify that if the UE has an indication of MPS priority for redirection, the UE sets the RRC Establishment Cause to mps-PriorityAccess to indicate to the network that the request is to be handled with MPS priority. |
| 38.523-1 | 8.1.5.5.x | Rel-16 | Redirection to NR / From another NR frequency / MPS Priority Indication | To verify that if the UE has an indication of MPS priority for redirection, the UE sets the RRC Establishment Cause to mps-PriorityAccess to indicate to the network that the request is to be handled with MPS priority. |
| 38.523-1 | 8.1.5.5.x | Rel-16 | Redirection to E-UTRA to NR / MPS Priority Indication | To verify that if the UE has an indication of MPS priority for redirection, the UE sets the RRC Establishment Cause to mps-PriorityAccess to indicate to the network that the request is to be handled with MPS priority. |
| 38.523-1 | 11.3.x | Rel-16 | UAC / Access Identity 1/ 0% access probability / release with redirect with mpsPriorityIndication / RRC_INACTIVE | To verify that the UE access attempt shall not be barred if the UE connection establishment is the result of release with redirection with mpsPriorityIndication and access barring for MPS priority is not in effect. |
| 36.523-1 | x.y | Rel-16 | TBD | TBD |

# 3	Proposal

1. Given RAN5 has de-scoped LTE(5GC), NR to NR, NR to LTE(EPC) or vice versa combinations and requirements therefore are proposed to be added in TS38.523-1[3]

2. LTE (EPC) to LTE (EPC) tests will also be scoped and proposed later.

3. A separate WID is not required to track this feature given the limited impact

# 4	References

- R2-2106796	Redirection with MPS Indication [Redirect_MPS_I]

- R2-2106797	Redirection with MPS Indication [Redirect_MPS_I]

- TS 38.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 1: Protocol