3GPP TSG-RAN WG5 Meeting # 99	R5-233218

# Incheon, South Korea, 22 – 26 May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2309 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the minimum guardband calculation for FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | [RAN4 dependency] In RAN4#106 meeting, a CR in R4-2303542 was agreed to correct the missing [kHz] unit in the definition of the minimum guardband and SCS for FR1. However, there are no corresponding changes for FR2. Furthermore, the symbol GBchannel, said to be defined in clause 5.3.3 which is referred by other clauses in the spec, does not appear in clause 5.3.3. It may cause confusion when other clauses reference it. The definition of symbol GBchannel should be added for the consistency in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add missing unit for minimum guardband and for SCS in clause 3.2. - Add the definition for symbol GBchannel in clause 5.3.3. - Some other typo corrections in clause 3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition for symbol GBchannel is missing and may cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔFTX-RX	Δ Frequency of default TX-RX separation of the FDD operating band

∆MPRc	Allowed Maximum Power Reduction relaxation for serving cell c

ΔPPowerClass	Adjustment to maximum output power for a given power class

ΔR1R	Reference sensitivity adjustment due to support for 1 antenna ports

RB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔRIB,4R	Reference sensitivity adjustment due to support for 4 antenna ports

ΔShift	Channel raster offset

TC	Allowed operating band edge transmission power relaxation

TC,c	Allowed operating band edge transmission power relaxation for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

BWChannel	Channel bandwidth

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz

BWChannel,max	Maximum channel bandwidth supported among all bands in a release

BWGB	max(BWGB,Channel(k))

BWGB,Channel(k)	Minimum guard band defined in clause 5.3A.1 of carrier k

BWDL	Channel bandwidth for DL

BWUL	Channel bandwidth for UL

BWinterferer	Bandwidth of the interferer

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

FC	RF reference frequency on the channel raster, given in table 5.4.2.2-1

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC, low	The Fc of the lowest carrier, expressed in MHz

FC, high	The Fc of the highest carrier, expressed in MHz

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset., low

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset., high

Fedge, low	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge,low = FC,low - Foffset,low

Fedge, high	The higher edge of aggregated channel bandwidth, expressed in MHz. Fedge,high = FC,high + Foffset,high

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FInterferer	Frequency of the interferer

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

Foffset	Frequency offset from FC, high to the higher edge or FC, low to the lower edge

Foffset,high	Frequency offset from FC,high to the upper UE RF Bandwidth edge, or from FC,block, high to the upper sub-block edge

Foffset,low	Frequency offset from FC,low to the lower UE RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FREF,Shift	RF reference frequency for Supplementary Uplink (SUL) bands and for the uplink for all FDD bands

Fuw (offset)	The frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

GBChannel	Minimum guard band defined in clause 5.3.3, expressed in kHz

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

expressed in units of resources blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

Physical resource block number

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth

NRB_agg=1jNRBj*2μj for carrier 1 to j, where μ is defined in TS 38.211 [8]

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks

NRB,cj=NRBj*2μj for carrier j, where μ is defined in TS 38.211 [8]

NRB,largest BW	The largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in units of resource blocks

NRB,low	The transmission bandwidth configurations according to Table 5.3.2-1 for the lowest assigned component carrier in clause 5.3A.1

NRB,high	The transmission bandwidth configurations according to Table 5.3.2-1 for the highest assigned component carrier in clause 5.3A.1

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

PCMAX	The configured maximum UE output power

PCMAX, c	The configured maximum UE output power for serving cell c

PCMAX, f, c	The configured maximum UE output power for carrier f of serving cell c in each slot

PEMAX	Maximum allowed UE output power signalled by higher layers

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c

PInterferer	Modulated mean power of the interferer

Plargest BW	Power of the largest transmission bandwidth configuration of the component carriers in the bandwidth combination

PPowerClass	The nominal UE power (i.e., no tolerance)

P-MPRc	Power Management Maximum Power Reduction for serving cell c

PRB	The transmitted power per allocated RB, measured in dBm

PUMAX	The measured configured maximum UE output power

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

RBStart	Indicates the lowest RB index of transmitted resource blocks

RBstart_CA	Indicates the lowest RB index of transmitted resource blocks for intra-band contiguous CA

SCSc	SCS for the component carrier c, expressed in kHz

SCSlargest BW	SCS for the largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in kHz

SCSlow	SCS for the lowest assigned component carrier in section 5.3A.1, expressed in kHz

SCShigh	SCS for the highest assigned component carrier in section 5.3A.1, expressed in kHz

tp	Transient Period value signalled by the UE

tpstart	Start position of transient period relative to the symbol boundary

T(PCMAX, f, c)	Tolerance for applicable values of PCMAX, f, c for configured maximum UE output power for carrier f of serving cell c

TL,c	Absolute value of the lower tolerance for the applicable operating band as specified in section 6.2.1

SSREF	SS block reference frequency position

UTRAACLR		UTRA ACLR

<< Unchanged sections omitted >>

5.3.3	Minimum guard band and transmission bandwidth configuration

The minimum guard band for each UE channel bandwidth and SCS is specified in Table 5.3.3-1.

Table 5.3.3-1: Minimum guard band for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1065 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1610 | 1590 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

NOTE: The minimum guard bands have been calculated using the following equation: GBchannel = (BWchannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2, where NRB are from Table 5.3.2-1 and GBchannel expressed in kHz.

Figure 5.3.3-1: Void

The number of RBs configured in any channel bandwidth shall ensure that the minimum guard band specified in this clause is met.

Figure 5.3.3-2: UE PRB utilization

In the case that multiple numerologies are multiplexed in the same symbol due to BS transmission of SSB, the minimum guard band on each side of the carrier is the guard band applied at the configured channel bandwidth for the numerology that is received immediately adjacent to the guard.

If multiple numerologies are multiplexed in the same symbol and the UE channel bandwidth is >50 MHz, the minimum guardband applied adjacent to 15 kHz SCS shall be the same as the minimum guardband defined for 30 kHz SCS for the same UE channel bandwidth.

Figure 5.3.3-3: Guard band definition when transmitting multiple numerologies

NOTE: Figure 5.3.3-3 is not intended to imply the size of any guard between the two numerologies. Inter-numerology guard band within the carrier is implementation dependent.

[For a UE supporting wideband operation], the nominal intra-cell guard bands and the corresponding sizes of the RB sets separated by the said guard bands are as specified in Table 5.3.3-2 for each UE channel bandwidth and sub-carrier spacing for the downlink and uplink. The nominal intra-cell guard bands in Table 5.3.3-2 are applicable when the respective IE intraCellGuardBandsUL-List and intraCellGuardBandsDL-List [6] for the uplink and downlink are not provided, as specified in [12] clause 7.

Table 5.3.3-2: Nominal intra-cell guard bands for wideband operation

| SCS (kHz) | 40 MHz | 60 MHz | 80 MHz |
| --- | --- | --- | --- |
| 15 | 105-6-105 (216) | N/A | N/A |
| 30 | 50-6-50 (106) | 50-6-50-6-50 (162) | 50-6-50-5-50-6-50 (217) |
| 60 | 23-5-23 (51) | 23-5-23-5-23 (79) | 23-5-23-5-23-5-23 (107) |
| NOTE 1: The intra-cell guard band is denoted TBW0-GB0-…-GBN_RBset-2-TBWN_RBset-1 for N_RBset > 1 number of RB-sets with TBWr the maximum transmission bandwidth (PRB) of RB-set r and GBr the guard band (PRB) above the upper edge of RB-set r. The RB-set 0 is starting at the first common resource block (CRB) of the carrier as indicated by offsetToCarrier. The total transmission bandwidth configuration (size of resource grid) including guard bands is given in between parentheses. |  |  |  |

For a UE that supports shared spectrum channel access, there are no uplink or downlink intra-cell guard bands for operation with 10 MHz and 20 MHz channel bandwidths; the maximum transmission bandwidth configurations for these channel bandwidths are in accordance with clause 5.3.2.

For each UE channel bandwidth and sub-carrier spacing given by Table 5.3.3-2, the maximum transmission bandwidth configuration of the carrier including intra-cell guard bands, if configured for the uplink and downlink by the respective IE intraCellGuardBandsUL-List and intraCellGuardBandsDL-List [6], and corresponding RB-set(s) shall be in accordance with clause 5.3.2 with a minimum inter-cell guard band of the UE channel bandwidth as specified in Table 5.3.3-1 for the uplink and downlink. Minimum requirements specified for wideband operation in Clause 6 and Clause 7 also apply for intra-cell guard bands larger than the nominal sizes in Table 5.3.3-2 as listed in Table 5.3.3-3 for each sub-carrier spacing; each guard band in order of CRB index must be larger than or equal to the corresponding nominal guard band specified in Table 5.3.3-2 for each channel bandwidth.

Table 5.3.3-3: Applicable intra-cell guard bands for wideband operation

| Parameter | Unit | SCS |  |
| --- | --- | --- | --- |
|  |  | 15 kHz | 30 kHz |
| Intra-cell guard band (size) | PRB | 6,7 | 5,6,7 |
| Transmission bandwidth (size) of RB-set | PRB | 104,105 | 49,50,51 |

If the UE is configured with zero width intra-cell guard bands for the uplink and downlink by the IE intraCellGuardBandsUL-List and intraCellGuardBandsDL-List [6] on a carrier greater than 20 MHz, the maximum transmission bandwidth configuration for the uplink and downlink shall be in accordance with clause 5.3.2 with a minimum inter-cell guard band of the UE channel bandwidth as specified in Table 5.3.3-1.

<< End of changes >>