3GPP TSG-RAN WG5 Meeting #99	R5-233232

Incheon, Korea, 22nd -26th May 2023

Title:	IoT NTN discussion on satellite type coverage in testing

Source:	Keysight Technologies UK Ltd

Agenda item:	5.3.38.9

Document for:	Discussion and endorsement

# 1.	Introduction

RAN4 has been working on the definition of core and performance requirements for NR User Equipment (UE) supporting satellite access operation as part of the work item defined in [1] during Rel-18 with outcome applicable from Rel-17 ([2]):

The outcome of this work has been included in 3GPP TS 36.102 ([3]) for RF and demodulation requirements while RRM requirements have been added to 3GPP TS 36.133 ([4]).

These specifications are supposed to cover both Geosynchronous Orbit (GSO) satellites as well as Non-Geosynchronous (NGSO) satellites although not explicitly stated in the work item definition ([1])

This document analyses the applicability of the RAN4 requirements defined for IoT NTN in terms of the type of satellite and proposes a way forward to progress on test specifications implications related to this topic.

# 2. 	Discussion

## 2.1 Types of satellites

Both Geosynchronous and Non-Geosynchronous orbit satellites are under the scope of NR NTN Work Item.

These satellites have different characteristics:

- NGSO (Non – Geosynchronous Orbit)

The main characteristics of NGSO satellites are:

- NGSO satellites includes Low-Earth Orbit (LEO) and Medium-Earth Orbit (MEO) satellites:

  - LEO satellites are in orbits in an altitude range between 300 km and 1500 km [5].

  - MEO satellites are in orbits in an altitude range between 7000 km and 25000 km [5].

- High relative satellite velocity with respect to the UE.

-GSO (Geosynchronous Orbit)

The main characteristics of GSO satellites are:

- GSO satellites are located in an Earth-centred orbit at a constant altitude of 35,786 km with an orbital period that matches Earth's rotation on its axis (23 hours, 56 minutes, and 4 seconds).

  - For an observer on Earth's surface, a satellite in a geosynchronous orbit returns to exactly the same position in the sky after a period of one sidereal day.

- Over the course of a day, the object's position in the sky may remain still or trace out a path, typically in a figure-8 form, whose precise characteristics depend on the orbit's inclination and eccentricity.

- A special case of geosynchronous orbit satellites is the geostationary orbit satellite (GEO), which is a circular geosynchronous orbit in Earth's equatorial plane with both inclination and eccentricity equal to 0.

  - A satellite in a geostationary orbit remains in the same position in the sky to observers on the surface.

- Low relative satellite velocity with respect to the UE (compared to NGSO).

## 2.2 UE capability indicating the type of satellite supported

According to 3GPP TS 36.306 ([6]), in IoT Non-Terrestrial Networks communications, the device indicates that supports IoT NTN access through the UE capability ntn-Connectivity-EPC-r17:

UE can indicate the type of satellite supported through UE capability ntn-ScenarioSupport-r17:

Observation 1: There could be devices supporting only GSO satellites.

Observation 2: There could be devices supporting only NGSO satellites.

Observation 3: There could be devices supporting both GSO and NGSO satellites.

## 2.2 Configuration of the type of satellite

According to 3GPP TS 36.331 ([7]), the type of satellite will be configured through the satellite ephemerisInfo-r17 IE embedded in servingSatelliteInfo-r17 IE sent in SIB31:

Ephemeris can be configured through position and velocity or through orbital parameters.

Observation 4: The type of satellite is configured through the ephemerisInfor-r17 IE.

## 2.3 Type of satellites in RF core requirements

There is no reference to the type of satellites in RF core requirements ([3]). Hence, it is assumed that all core requirements for UE transmitter and receiver apply to both GSO and NGSO satellites.

Observation 5: All core requirements for UE transmitter and receiver apply to both GSO and NGSO satellites.

When defining UE transmitter and receiver test cases for IoT NTN, considering observations 1-5, different message contents should be defined for either GSO and NGSO.

Proposal 1: Define message contents for GSO and NGSO satellites in NR NTN UE transmitter and receiver test cases ([8]).

In case the device supports both GSO and NGSO satellites, it should be discussed whether test cases should be executed for each type of satellite or whether testing one of them could ensure compliance with the core requirements.

Proposal 2: For UE supporting both GSO and NGSO satellites, further discuss whether RF test cases should be executed for each type of satellite or whether testing one of them could ensure compliance with the core requirements.

Within GSO satellites, considering that varying delay and Doppler shifts in satellites with inclination different from 0º (not GEO) represent a worse case than GEO satellites, it is proposed to test only GSO satellites different from GEO.

Proposal 3: In IoT NTN UE transmitter and receiver test cases ([8]), when defining message contents for GSO, consider only GSO satellites different from GEO.

## 2.4 Type of satellites in demod requirements

Regarding demod test cases, R4-2220278 ([9]) included the following agreements:

Consequently, section 8.2.1.2 is to be added to the 3GPP TS 36.102 specification ([2]) according to R4-2305961 ([10]):

Observation 6: The only demod requirement defined so far for PDSCH in 3GPP TS 36.102 ([3]) is only applicable to LEO-600 (NGSO) satellite.

Proposal 4: Define message contents for NGSO satellite only (LEO-600) in PDSCH demod NR NTN test cases ([8]).

In case demod requirements are added to 3GPP TS 36.521-4 for GSO satellites to consider required signalling for IoT NTN although non-Terrestrial demod requirements apply, define message contents for GSO satellite too in PDSCH demod IoT NTN test cases ([8]).

Within GSO satellites, considering that varying delay and Doppler shifts in satellites with inclination different from 0º (not GEO) represent a worse case than GEO satellites, it is proposed to test only GSO satellites different from GEO.

Proposal 5: In case demod requirements are added to 3GPP TS 36.521-4 for GSO satellites, define message contents for GSO satellite too (different from GEO) in PDSCH demod IoT NTN test cases ([8]).

## 2.5 Type of satellites in RRM requirements

Regarding RRM test cases, according to R4-2306386 ([11]), each performance requirement defines the applicability of each type of satellite as follows:

Observation 7: In RRM requirements, it is explicitly stated the applicability of the requirements according to the type of satellites supported.

As both types of satellites will be required depending on the UE capabilities (although only one type of satellite will be tested for each UE) RRM test cases should consider the definition of message contents for both types of satellites.

Within GSO satellites, considering that varying delay and Doppler shifts in satellites with inclination different from 0º (not GEO) represent a worse case than GEO satellites, it is proposed to test only GSO satellites different from GEO.

Proposal 6: Define message contents for GSO (different from GEO) and NGSO satellites in NR NTN RRM test cases ([12]).

# 3. 	Conclusion

This document analyses the applicability of the RAN4 requirements defined for NR NTN in terms of the type of satellite and proposes a way forward to progress on test specifications implications related to this topic.

The following observations and proposals are made:

Observation 1: There could be devices supporting only GSO satellites.

Observation 2: There could be devices supporting only NGSO satellites.

Observation 3: There could be devices supporting both GSO and NGSO satellites.

Observation 4: The type of satellite is configured through the ephemerisInfor-r17 IE.

Observation 5: All core requirements for UE transmitter and receiver apply to both GSO and NGSO satellites.

Proposal 1: Define message contents for GSO and NGSO satellites in NR NTN UE transmitter and receiver test cases ([8]).

Proposal 2: For UE supporting both GSO and NGSO satellites, further discuss whether RF test cases should be executed for each type of satellite or whether testing one of them could ensure compliance with the core requirements.

Proposal 3: In IoT NTN UE transmitter and receiver test cases ([8]), when defining message contents for GSO, consider only GSO satellites different from GEO.

Observation 6: The only demod requirement defined so far for PDSCH in 3GPP TS 36.102 ([3]) is only applicable to LEO-600 (NGSO) satellite.

Proposal 4: Define message contents for NGSO satellite only (LEO-600) in PDSCH demod NR NTN test cases ([8]).

Proposal 5: In case demod requirements are added to 3GPP TS 36.521-4 for GSO satellites, define message contents for GSO satellite too (different from GEO) in PDSCH demod IoT NTN test cases ([8]).

Observation 7: In RRM requirements, it is explicitly stated the applicability of the requirements according to the type of satellites supported.

Proposal 6: Define message contents for GSO (different from GEO) and NGSO satellites in NR NTN RRM test cases ([12]).

# 4. 	References

- RP-223437, “WID on NB-IoT/eMTC core & performance requirements for NTN”, MediaTek Inc., RAN#98-e

- 3GPP TS 36.307, “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements on User Equipments (UEs) Supporting a release-independent frequency band”

- 3GPP TS 36.102, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access”

- 3GPP TS 36.133, “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”

- 3GPP TR 38.821, " Solutions for NR to support non-terrestrial networks (NTN)"

- 3GPP TS 36.306, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities”

- 3GPP TS 36.331, “Evolved Universal Terrestrial Radio Access (E-UTRA; Radio Resource Control (RRC) protocol specification”

- 3GPP TS 36.521-4, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Satellite access Radio Frequency (RF) and performance Conformance Testing”

- R4-2220278, “WF on IoT-NTN UE demodulation and CQI reporting requirements”, MediaTek, RAN4#105

- R4-2305961, “Big CR to 36.102: Demodulation requirements for IoT-NTN”, MediaTek, RAN4#106bis-e

- R4-2306386, “Draft Big CR on NB-IoT/eMTC RRM performance requirements for NTN “, MediaTek Inc., RAN4#106bis-e

- 3GPP TS 36.533, “Evolved Universal Terrestrial Radio Access (E-UTRA; User Equipment (UE) conformance specification; Radio Resource Management (RRM)”