3GPP TSG-RAN5 Meeting #99	R5-233286

# Incheon, Korea, 22th May 2023 – 26th May 2023

Agenda item:	6.4.7

Source:	Samsung

Title:	Discussion for new test case for 6.1.2 Inter frequency cell reselection

Document for:	Discussion

- Introduction

The aim of this contribution is to discuss the necessity of the new test case for TS 38.523-1 clause 6.1.2 Inter frequency cell reselection to the highest ranked cell among the cells from different inter frequencies having same reselection priority.

- Discussion

  - Background

As per the below lines from 38.304v1703, section 5.2.4.5 (5.2.4.5 NR Inter-frequency and inter-RAT Cell Reselection criteria),

If more than two cell meets the above criteria, the UE shall reselect a cell as follows:

- If the highest-priority frequency is an NR frequency, the highest ranked cell among the cells on the highest priority frequency(ies) meeting the criteria according to clause 5.2.4.6;

- If the highest-priority frequency is from another RAT, the strongest cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

If we elaborate above lines from the specification, they look like below:

- When there are multiple cells meeting the cell reselection criteria and

- These cells belong to two different highest priority NR Inter frequencies,

- Then UE should reselect to the highest ranked cell among this highest priority inter frequencies.

The main motivation of adding this test cases are, to verify whether User Equipment’s are following the points mentioned above in 38.304 specification.

As of now, there are multiple test cases covering cell reselection scenarios in 38.523-1. However, all of them are covering mostly one target cell belonging to highest priority frequency.

For example, if we take test case 6.1.2.4 Cell reselection for inter-band operation in 38.523-1,

- This test case covers only one target cell from high priority inter frequency for N2N Cell Reselection.

Whereas in other test case 6.1.2.20 Inter-frequency cell reselection according to cell reselection priority provided by SIBs,

- Here also, test case does not cover 2 neighboring target cells belonging to two different inter frequencies having the highest cell reselection priority.

- This test case covers only one target cell at a time meeting the cell reselection criteria.

Like this, we have different test cases for covering cell reselection procedures.

Observation 1: All of the test cases about the cell reselection are covering mostly one target cell belonging to highest priority frequency

  - Scenario

- UE camped on Cell 1 with Cell Reselection Priority say 1.

- NW has configured two neighboring cells (Cell 2 & Cell 3) through SIB and those two cells belong to two different NR inter frequencies and both NR inter frequencies are having same cell reselection priority say 7.

- When User Equipment perform measurements on the NR inter frequencies, UE identified that both of cells (Cell 2& Cell 3) meeting high priority cell reselection criteria.

- As per the lines mentioned above from 38.304 specification, when more than one cell meets the cell reselection criteria, UE should choose highest ranked cell among the cells from highest priority frequencies.

To verify this scenario, there is no any test case available in 38.523-1 specification with two neighbouring cells belonging to two different NR inter frequencies and both frequencies having same cell reselection priority and the reselection priority is higher than Serving Cell Reselection priority.

Observation 2:

If UE does not follow specification in above scenario, UE might reselect to any one cell among the two candidate cells. If UE does not choose the highest ranked cell among the highest priority frequencies, it can further lead to unwanted cell reselections.

Proposal:

Propose the meeting to agree to add a new test case in the R5-232893, “Inter frequency cell reselection to the highest ranked cell among the cells from different inter frequencies having same reselection priority” in order to eliminate the issue mentioned above.

- Conclusion

Observation 1: All of the test cases about the cell reselection are covering mostly one target cell belonging to highest priority frequency

Observation 2: If UE does not follow specification in above scenario, UE might reselect to any one cell among the two candidate cells. If UE does not choose the highest ranked cell among the highest priority frequencies, it can further lead to unwanted cell reselections.

Proposal: Propose the meeting to agree to add a new test case in the R5-232893, “Inter frequency cell reselection to the highest ranked cell among the cells from different inter frequencies having same reselection priority” in order to eliminate the issue mentioned above.