3GPP TSG-RAN5 Meeting #99	R5-233315

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5218 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RRC downlink segmentation test case 8.5.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - There is no specific message contents defined for Step 9. - In tables Table 8.5.5.2.3.3-3 the last nonCriticalExtension used to transfer octet content is not defined as an OCTETSTRING. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Specifica message content defined for step 9. - Field replaced with RRCConnectionReconfiguration-v8m0-IEs/ lateNonCriticalExtension |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specific message contents will be ioncorrect or missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.5.5.2	DL Message Segment transfer / RRC connection reconfiguration / RLF / Success

8.5.5.2.1	Test Purpose (TP)

(1)

with { UE has previously received and stored a segment of a segmented RRCConnectionReconfiguration message }

ensure that {

when { UE detects radio link failure }

then { UE discards all the segments of the RRCConnectionReconfiguration message }

}

(2)

with { UE has previously received and stored segment(s) of a segmented RRCConnectionReconfiguration message }

ensure that {

when { UE receives all remaining segment(s) of the RRCConnectionReconfiguration message }

then { UE successfully performs the RRC connection reconfiguration procedure }

}

8.5.5.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.11.3 and 5.6.25.3.

[TS 36.331, clause 5.3.11.3]

The UE shall:

1>	in case any DAPS bearer is configured, only the target PCell is considered in the following;

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon T318 expiry; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from MCG RLC, which is allowed to be send on PCell, that the maximum number of retransmissions has been reached for an SRB or DRB:

2>	consider radio link failure to be detected for the MCG i.e. RLF;

2>	discard any segments of segmented RRC messages received;

[TS 36.331, clause 5.6.25.3]

Upon receiving DLDedicatedMessageSegment message, the UE shall:

1>	store the segment;

1>	if all segments of the message have been received:

2>	assemble the message from the received segments and process the message according to 5.3.5 for the RRCConnectionReconfiguration message or 5.3.3.4a for the RRCConnectionResume message;

2>	discard all segments.

8.5.5.2.3	Test description

8.5.5.2.3.1	Pre-test conditions

System Simulator:

- Cell 1 and Cell 2

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4) on Cell 1 according to [18] using the UE TEST LOOP MODE B.

8.5.5.2.3.2	Test procedure sequence

Table 8.5.5.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after Preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.5.5.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | “Off” |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | “Off” | “Off” |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level value shall be such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 + Hys < M1). |

Table 8.5.5.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message | - | - |
| 1 | The SS transmits a DLDedicatedMessageSegment containing the first segment of an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 2. | <-- | DLDedicatedMessageSegment | - | - |
| 2 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 8.5.5.2.3.2-1 | - | - | - | - |
| 3 | T310 seconds after Step 2 the SS changes Cell 1 and Cell 2 parameters according to the row "T2" in table 8.5.5.2.3.2-1 | - | - | - | - |
| - | The following messages are sent on Cell 1 unless otherwise stated. | - | - | - | - |
| 4 | The UE sends RRCConnectionReestablishmentRequest message | --> | RRCConnectionReestablishmentRequest | - | - |
| 5 | The SS transmits RRCConnectionReestablishment message. | <-- | RRCConnectionReestablishment | - | - |
| 6 | The UE transmits RRCConnectionReestablishmentComplete message. | --> | RRCConnectionReestablishmentComplete | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationtComplete | - | - |
| 9 | The SS transmits a DLDedicatedMessageSegment containing the second and last segment of an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 2. | <-- | DLDedicatedMessageSegment | - | - |
| 10 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 2 in the next 1s? | --> | RRCConnectionReconfigurationComplete | 1 | F |
| 11 | The SS transmits a DLDedicatedMessageSegment containing the first segment of an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 2. | <-- | DLDedicatedMessageSegment | - | - |
| 12 | The SS transmits a DLDedicatedMessageSegment containing the second (last) segment of an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to Cell 2. | <-- | DLDedicatedMessageSegment | - | - |
| 13 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on cell 2? | --> | RRCConnectionReconfigurationComplete | 2 | P |
| 14 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 2. | <-- | IP packet | - | - |
| 15 | Check: Does the UE loop back the IP packet on the DRB associated with the default EPS bearer context on Cell 2? | --> | IP packet | 2 | P |

8.5.5.2.3.3	Specific message contents

Table 8.5.5.2.3.3-1: ACTIVATE TEST MODE (preamble)

Derivation Path: 36.508, Table 4.7A-1, condition UE TEST LOOP MODE B

Table 8.5.5.2.3.3-2: DLDedicatedMessageSegment (Steps 1, 9, 11 and 12, Table 8.5.5.2.3.2-2)

| Derivation Path: 36.331, Clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| DLDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| dlDedicatedMessageSegment-r16 SEQUENCE { |  |  |  |
| segmentNumber-r16 | 0 |  | Step 1, Step 11 |
| segmentNumber-r16 | 1 |  | Step 12 |
| rrc-MessageSegmentContainer-r16 | OCTET STRING (CONTAINING First segment of the RRCConnectionReconfiguration message) | Table 8.5.5.2.3.3-3 | Step 1, Step 11 |
| rrc-MessageSegmentContainer-r16 | OCTET STRING (CONTAINING Second segment of the RRCConnectionReconfiguration message) | Table 8.5.5.2.3.3-3 | Step 9, 12 |
| lateNonCriticalExtension | Not present |  |  |
| rrc-MessageSegmentType-r16 | notLastSegment |  | Step 1, Step 11 |
| rrc-MessageSegmentType-r16 | lastSegment |  | Step 9, 12 |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |

Table 8.5.5.2.3.3-3: RRCConnectionReconfiguration (Steps 1, 9, 11 and 12 Table 8.5.5.2.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| mobilityControlInfo | MobilityControlInfo |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  | OCTET STRING (CONTAINING RRCConnectionReconfiguration-v8m0-IEs) |  |
| lateNonCriticalExtension | '00,,,00'O | Add enough padding so that: maximum PDCP SDU size < encoded DLDedicatedMessageSegment size < 2 x maximum PDCP SDU size |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-r9 | Not present |  |  |
| fullConfig-r9 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |
| sCellToAddModList-r10 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| systemInformationBlockType1Dedicated-r11 | SystemInformationBlockType1-BR-r13 of Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.5.2.3.3-4: MobilityControlInfo (steps 1, 9, 11 and 12, Table 8.5.5.2.3.2-2)

| Derivation Path: 36.508 clause 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |
| } |  |  |  |