3GPP TSG-RAN5 Meeting #99	R5-233318

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Title:	Proposing 2 new test cases for closed WI NR 2-step RACH

Source:	ZTE

Agenda item:	6.4.7

Document for:	Endorsement

- Introduction

The purpose of this document is to discuss about adding 2 new NR 2-step RACH test cases in 38.523-1.

- Discussion

Work Item NR 2-step RACH was officially closed in RAN5#96 meeting. In the work plan of the work item, the below test cases have been introduced to 38.523-1. And all the 4 test cases are verifying UE’s behaviour in RRC_ CONNECTED state.

| Clause | Title |
| --- | --- |
| 7.1.1.1.7 | Random access procedure / 2-step RACH / RA_TYPE selection |
| 7.1.1.1.8 | Correct selection of RACH parameters / 2-step RACH/MSGA and PRACH resource explicitly signalled to the UE by RRC / contention free random access procedure |
| 7.1.1.1.9 | Random access procedure / Successful / 2-step RACH/ C-RNTI Based / Preamble selected by MAC itself |
| 7.1.1.1.10 | Random access procedure / 2-step RACH / not complete / RA_TYPE to 4-stepRA |

The below 2 features are not covered in the work plan:

- The UE initiates 2-step RACH procedure from RRC_IDLE state

- The UE receives MSGB with fallback indication, and fallback to 4-step RACH procedure.

Therefore, 2 new test cases need to be added to verify the features in clause 7.1.1.1 in TS 38.523-1.

To verify the feature “The UE initiates 2-step RACH procedure from RRC_IDLE state”, test case 1 may include the below TPs:

- (1) with { UE in RRC Idle state and 2-step RA type Random Access resources is configured} ensure that { when { UE has UL CCCH PDU to send } then { UE transmits a MSGA } } - (2) with { UE in RRC Idle state initiated Random Access procedure to transmit UL CCCH PDU and transmitted MSGA } ensure that { when { SS does not answer with a MSGB containing a matching successRAR within msgB-ResponseWindow } then { UE retransmits a MSGA } } - (3) with { UE in RRC Idle state initiated Random Access procedure to transmit UL CCCH PDU and transmitted MSGA } ensure that { when { SS answers with a matching MSGB within msgB-ResponseWindow } then { UE assumes RACH procedure as complete } }

To verify the feature “The UE receives MSGB with fallback indication, and fallback to 4-step RACH procedure”, test case 2 may include the below TPs:

- (1) with { UE in RRC Idle state initiated Random Access procedure to transmit UL CCCH PDU and transmitted MSGA } ensure that { when { SS answers with a matching MSGB including fallbackRAR within msgB-ResponseWindow } then { UE transmits a MSG3 using the UL grant scheduled in the fallbackRAR } } - (2) with { UE received a MSGB including fallback indication within msgB-ResponseWindow, and transmitted a MSG3 using the UL grant scheduled in the fallbackRAR } ensure that { when { The SS does not schedule any PDCCH transmission addressed to UE Temporary C-RNTI before Contention resolution timer expiry } then { UE retransmits a MSGA } } - (3) with { UE received a MSGB including fallbackRAR within msgB-ResponseWindow, and transmitted a MSG3 using the UL grant scheduled in the fallbackRAR } ensure that { when { The SS schedules a PDCCH transmission addressed to UE Temporary C-RNTI before Contention resolution timer expiry } then { UE assumes RACH procedure as complete } }

In 38.523-1 clause 7.1.1.1 for Random Access Procedures, there were 19 test case and the clause number of the last one is 7.1.1.1.18. We can set the new 2 test cases with number 7.1.1.1.19 and 7.1.1.1.20.

7.1.1.1 Random Access Procedures 566 7.1.1.1.1 Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by RRC / contention free random access procedure 566 7.1.1.1.1a Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / contention free random access procedure 575 7.1.1.1.2 Random access procedure / Successful / C-RNTI Based / Preamble selected by MAC itself 581 7.1.1.1.3 Random access procedure / Successful / SI request 597 7.1.1.1.4 Random access procedure / Successful / Beam Failure / Preamble selected by MAC itself / Non Contention Free RACH procedure 607 7.1.1.1.5 Random access procedure / Successful / Supplementary Uplink 623 7.1.1.1.6 Random access procedure / Successful/ Temporary C-RNTI Based / Preamble selected by MAC itself 628 7.1.1.1.7 Random access procedure / Successful/ Temporary C-RNTI Based / Preamble selected by MAC itself 641 7.1.1.1.8 Correct selection of RACH parameters / 2-step RACH/MSGA and PRACH resource explicitly signalled to the UE by RRC / contention free random access procedure 650 7.1.1.1.9 Random access procedure / Successful / 2-step RACH/C-RNTI Based / Preamble selected by MAC itself 661 7.1.1.1.10 Random access procedure / 2-step RACH/not complete/ RA_TYPE to 4-stepRA 676 7.1.1.1.11 Random access procedure / Successful/ Slice specific RACH configuration 683 7.1.1.1.12 Random access procedure / Successful/ ra-PrioritizationForSlicing 687 7.1.1.1.13 Random access procedure / Successful / Slice specific RACH configuration / 2-step RACH 691 7.1.1.1.14 Random access procedure / Successful/ ra-PrioritizationForSlicingTwoStep / 2-step RACH 695 7.1.1.1.15 701 7.1.1.1.16 Random access procedure / RedCap UE identification / Msg3-based / CCCH1 701 7.1.1.1.17 Random access procedure / RedCap UE identification 703 7.1.1.1.18 Random access procedure / Msg3 repetition indication / Random access resources selection 708 7.1.1.2 Downlink Data Transfer 710

According to the TPs and features of the 2 test case, RAN5 can added 2 new test cases with following information:

| Clause | Title |
| --- | --- |
| 7.1.1.1.9a | Random access procedure / 2-step RACH / Successful / RRC_IDLE |
| 7.1.1.1.10a | Random access procedure / 2-step RACH/ Fallback for CBRA |

- Proposal

Proposal 1: RAN5 add new 2 test cases in 38.523-1 with following information, and add applicability for the new test cases in 38.523-2.

| Clause | Title |
| --- | --- |
| 7.1.1.1.9a | Random access procedure / 2-step RACH / Successful / RRC_IDLE |
| 7.1.1.1.10a | Random access procedure / 2-step RACH/ Fallback for CBRA |

References

- R5-224064, WP   NR_2step_RACH-UEConTest

- R5-224065, SR   NR_2step_RACH-UEConTest

- R5-232286, New 2-step RACH test case for UE in RRC_IDLE state

- R5-232287, New 2-step RACH test case for Fallback for CBRA

- R5-232295, Add applicabilities for new NR 2 step RACH test cases