3GPP TSG-RAN5 Meeting #99	R5-233334

# Incheon, South Korea, 22nd – 26th May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3811 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to test case 8.2.2.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Main behaviour require verdict for IP data check. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case 8.2.2.7.1 have been updated for - Main behaviour to remove verdict for data check. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Verdict remains for IP data check in main behaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-233120 and the outcome of 1 revision to this document. |  |  |  |  |  |  |  |  |

##### 8.2.2.7.1	Bearer Modification / Handling for bearer type change without security key change / EN-DC

8.2.2.7.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with EN-DC, and, MCG(s) (E-UTRA PDCP) and SCG }

ensure that {

when { UE receives an RRCConnectionReconfiguration message to modify SCG DRB to Split DRB }

then { UE reconfigures the DRB and sends an RRCConnectionReconfigurationComplete message }

}

(2)

with { UE in RRC_CONNECTED state with EN-DC, and, MCG(s) (E-UTRA PDCP) and Split }

ensure that {

when { UE receives an RRCConnectionReconfiguration message to modify Split DRB to MCG DRB (NR PDCP) }

then { UE reconfigures the DRB and sends an RRCConnectionReconfigurationComplete message }

}

(3)

with { UE in RRC_CONNECTED state with EN-DC, and, MCG(s) (E-UTRA PDCP) and MCG (NR PDCP) }

ensure that {

when { UE receives an RRCConnectionReconfiguration message to modify MCG DRB (NR PDCP) to Split DRB  }

then { UE reconfigures the DRB and sends an RRCConnectionReconfigurationComplete message }

}

(4)

with { UE in RRC_CONNECTED state with EN-DC, and, MCG(s) (E-UTRA PDCP) and Split }

ensure that {

when { UE receives an RRCConnectionReconfiguration message to modify Split DRB to SCG DRB }

then { UE reconfigures the DRB and sends an RRCConnectionReconfigurationComplete message }

}

(5)

with { UE in RRC_CONNECTED state with EN-DC, and, MCG(s) (E-UTRA PDCP) and SCG }

ensure that {

when { UE receives an RRCConnectionReconfiguration message to modify SCG DRB to MCG DRB (NR PDCP) }

then { UE reconfigures the DRB and sends an RRCConnectionReconfigurationComplete message }

}

(6)

with { UE in RRC_CONNECTED state with EN-DC, and, MCG(s) (E-UTRA PDCP) and MCG (NR PDCP) }

ensure that {

when { UE receives an RRCConnectionReconfiguration message to modify MCG DRB (NR PDCP) to SCG DRB }

then { UE reconfigures the DRB and sends an RRCConnectionReconfigurationComplete message }

}

8.2.2.7.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in TS 36.331, clauses 5.3.5.3 and TS 38.331: 5.3.5.3, 5.3.5.5.1, 5.3.5.6.1 and 5.3.5.6.5. Unless and otherwise stated these are Rel-15 requirements

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if this is the first RRCConnectionReconfiguration message after successful completion of the RRC connection re-establishment procedure:

…

1>	else:

2>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10;

NOTE 3:	If the RRCConnectionReconfiguration message includes the establishment of radio bearers other than SRB1, the UE may start using these radio bearers immediately, i.e. there is no need to wait for an outstanding acknowledgment of the SecurityModeComplete message.

…

1>	if the received RRCConnectionReconfiguration includes the nr-SecondaryCellGroupConfig:

2>	perform NR RRC Reconfiguration as specified in TS 38.331 [82, 5.3.5.3];

1>	if the received RRCConnectionReconfiguration includes the nr-RadioBearerConfig1:

2>	perform radio bearer configuration as specified in TS 38.331 [82, 5.3.5.6];

1>	if the received RRCConnectionReconfiguration includes the nr-RadioBearerConfig2:

2>	perform radio bearer configuration as specified in TS 38.331 [82, 5.3.5.6];

…

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

…

2>	if the received RRCConnectionReconfiguration message included nr-SecondaryCellGroupConfig:

3>	include scg-ConfigResponseNR in accordance with TS 38.331 [82, 5.3.5.3];

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration:

…

1>	if the RRCReconfiguration message contains the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

…

1>  if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (MCG is E-UTRA):

2> if RRCReconfiguration was received via SRB1:

3> submit the RRCReconfigurationComplete via the EUTRA MCG embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10];

[TS 38.331, clause 5.3.5.5.1]

The network configures the UE with one Secondary Cell Group (SCG). For EN-DC, the MCG is configured as specified in TS 36.331 [10]. The network provides the configuration parameters for a cell group in the CellGroupConfig IE.

The UE performs the following actions based on a received CellGroupConfig IE:

1>	if the CellGroupConfig contains the spCellConfig with reconfigurationWithSync:

2> perform Reconfiguration with sync according to 5.3.5.5.2;

2> resume all suspended radio bearers and resume SCG transmission for all radio bearers, if suspended;

1>	if the CellGroupConfig contains the rlc-BearerToReleaseList:

2>	perform RLC bearer release as specified in 5.3.5.5.3;

1>	if the CellGroupConfig contains the rlc-BearerToAddModList:

2>	perform the RLC bearer addition/modification as specified in 5.3.5.5.4;

1>	if the CellGroupConfig contains the mac-CellGroupConfig:

2>	configure the MAC entity of this cell group as specified in 5.3.5.5.5;

1>	if the CellGroupConfig contains the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.5.5.8;

1>	if the CellGroupConfig contains the spCellConfig:

2>	configure the SpCell as specified in 5.3.5.5.7;

1>	if the CellGroupConfig contains the sCellToAddModList:

2> perform SCell addition/modification as specified in 5.3.5.5.9

[TS 38.331, clause 5.3.5.6.1]

The UE shall perform the following actions based on a received RadioBearerConfig IE:

…

1>	if the RadioBearerConfig includes the drb-ToReleaseList:

2>	perform DRB release as specified in 5.3.5.6.4;

1>	if the RadioBearerConfig includes the drb-ToAddModList:

2>	perform DRB addition or reconfiguration as specified in 5.3.5.6.5.

[TS 38.331, clause 5.3.5.6.5]

The UE shall:

1>	for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used):

2>	establish a PDCP entity and configure it in accordance with the received pdcp-Config;

2>	configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KUPenc) associated with the KeNB/S-KgNB as indicated in keyToUse;

2>	if the DRB was configured with the same eps-BearerIdentity either by NR or E-UTRA prior to receiving this reconfiguration:

3> associate the established DRB with the corresponding eps-BearerIdentity;

2> else:

3>	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration:

2>	if reestablishPDCP is set:

3>	configure the PDCP entity of this RadioBearerConfig to apply the ciphering algorithm and KUPenc key associated with the KeNB/S-KgNB as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

3>	re-establish the PDCP entity of this DRB as specified in 38.323 [5], section 5.1.2;

2>	else, if recoverPDCP is set:

3>	trigger the PDCP entity of this DRB to perform data recovery as specified in 38.323;

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config.

8.2.2.7.1.3	Test description

8.2.2.7.1.3.1	Pre-test conditions

System Simulator:

-	E-UTRA Cell 1 is the PCell and NR Cell 1 is the PSCell.

UE:

None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC), Bearers (MCG(s) and SCG) established according to TS 38.508-1 [4], clause 4.5.4.

-	Else, the UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC), Bearers (MCG(s) and SCG) established and Test Loop Function (On) with UE test loop mode B according to TS 38.508-1 [4], clause 4.5.4.

8.2.2.7.1.3.2	Test procedure sequence

Table 8.2.2.2.7.1.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message containing NR RadioBearerConfig to modify SCG DRB to Split DRB. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 3 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on split DRB using NR radio path? | - | - | 1 | P |
| 4 | The SS transmits RRCConnectionReconfiguration message to modify Split DRB to MCG DRB using NR PDCP. | <-- | RRCConnectionReconfiguration | - | - |
| 5 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete (RRCReconfigurationComplete) | 2 | P |
| 6 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on MCG DRB? | - | - | 2 | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message containing NR RRCReconfiguration message to modify MCG DRB to Split DRB. | <-- | RRCConnectionReconfiguration (RRCReconfiguration) | - | - |
| 8 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete (RRCReconfigurationComplete) | 3 | P |
| 9 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on split DRB using NR radio path? | - | - | 3 | - |
| 10 | SS transmits RRCConnectionReconfiguration message containing NR RadioBearerConfig to modify Split DRB to SCG DRB. | <-- | RRCConnectionReconfiguration (RRCReconfiguration) | - | - |
| 11 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete | 4 | P |
| 12 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on SCG DRB? | - | - | 4 | - |
| 13 | SS transmits RRCConnectionReconfiguration message containing NR RRCReconfiguration message to modify SCG DRB to MCG DRB. | <-- | RRCConnectionReconfiguration (RRCReconfiguration) | - | - |
| 14 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete (RRCReconfigurationComplete) | 5 | P |
| 15 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on MCG DRB? | - | - | 5 | - |
| 16 | SS transmits RRCConnectionReconfiguration message containing NR RRCReconfiguration message to modify MCG DRB to SCG DRB. | <-- | RRCConnectionReconfiguration (RRCReconfiguration) | - | - |
| 17 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message? | --> | RRCConnectionReconfigurationComplete (RRCReconfigurationComplete) | 6 | P |
| 18 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on SCG DRB? | - | - | 6 | - |

8.2.2.7.1.3.3	Specific message contents

Table 8.2.2.7.1.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.2.7.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-SCG-to-Split |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig-SCG-to-Split. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-2: RadioResourceConfigDedicated-SCG-to-Split (step 1, Table 8.2.2.7.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.3-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-DRB ::= SEQUENCE { |  |  |  |
| drb-ToAddModList | DRB-ToAddModList-SCG-to-Split |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-3: DRB-ToAddModList-SCG-to-Split (step 1, Table 8.2.2.7.1.3.2-1)

| Derivation Path: 36.508 [7], Table 4.6.3-2A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRB-ToAddModList ::= SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 Entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| eps-BearerIdentity | 6 | Dedicated EPS bearer Id of SCG DRB |  |
| drb-Identity | 2 | SCG DRB Id |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-4: RadioBearerConfig-SCG-to-Split (step 1, Table 8.2.2.7.1.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 with condition EN-DC_DRB

Table 8.2.2.7.1.3.3-5: PDCP-Config (step 1, Table 8.2.2.7.1.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 with condition Split

Table 8.2.2.7.1.3.3-6: RRCConnectionReconfiguration (step 4, Table 8.2.2.7.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |
| release |  |  |  |
| } |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING containing RadioBearerConfig-Split-to-MCG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-7: Void

Table 8.2.2.7.1.3.3-8: Void

Table 8.2.2.7.1.3.3-9: RadioBearerConfig-Split-to-MCG (step 4, Table 8.2.2.7.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | 6 |  |  |
| } |  |  |  |
| drb-Identity | 2 |  |  |
| recoverPDCP | true |  |  |
| pdcp-Config SEQUENCE { |  |  |  |
| Drb | Omit |  |  |
| moreThanOneRLC | Omit |  |  |
| t-Reordering | Omit |  |  |
| cipheringDisabled | Omit |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-9a: Void

Table 8.2.2.7.1.3.3-10: RRCConnectionReconfiguration (step 7, Table 8.2.2.7.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING containing RRCReconfiguration-MCG-to-Split |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig-MCG-to-Split. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-11: RRCReconfiguration-MCG-to-Split (step7, Table 8.2.2.7.1.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13, condition EN-DC

Table 8.2.2.7.1.3.3-12: Void

Table 8.2.2.7.1.3.3-13: RadioBearerConfig-MCG-to-Split (step 7, Table 8.2.2.7.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | 6 |  |  |
| } |  |  |  |
| drb-Identity | 2 |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-14: PDCP-Config (step 7, Table 8.2.2.7.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| moreThanOneRLC SEQUENCE { |  |  |  |
| primaryPath SEQUENCE { |  |  |  |
| cellGroup | 1 |  |  |
| } |  |  |  |
| ul-DataSplitThreshold | infinity |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-15: RRCConnectionReconfiguration (step 10, Table 8.2.2.7.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-DRB-REL(2) |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig-Split-to-SCG. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-16: RadioResourceConfigDedicated-DRB-REL(bid) (step 10, Table 8.2.2.7.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.3-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-DRB-REL(bid) ::= SEQUENCE { |  | bid is the bearer identity |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF | one entry |  |  |
| DRB-Identity[1] | 2 | Split DRB Id |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-17: RadioBearerConfig-Split-to-SCG (step 10, Table 8.2.2.7.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | 6 |  |  |
| } |  |  |  |
| drb-Identity | 2 |  |  |
| recoverPDCP | true |  |  |
| pdcp-Config::= SEQUENCE { |  |  |  |
| Drb | Omit |  |  |
| moreThanOneRLC | Omit |  |  |
| t-Reordering | Omit |  |  |
| cipheringDisabled | Omit |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-18: RRCConnectionReconfiguration (step 13, Table 8.2.2.7.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-SCG-to-MCG |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |
| release |  |  |  |
| } |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig-SCG-to-MCG. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-19: RadioResourceConfigDedicated-SCG-to-MCG (step 13, Table 8.2.2.7.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.3-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-DRB ::= SEQUENCE { |  |  |  |
| drb-ToAddModList | DRB-ToAddModList-SCG-to-MCG |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-20: DRB-ToAddModList-SCG-to-MCG (step 13, Table 8.2.2.7.1.3.2-1)

| Derivation Path: 36.508 [7], Table 4.6.3-2A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRB-ToAddModList ::= SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 Entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| eps-BearerIdentity | 6 |  |  |
| drb-Identity | 2 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.2.4.3.3-21: Void

Table 8.2.2.7.1.3.3-22: Void

Table 8.2.2.7.1.3.3-23: RadioBearerConfig-SCG-to-MCG (step 13, Table 8.2.2.7.1.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 with condition EN-DC_DRB AND Recover_PDCP

Table 8.2.2.7.1.3.3-24: RRCConnectionReconfiguration (step 16, Table 8.2.2.7.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-DRB-REL(2) |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING containing RRCReconfiguration-MCG-to-SCG |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig-MCG-to-SCG. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-25: RadioResourceConfigDedicated-DRB-REL (bid) (step 16, Table 8.2.2.7.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.3-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-DRB-REL(bid) ::= SEQUENCE { |  | bid is the bearer identity |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF | one entry |  |  |
| DRB-Identity[1] | 2 |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-26: RRCReconfiguration-MCG-to-SCG (step 16, Table 8.2.2.7.1.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition EN-DC

Table 8.2.2.7.1.3.3-27: Void

Table 8.2.2.7.1.3.3-28: RadioBearerConfig-MCG-to-SCG (step 16, Table 8.2.2.7.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | 6 |  |  |
| } |  |  |  |
| drb-Identity | 2 |  |  |
| recoverPDCP | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.7.1.3.3-29: RRCConnectionReconfigurationComplete (steps 2, 5, 8, 11, 14, 17 Table 8.2.2.7.1.3.2-1)

| Derivation Path: TS 36.508 [7] Table 4.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| scg-ConfigResponseNR-r15 | Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |