3GPP TSG-RAN5 Meeting #99	R5-233359

Incheon, South Korea 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3736 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test case 8.2.5.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To update the test case 8.2.5.7.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test procedure sequence and specific message contents have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work plan cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-232592 and the outcome of 1 revision to this document. |  |  |  |  |  |  |  |  |

<Start of first modified section>

##### 8.2.5.7.1	Radio link failure / LBT Failure / EN-DC

8.2.5.7.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with EN-DC, and SharedSpectrum }

ensure that {

when { UE receives an IP Packet to loopback on SCG DRB and UE encounters consistent uplink LBT failures problem }

then { UE initiates the NR SCG failure information procedure to report SCGFailureInformationNR with failure type scg-lbtFailure }

}

Editor’s Note: When UE attempts to send the data at step 2, it is necessary from the SS to have a test model (FFS) that keeps channel busy, so UE detects consistent LBT failures when channel is sensed.

8.2.5.7.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321 clause 5.21.1, TS 36.331, clauses 5.6.13a.3, TS 38.331, clauses 5.3.10.3.

[TS 38.321, clause 5.21.1]

The lower layer may perform an LBT procedure, see TS 37.213 [18], according to which a transmission is not performed by lower layers if the channel is identified as being occupied. When lower layer performs an LBT procedure before a transmission and the transmission is not performed, an LBT failure indication is sent to the MAC entity from lower layers.

[TS 36.331, clause 5.6.13a.3]

The UE shall set the contents of the SCGFailureInformationNR message as follows:

1>	include failureType within failureReportSCG-NR and set it to indicate the SCG failure in accordance with TS 38.331 [82], clause 5.7.3.3;

...

The UE shall submit the SCGFailureInformationNR message to lower layers for transmission.

[TS 38.331, clause 5.3.10.3]

The UE shall:

1>	upon consistent uplink LBT failure indication from SCG MAC:

...

2>	else:

3>	consider radio link failure to be detected for the SCG, i.e. SCG RLF;

...

3>	if MCG transmission is not suspended:

4>	initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure.

8.2.5.7.1.3	Test description

8.2.5.7.1.3.1	Pre-test conditions

System Simulator:

-	E-UTRA Cell 1 is the PCell and NR Cell 1 is the PSCell

-	System Information combination as defined in TS 38.508-1 [4] clause 4.4.3.1.1 is used in E-UTRA Cell 1 and NR Cell 1.

None.

Preamble:

-	The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC) and Bearers (MCG(s) and SCG) according to TS 38.508-1 [4], Table 4.5.1-1. Test Mode (On) and Test Loop Function (On) and mode B according to TS 38.508-1 [4], Table 4.5.1-1.

8.2.5.7.1.3.2	Test procedure sequence

Table 8.2.5.7.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits one IP Packet on SCG DRB | - | - | - | - |
| 2 | The SS shall keep channel on SCG busy, so the UE detects consistent LBT failures when channel is sensed. | - | - | - | - |
| 3 | Check: Does the UE transmit in the next 5 sec (arbitrary value) a SCGFailureInformationNR message on SRB2 with failureType set to scg-lbtFailure? | --> | SCGFailureInformationNR | 1 | P |

8.2.5.7.1.3.3	Specific message contents

Table 8.2.5.7.1.3.3-1: SCGFailureInformationNR (step 3, Table 8.2.5.7.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-18AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCGFailureInformationNR-r15 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| scgFailureInformationNR-r15 SEQUENCE { |  |  |  |
| failureReportSCG-NR-r15 SEQUENCE { |  |  |  |
| failureType-r15 | Not checked |  |  |
| measResultFreqListNR-r15 | Not checked |  |  |
| measResultSCG-r15 | Not checked |  |  |
| locationInfo-r16 | Not checked |  |  |
| logMeasResultListBT-r16 | Not checked |  |  |
| logMeasResultListWLAN-r16 | Not checked |  |  |
| failureType-v1610 | scg-lbtFailure |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>