# 3GPP TSG-RAN5 Meeting #99	R5-233377

# Incheon, Korea, May 22nd - May 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3758 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of test case 8.1.6.1.2.15 for SON_MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To revise SON/MDT TC 8.1.6.1.2.15 Logged MDT / RRC_IDLE / early measurements. 1. The wording of test procedure has some misalignments with specific message, which is revised in this CR. 2. The specific message contents for step 2, 13, 14, 15, 25 and 26 are missing and needs to be added. 3. The ‘reportType’ IE in LoggedMeasurementConfiguration needs to be added as it’s mandatory. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise SON/MDT TC 8.1.6.1.2.15 Logged MDT / RRC_IDLE / early measurements. 1. Update the test procedure to resolve the misalignment between the wording in test procedure and specific message. 2. Add the specific message contents for step 2, 13, 14, 15, 25 and 26, RRCRelease, UEinformationRequest and UEInformationResponse message, as they are missing in the current TC. 3. Add ‘reportType’ IE in LoggedMeasurementConfiguration as it’s mandatory. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

8.1.6.1.2.15	Logged MDT / RRC_IDLE / early measurements

8.1.6.1.2.15.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state and has already received LoggedMeasurementConfiguration message sending by the network }

ensure that {

when { The earlyMeasIndication-r17 is configured in LoggedMeasurementConfiguration }

then { UE is allowed to log measurements on early measurement frequencies in logged  MDT }

}

(2)

with { UE in NR RRC_IDLE state and has already received LoggedMeasurementConfiguration message sending by the network }

ensure that {

when { The earlyMeasIndication-r17 is not configured in LoggedMeasurementConfiguration }

then { UE shall not log measurements on early measurement frequencies in logged  MDT }

}

8.1.6.1.2.15.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.331, clause 5.5a.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.5a.3.2]

While T330 is running and SDT procedure is not ongoing, the UE shall:

1>	if measurement logging is suspended:

2>	if during the last logging interval the IDC problems detected by the UE is resolved, resume measurement logging;

1>	if not suspended, perform the logging in accordance with the following:

……

2>	when performing the logging:

……

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include measurements of neighbouring cell that became available during the last logging interval and according to the following:

4>	include measurement results for at most 6 neighbouring cells on the NR serving frequency and for at most 3 cells per NR neighbouring frequency and for the NR neighbouring frequencies in accordance with the following:

5>	if interFreqTargetInfo is included in VarLogMeasConfig:

6>	if earlyMeasIndication is included in VarLogMeasConfig;

7>	include measurement results for NR neighbouring frequencies that are included in both interFreqTargetInfo and either in measIdleCarrierListNR (within the VarMeasIdleConfig) or SIB4;

6>	else:

7>	include measurement results for NR neighbouring frequencies that are included in both interFreqTargetInfo and SIB4;

5>	else:

6>	if earlyMeasIndication is included in VarLogMeasConfig;

7>	include measurement results for NR neighbouring frequencies that are included in either measIdleCarrierListNR (within the VarMeasIdleConfig) or SIB4;

6>	else:

7>	include measurement results for NR neighbouring frequencies that are included in SIB4;

4>	include measurement results for at most 3 neighbours per inter-RAT frequency in accordance with the following:

5>	if earlyMeasIndication is included in VarLogMeasConfig:

6>	include measurement results for inter-RAT neighbouring frequencies that are included in either measIdleCarrierListEUTRA (within the VarMeasIdleConfig) or SIB5;

5>	else:

6>	include measurement results for inter-RAT frequencies that are included in SIB5;

4>	for each neighbour cell included, include the optional fields that are available;

……

8.1.6.1.2.15.3	Test description

8.1.6.1.2.15.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.6.1.2.15.3.2	Test procedure sequence

Table 8.1.6.1.2.15.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an LoggedMeasurementConfiguration message with earlyMeasIndication set to ‘true’. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCRelease message with measIdleConfig including measIdleCarrierListNR with ssb-MeasConfig to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 30 seconds for the UE to perform the logging at regular time intervals. | - | - | - | - |
| 4-11 | Steps 1 to 8 of the generic procedure in TS 38.508-1 [4], Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 12 | SS transmits an LoggedMeasurementConfiguration message without earlyMeasIndication. | <-- | LoggedMeasurementConfiguration | - | - |
| 13 | The SS sends an UEInformationRequest message including idleModeMeasurementReq and logMeasReportReq. | <-- | NR RRC: UEInformationRequest | - | - |
| 14 | Check: Does the UE send an UEInformationResponse message with measurement results on early measurement frequencies? | --> | NR RRC: UEInformationResponse | 1 | P |
| 15 | The SS transmits a RRCRelease message with measIdleConfig including measIdleCarrierListNR with ssb-MeasConfig to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 16 | Wait 30 seconds for the UE to perform the logging at regular time intervals. | - | - | - | - |
| 17-24 | Steps 1 to 8 of the generic procedure in TS 38.508-1 [4], Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 25 | The SS sends an UEInformationRequest message including idleModeMeasurementReq and logMeasReportReq. | <-- | NR RRC: UEInformationRequest | - | - |
| 26 | Check: Does the UE send an UEInformationResponse message without measurement results on early measurement frequencies? | --> | NR RRC: UEInformationResponse | 2 | P |

8.1.6.1.2.15.3.3	Specific message contents

Table 8.1.6.1.2.15.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.15.3.2-1)

| Derivation Path: TS 38.508-1 Table 4.6.1-5AA |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |  |
| reportType CHOICE { |  |  |  |  |
| periodical SEQUENCE { |  |  |  |  |
| loggingInterval-r16 |  | ms61440 | 61.44s |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| LoggedMeasurementConfiguration-v1700-IEs |  | LoggedMeasurementConfiguration-v1700-IEs with condition EarlyMeas |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.6.1.2.15.3.3-1a: RRCRelease (steps 2 and 15, Table 8.1.6.1.2.15.3.2-1)

| Derivation Path: TS 38.508-1 [4], clause 4.6.1 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| measIdleConfig-r16 SetupRelease { |  |  |  |
| MeasIdleConfigDedicated-r16 SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16 { |  | 1 entry |  |
| MeasIdleCarrierNR-r16[1] SEQUENCE { | entry 1 |  |  |
| carrierFreq-r16 | ARFCN value of configured NR cell 1 |  |  |
| ssbSubcarrierSpacing-r16 | kHz15 |  |  |
| frequencyBandList | Not present |  |  |
| measCellListNR-r16 | Not present |  |  |
| reportQuantities-r16 | both |  |  |
| qualityThreshold-r16 | Not present |  |  |
| ssb-MeasConfig-r16 SEQUENCE { |  |  |  |
| nrofSS-BlocksToAverage-r16 | 2 |  |  |
| absThreshSS-BlocksConsolidation-r16 SEQUENCE { |  |  |  |
| thresholdRSRP | RSRP-Range | TS 38.508-1 [4] Table 4.6.3-152 |  |
| thresholdRSRQ | Not present |  |  |
| thresholdSINR | Not present |  |  |
| } |  |  |  |
| smtc-r16 | SSB-MTC | TS 38.508-1 [4] Table 4.6.3-185 |  |
| ssb-ToMeasure-r16 | SSB-ToMeasure | TS 38.508-1 [4] Table 4.6.3-187 |  |
| ss-RSSI-Measurement-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdleCarrierListEUTRA-r16 | Not present |  |  |
| measIdleDuration-r16 | sec30 |  |  |
| validityAreaList-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.15.3.3-2: LoggedMeasurementConfiguration (step 12, Table 8.1.6.1.2.15.3.2-1)

| Derivation Path: TS 38.508-1 Table 4.6.1-5AA |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |  |
| reportType CHOICE { |  |  |  |  |
| periodical SEQUENCE { |  |  |  |  |
| loggingInterval-r16 |  | ms61440 | 61.44s |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| LoggedMeasurementConfiguration-v1700-IEs SEQUENCE { |  |  |  |  |
| earlyMeasIndication-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.6.1.2.15.3.3-3: UEInformationRequest (steps 13 and 25, Table 8.1.6.1.2.15.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-32A with condition IDLE and LOG

Table 8.1.6.1.2.15.3.3-4: UEInformationResponse (steps 14, Table 8.1.6.1.2.15.3.2-1)

| Derivation Path: TS 38.508-1 [4], clause 4.6.1 table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleNR-r16 SEQUENCE { |  |  |  |
| measResultServingCell-r16 SEQUENCE { |  |  |  |
| rsrp-Result-r16 | Any allowed value |  |  |
| rsrq-Result-r16 | Any allowed value |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index-r16 | Any allowed value |  |  |
| ssb-Results-r16 SEQUENCE { |  |  |  |
| ssb-RSRP-Result-r16 | Any allowed value |  |  |
| ssb-RSRQ-Result-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultsPerCarrierListIdleNR-r16 SEQUENCE (SIZE (1.. maxFreqIdle-r16)) OF MeasResultsPerCarrierIdleNR-r16 | 1 entry |  |  |
| MeasResultsPerCarrierIdleNR-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r16 | ARFCN value corresponding to NR cell 1 |  |  |
| measResultsPerCellListIdleNR-r16 SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF MeasResultsPerCellIdleNR-r16 { | 1 entry |  |  |
| MeasResultsPerCellIdleNR-r16[1] SEQUENCE { |  | entry 1 |  |
| physCellId-r16 | PhysCellId corresponding to NR cell 1 |  |  |
| measIdleResultNR-r16 SEQUENCE { |  |  |  |
| rsrp-Result-r16 | Any allowed value |  |  |
| rsrq-Result-r16 | Any allowed value |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index-r16 | Any allowed value |  |  |
| ssb-Results-r16 SEQUENCE { |  |  |  |
| ssb-RSRP-Result-r16 | Any allowed value |  |  |
| ssb-RSRQ-Result-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasReport-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.15.3.3-5: UEInformationResponse (step 26, Table 8.1.6.1.2.15.3.2-1)

| Derivation Path: TS 38.508-1 [4], clause 4.6.1 table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleNR-r16 | Not present |  |  |
| logMeasReport-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>