3GPP TSG-RAN5 Meeting #99	R5-233388

# Incheon, Korea, 22th May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3784 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MBS Multicast TC 14.2.5.1.2-Network-requested PDU session modification to update MBS service area |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MBS TC 14.2.5.1.2 needs to be added to test that UE update MBS service area according to new MBS service area in Received MBS container IE in PDU SESSION MODIFICATION COMMAND. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of MBS TC 14.2.5.1.2 MBS Multicast/ Session management / Network-requested PDU session modification / MBS service area update |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_MBS_5MBS_5MBUSA-UEConTest WP will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.5.1.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-2 CR 0356 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised by R5-232962r2 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.2.5.1.2	MBS Multicast/ Session management / Network-requested PDU session modification / MBS service area update

14.2.5.1.2.1	Test Purpose (TP)

(1)

with { UE is in 5GMM-REGISTERED state and UE is camping on a cell that is outside the received MBS service area }

ensure that {

when { UE receives PDU SESSION MODIFICATION COMMAND message including Received MBS container IE with MBS decision setting to "MBS service area update" for including the current cell into MBS servcie area }

then { UE sends PDU SESSION MODIFICATION COMPLETE and UE shall store the received MBS service area associated with the received TMGI and replace the current MBS service area with the received one }

}

(2)

with { UE is in 5GMM-REGISTERED state and UE updates MBS service area associated with TMGI based on received MBS service area in PDU SESSION MODIFICATION COMMAND message }

ensure that {

when { UE is camping on a cell that is inside the received MBS service area associated with one TMGI and join MBS session with this TMGI is still needed }

then { UE sends PDU SESSION MODIFICATION REQUEST message to join MBS session with this TMGI }

}

14.2.5.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clauses 6.4.1.3 and 6.3.2.3. Unless otherwise stated these are Rel-17 requirements.

[TS 24.501, clause 6.3.2.2]

If the SMF wants to update the MBS service area of an multicast MBS session that the UE has joined, the SMF shall include the corresponding multicast MBS session ID and the updated MBS service area in the Received MBS container IE in the PDU SESSION MODIFICATION COMMAND message, and shall set the MBS decision to "MBS service area update" in the Received MBS information.

[TS 24.501, clause 6.3.2.3]

If the PDU SESSION MODIFICATION COMMAND message includes the Received MBS container IE, for each of the Received MBS informations:

…

d)	if the MBS decision is set to "MBS service area update", the UE shall store the received MBS service area associated with the received TMGI and replace the current MBS service area with the received one. Or

…

14.2.5.1.2.3	Test description

14.2.5.1.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the Serving Cell.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

UE:

-	UE is made interested in receiving MBS Multicast service with MBS Service ID ‘000101’H.

Preamble:

-	The UE is in state 1N-A on NR Cell 1 (serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

14.2.5.1.2.3.2	Test procedure sequence

Table 14.2.5.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 to 1b12a1 describe behaviour that depends on the UE capability the “lower case letter” identifies a step sequence that take place. | - | - | - | - |
| 1a1-1a14 | IF pc_Join_MBS_by_PDU_Modification (UE firstly establishes a associated PDU session and then joins MBS Multicast by PDU seession modification procedure) THEN steps 1a1 to 1a14 of the generic procedures described in TS 38.508-1[4] subclause 4.9.34 are performed on NR Cell 1 to establish an associated PDU Session to the MBS DNN and request to join a MBS Multicast session with TMGI-1. | - | - | - | - |
| 1a15 | The SS transmits a PDU SESSION MODIFICATION COMMAND to reject MBS Multicast session UE requested to join. The reject cause is "User is outside of local MBS service area". | <-- | NR RRC: DLInformationTransfer 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| 1a16 | The UE transmits an ULInformationTransfer message and a PDU SESSION MODIFICATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | - |
| 1b1-1b12a1 | ELSE (UE establishes a associated PDU session and joins MBS Multicast at the same time) steps 1b1 to 1b12a1 of the generic procedures described in TS 38.508-1[4] subclause 4.9.34 are performed on NR Cell 1 to establish an associated PDU Session to the MBS DNN and request to join a MBS Multicast session with TMGI-1. MBS Multicast session is rejected with cause "User is outside of local MBS service area". | - | - | - | - |
| 2 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| 3-10 | Steps 1 to 8 of the procedure in TS 38.508-1[4] Table 4.5.4.2-3 to trigger UE move to RRC_CONNECTED mode with CN paging. | - | - | - | - |
| 11 | The SS transmits a PDU SESSION MODIFICATION COMMAND to update MBS service area. | <-- | NR RRC: DLInformationTransfer 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| 12 | The UE transmits a PDU SESSION MODIFICATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | 1 | P |
| 13 | Start Timer = 5 sec. | - | - | - | - |
| - | EXCEPTION: Steps 14a1-14b3 describe optional behaviour that depends on the UE implementation. | - | - | - | - |
| 14a1 | Check: Does the UE transmit a PDU SESSION MODIFICATION REQUEST message to join MBS Multicast session with TMGI-1? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION REQUEST | 2 | P |
| 14a2 | Stop Timer = 5 sec. | - | - | - | - |
| 14b1 | Timer =5 sec expires. | - | - | - | - |
| 14b2 | Cause the UE to join MBS session with TMGI-1. (NOTE 1) | - | - | - | - |
| 14b3 | Check: Does the UE transmit a PDU SESSION MODIFICATION REQUEST message to join MBS Multicast session with TMGI-1? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION REQUEST | 2 | P |
| 15 | The SS transmits an RRCReconfiguration message and a PDU SESSION MODIFICATION COMMAND to accept MBS Multicast session join request. | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| - | EXCEPTION: Depending upon UE implementation, step 16 and 17 can occur in any order | - | - | - | - |
| 16 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 17 | The UE transmits a PDU SESSION MODIFICATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | - |
| 18 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| 19-33 | Check: Does UE respond to paging with TMGI-1 and receive the MRB associated with TMGI-1 as specified in steps 1 to 15 of the procedure in TS 38.508-1[4] Table 4.9.38.2.2-1? | - | - | 2 | - |
| NOTE 1: This could be done by e.g. MMI or AT command. |  |  |  |  |  |

14.2.5.1.2.3.3	Specific message contents

Table 14.2.5.1.2.3.3-1: ACTIVATE TEST MODE (preamble, Table 14.2.5.1.2.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.2.5.1.2.3.3-2: PDU SESSION MODIFICATION REQUEST (step 1a14, step 14a1 and 14b3, Table 14.2.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Requested MBS container |  |  |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MNC for NR Cell 1 |  |

Table 14.2.5.1.2.3.3-3:  PDU SESSION MODIFICATION COMMAND (step 1a15, Table 14.2.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Received MBS container |  |  |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘100’B | User is outside of local MBS service area |  |
| MSAI |  | ‘01’B | MBS service area included as MBS TAI list |  |
| MD |  | ‘011’B | MBS join is rejected |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MNC for NR Cell 1 |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  |  |  |  |
| Length of 5GS tracking area identity list contents |  |  |  |  |
| Partial tracking area identity list 1 |  |  |  |  |
| Number of elements |  | '0 0000'B | 1 element |  |
| Type of list |  | '00'B | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 11 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MNC for NR Cell 11 |  |
| TAC 1 |  | See 38.508-1[4] table 4.4.2-3 | TAC for NR Cell 11 |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |

Table 14.2.5.1.2.3.3-4: PDU SESSION ESTABLISHMENT REQUEST (step 1b9, Table 14.2.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Requested MBS container |  |  |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |

Table 14.2.5.1.2.3.3-5: PDU SESSION ESTABLISHMENT ACCEPT (step 1b10, Table 14.2.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Received MBS container |  |  |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘100’B | User is outside of local MBS service area |  |
| MSAI |  | ‘01’B | MBS service area included as MBS TAI list |  |
| MD |  | ‘011’B | MBS join is rejected |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  |  |  |  |
| Length of 5GS tracking area identity list contents |  |  |  |  |
| Partial tracking area identity list 1 |  |  |  |  |
| Number of elements |  | '0 0000'B | 1 element |  |
| Type of list |  | '00'B | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 11 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MNC for NR Cell 11 |  |
| TAC 1 |  | See 38.508-1[4] table 4.4.2-3 | TAC for NR Cell 11 |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |

Table 14.2.5.1.2.3.3-6: RRCReconfiguration (step 1b10, Table 14.2.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition DRBn | n is chosen as the next available number higher or equal to 2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition DRBn | n is set to the same value as for the radioBearerConfig IE |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.5.1.2.3.3-7:  PDU SESSION MODIFICATION COMMAND (step 11, Table 14.2.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Received MBS container |  |  |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘01’B | MBS service area included as MBS TAI list |  |
| MD |  | ‘001’B | MBS service area update |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  |  |  |  |
| Length of 5GS tracking area identity list contents |  |  |  |  |
| Partial tracking area identity list 1 |  |  |  |  |
| Number of elements |  | '0 0000'B | 1 element |  |
| Type of list |  | '00'B | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 | MCC for NR Cell 1 |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 | MNC for NR Cell 1 |  |
| TAC 1 |  | See 38.508-1[4] table 4.4.2-3 | TAC for NR Cell 1 |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |

Table 14.2.5.1.2.3.3-8: RRCReconfiguration (step 15, Table 14.2.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition MRBm and UM_PTM | m=1 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition MRBm and UM_PTM | m=1 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.5.1.2.3.3-9:  PDU SESSION MODIFICATION COMMAND (step 15, Table 14.2.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Received MBS container |  |  |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘010’B | MBS join is accepted |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  |  |

<End of modified section 1>