3GPP TSG-RAN5 Meeting #99	R5-233392

# Incheon, Korea, 22th May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3787 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of UPIP TC 8.2.6.4.3-HO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPIP_SEC_LTE-RAN-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New test case is added to check EN-DC capable UE connected to EUTRA/EPC could perform UPIP(user plane integrity protection) correctly after handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of UPIP TC 8.2.6.4.3 UPIP / RRC connection reconfiguration / Handover / Success / Reception of RRCConnctionReconfiguration including mobilityControlInfo by UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UPIP_SEC_LTE-RAN-UEConTest will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6.4.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0357 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised by R5-232967r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.2.6.4.3	UPIP / RRC connection reconfiguration / Handover / Success / Reception of RRCConnctionReconfiguration including mobilityControlInfo by UE

8.2.6.4.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and UE handover from a legacy source eNB (non-supporting EPS-UPIP) to an upgraded target eNB (supporting EPS-UPIP) and UP integrity is de-activated for one DRB }

ensure that {
  when { Target eNB update the UE's UP integrity protection policy with the received UE's UP integrity protection policy from MME and target eNB active the UP integrity for this DRB by transmiting RRCConnectionReconfiguration }

then { UE replies RRCConnectionReconfigurationComplete and starts to activate the UP integrity for this DRB }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and UP integrity has been activated for one DRB }

ensure that {
  when { UE handover from a upgraded source eNB to an upgraded target eNB }

then { UE replies RRCConnectionReconfigurationComplete and keeps on activating the UP integrity for this DRB }

}

8.2.6.4.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.331 clause 5.3.5.4; TS 38.331 clause 5.3.5.6.5; TS 33.401 clause 7.3.3. Unless otherwise stated these are Rel-17 requirements.

[TS 36.331, clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if daps-HO is configured for any DRB:

…

1>	else (if daps-HO is not configured):

2>	reset MCG MAC and SCG MAC, if configured;

2>	release uplinkDataCompression, if configured;

2>	re-establish PDCP for all RBs configured with pdcp-config that are established;

NOTE 2:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

NOTE 2a:	At handover the reestablishPDCP flag will be set for all RBs configured with NR PDCP in nr-RadioBearerConfig1 or nr-RadioBearerConfig2 TS 38.331 [82] which will cause the PDCP entity to be re-established also for these RBs.

2>	re-establish MCG RLC and SCG RLC, if configured, for all RBs that are established;

…

1>	if the securityConfigHO (without suffix) is included in the RRCConnectionReconfiguration:

2>	if the keyChangeIndicator received in the securityConfigHO is set to TRUE:

3>	update the KeNB key based on the KASME key taken into use with the latest successful NAS SMC procedure, as specified in TS 33.401 [32];

2>	else:

3>	update the KeNB key based on the current KeNB or the NH, using the nextHopChainingCount value indicated in the securityConfigHO, as specified in TS 33.401 [32];

NOTE 2b:	If the UE needs to update the S-KeNB key as specified in 5.3.10.10, the UE updates the S-KeNB after updating the KeNB key.

2>	store the nextHopChainingCount value;

2>	if the securityAlgorithmConfig is included in the securityConfigHO:

3>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

3>	if connected as an RN; or

3>	if capable of user plane integrity protection:

4>	derive the KUPint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

3>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

2>	else:

3>	derive the KRRCint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

3>	if connected as an RN; or

3>	if capable of user plane integrity protection:

4>	derive the KUPint key associated with the current integrity algorithm, as specified in TS 33.401 [32];

3>	derive the KRRCenc key and the KUPenc key associated with the current ciphering algorithm, as specified in TS 33.401 [32];

2>	configure lower layers to apply the integrity protection algorithm and the KRRCint key, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

2>	configure lower layers to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

[TS 38.331, clause 5.3.5.6.5]

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration and not configured as DAPS bearer:

2>	if the reestablishPDCP is set:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

…

3>	else (i.e., UE connected to NR or UE connected to E-UTRA/EPC (in EN-DC or capable of EN-DC)):

…

4>	if the PDCP entity of this DRB is configured with integrityProtection:

5>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KeNB/KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

…

NOTE 5: Ciphering and integrity protection can be enabled or disabled for a DRB. The enabling/disabling of ciphering or integrity protection can be changed only by releasing and adding the DRB.

[TS 33.401, clause 7.3.3]

At an X2-handover from the source eNB to the target eNB, the source eNB shall include in the HANDOVER REQUEST message, the UP integrity protection policy, the UE EPS security capability and the corresponding E-RAB ID, if the UP integrity protection policy is received from other entities. If the target eNB does not receive the UP integrity protection policy, but the EIA7 in the EPS security capability indicates that the UE supports user plane integrity protection with EPC, the target eNB shall use its locally configured UP integrity protection policy to activate or deactivate the UP integrity protection for all DRBs belonging to the E-RAB.

If the received UP integrity protection policy is 'Required', the target eNB shall reject all E-RABs for which it cannot comply with the corresponding UP integrity protection policy and indicate the reject-cause to the MME. For the accepted E-RABs, the target eNB shall activate UP integrity protection per DRB according to the UP integrity protection policy and shall indicate that to the UE in the HANDOVER COMMAND by the source eNB.

If the UE receives an indication in the HANDOVER COMMAND that UP integrity protection for an E-RAB is enabled at the target eNB, the UE shall generate or update the UP integrity protection key and shall activate UP integrity protection for the respective E-RAB.

NOTE 3:	If the UP integrity protection policy is 'Preferred', it is possible to have a change in activation or deactivation of UP integrity after the handover.

Further, in the Path-Switch message, the target eNB shall send the UE's UP integrity protection policy and corresponding E-RAB ID to the MME. The sent UP integrity protection policy can either be the one received from source eNB or the locally configured one if the target eNB does not receive it from the source eNB, but the EIA7 in the EPS security capability indicates that the UE supports user plane integrity protection with EPC. If the MME receives UP integrity protection policy, the MME shall verify that the UP integrity protection policy received from the target eNB is the same as the UP integrity protection policy that the MME has locally stored. If there is a mismatch, the MME shall send its locally stored UE's UP integrity protection policy of the corresponding E-RABs to the target eNB. This UP integrity protection policy, if included by the MME, is delivered to the target eNB in the Path-Switch Acknowledge message. The MME may support logging capabilities for this event and may take additional measures, such as raising an alarm.

NOTE 4:	An upgraded target eNB may not receive UE's UP integrity protection policy from a legacy source eNB, thus, mismatch of UP integrity protection policy may not be regarded as an abnormal case. The upgraded target eNB can get UE's UP integrity protection policy from the MME.

If the target eNB receives UE's UP integrity protection policy from the MME in the Path-Switch Acknowledge message, the target eNB shall update the UE's UP integrity protection policy with the received UE's UP integrity protection policy. If UE's current UP integrity protection activation is different from the determination of received UE's UP integrity protection policy, then the target eNB shall initiate intra-cell handover procedure which includes RRC Connection Reconfiguration procedure to reconfigure the DRBs to activate or de-activate the UP integrity as per the received policy from MME.

8.2.6.4.3.3	Test description

8.2.6.4.3.3.1	Pre-test conditions

System Simulator:

-	E-UTRA Cell 1, E-UTRA Cell 4 and E-UTRA Cell 11.

UE:

-	None

Preamble:

-	UE is in state 3E-A EUTRA RRC_CONNECTED using generic procedure parameter Connectivity (E-UTRA/EPC) and Test Mode (On) and Test Loop Function (Off) associated with UE test loop mode B configured on E-UTRA Cell 1 according to TS 38.508-1 [4], clause 4.5.4.

8.2.6.4.3.3.2	Test procedure sequence

Table 8.2.6.4.3.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.6.4.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | E-UTRA Cell 1 | E-UTRA Cell 4 | E-UTRA Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | ”Off” | The power level values are such that measurement results for E-UTRA Cell 1 (M1) and E-UTRA Cell 4 (M4) satisfy exit condition for event A3 (M4 < M1)(NOTE 1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | ”Off” | The power level values are such that measurement results for E-UTRA Cell 1 (M1) and E-UTRA Cell 4 (M4) satisfy entry condition for event A3 (M4 > M1)(NOTE 1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -85 | -79 | The power level value are such that measurement results for E-UTRA Cell 4 (M1) and E-UTRA Cell 11 (M11) satisfy entry condition for event A3 (M11 > M1). |
| NOTE 1: Power level “Off” is defined in TS 36.508 [7] Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.2.6.4.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup intra frequency measurement on E-UTRA Cell 1. | <-- | RRC: RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on E-UTRA Cell 1. | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes E-UTRA Cell 1, E-UTRA Cell 4 parameters according to the row "T1" in table 8.2.6.4.3.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message to report event A3 on E-UTRA Cell 1 with the measured RSRP, RSRQ value for E-UTRA Cell 4. | --> | RRC: MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to E-UTRA Cell 4. | <-- | RRC: RRCConnectionReconfiguration | - | - |
| 6 | The UE transmits an RRCConnectionReconfigurationComplete message on E-UTRA Cell 4. | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message to enable user plane integrity protection by releasing and adding the DRB on E-UTRA Cell 4. | <-- | RRC: RRCConnectionReconfiguration | - | - |
| 8 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on E-UTRA Cell 4? | --> | RRC: RRCConnectionReconfigurationComplete | 1 | P |
| 9-10 | Steps 1 to 2 of Loopback activation procedure in TS 36.508 [7] Table 4.5.4.3-1 are executed to activate test loop mode B on the DRB configured in step 7. | - | - | - | - |
| 11 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n configured in step7 and uplink IP data is correctly integrity protected? | - | - | 1 | - |
| 12 | The SS changes E-UTRA Cell 1, E-UTRA Cell 4 and E-UTRA Cell 11 parameters according to the row "T2" in table 8.2.6.4.3.3.2-1. | - | - | - | - |
| 13 | The UE transmits a MeasurementReport message to report event A3 on E-UTRA Cell 1 with the measured RSRP, RSRQ value for E-UTRA Cell 11. | --> | RRC: MeasurementReport | - | - |
| 14 | The SS transmits an RRCConnectionReconfiguration message to order the UE to perform intra frequency handover to E-UTRA Cell 11. | <-- | RRC: RRCConnectionReconfiguration | - | - |
| 15 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on E-UTRA Cell 11? | --> | RRC: RRCConnectionReconfigurationComplete | 2 | P |
| 16 | The UE transmit a TRACKING AREA UPDATE REQUEST | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE REQUEST | - | - |
| 17 | SS responds with a TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 18 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE | - | - |
| 19 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n configured in step7 and uplink IP data is correctly integrity protected? | - | - | 2 | - |

8.2.6.4.3.3.3	Specific message contents

Table 8.2.6.4.3.3.3-1: SecurityModeCommand message (preamble, Table 8.2.6.4.3.3.2-2)

| Derivation Path: TS 36.508 [7] Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SecurityModeCommand ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| securityModeCommand-r8 SEQUENCE { |  |  |  |  |
| securityConfigSMC SEQUENCE { |  |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |  |
| cipheringAlgorithm |  | Set according to PIXIT parameter for default ciphering algorithm |  |  |
| integrityProtAlgorithm |  | Set according to PIXIT parameter for default integrity protection algorithm | null algorithm is not allowed |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.6.4.3.3.3-2: RRCConnectionReconfiguration (step 1, Table 8.2.6.4.3.3.2-2)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8, condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| measConfig |  | MeasConfig | Table 8.2.6.4.3.3.3-3 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.6.4.3.3.3-3: MeasConfig (Table 8.2.6.4.3.3.3-2)

| Derivation Path: TS 36.508 [7], Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.2.6.4.3.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.2.6.4.3.3.2-2)

Derivation Path: TS 36.508 [7], Table 4.6.1-8, condition HO

Table 8.2.6.4.3.3.3-5: RRCConnectionReconfiguration (step 7, Table 8.2.6.4.3.3.2-2)

| Derivation Path: 36.508 [7], Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-MCG-DRB-NR-PDCP | Table 8.2.6.4.3.3.3-6 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-Config-r15 | Not present |  |  |
| nr-RadioBearerConfig1-r15 | RadioBearerConfig-MCG-DRB-NR-PDCP | Table 8.2.6.4.3.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.4.3.3.3-6: RadioResourceConfigDedicated-MCG-DRB-NR-PDCP (Table 8.2.6.4.3.3.3-5)

| Derivation Path: TS 36.508 [7], Table 4.6.3-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-MCG-DRB-NR-PDCP ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] | DRB-ToAddMod-MCG-DRB-NR-PDCP | entry 1 Table 8.2.6.4.3.3.3-7 |  |
| } |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { | 1 entry |  |  |
| drb-Identity[1] | Same as the DRB identity associated with the default EPS bearer | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.4.3.3.3-7: DRB-ToAddMod-MCG-DRB-NR-PDCP (Table 8.2.6.4.3.3.3-6)

| Derivation Path: TS 36.508 [7], Table 4.8.2.1.7-1, condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DRB-ToAddMod ::= SEQUENCE { |  |  |  |  |
| eps-BearerIdentity |  | Same as the default EPS bearer Identity |  |  |
| drb-Identity |  | Same as the DRB identity associated with the default EPS bearer |  |  |
| pdcp-Config |  | Not present |  |  |
| } |  |  |  |  |

Table 8.2.6.4.3.3.3-8: RadioBearerConfig-MCG-DRB-NR-PDCP (Table 8.2.6.4.3.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132, condition MCG_NR_PDCP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | Same as the default EPS bearer Identity |  |  |
| } |  |  |  |
| drb-Identity | Same as the DRB identity associated with the default EPS bearer |  |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config | Table 8.2.6.4.3.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |
| securityConfig SEQUENCE { |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |
| cipheringAlgorithm | Same as the ciphering algorithm configured in Table 8.2.6.4.3.3.3-1. (NOTE) |  |  |
| integrityProtAlgorithm | Same as the integrity algorithm configured in Table 8.2.6.4.3.3.3-1. (NOTE) |  |  |
| } |  |  |  |
| keyToUse | master |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE: For example, if eia1 (SNOW3G) is configured in Table 8.2.6.4.3.3.3-1, the nia1 (SNOW3G) is configured in this table. etc. |  |  |  |

Table 8.2.6.4.3.3.3-9: PDCP-Config (Table 8.2.6.4.3.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| integrityProtection | true |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.4.3.3.3-10: RRCConnectionReconfiguration (step 14, Table 8.2.6.4.3.3.2-2)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8, condition HO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| securityConfigHO | SecurityConfigHO | Table 8.2.6.4.3.3.3-12 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-Config-r15 | Not present |  |  |
| nr-RadioBearerConfig1-r15 | RadioBearerConfig-NR-PDCP-Reestablish | Table 8.2.6.4.3.3.3-11 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.4.3.3.3-11: RadioBearerConfig-NR-PDCP-Reestablish (Table 8.2.6.4.3.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | Same as the default EPS bearer Identity |  |  |
| } |  |  |  |
| drb-Identity | Same as the DRB identity associated with the default EPS bearer |  |  |
| reestablishPDCP | true |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| securityConfig | Not present |  |  |
| } |  |  |  |

Table 8.2.6.4.3.3.3-12: SecurityConfigHO (Table 8.2.6.4.3.3.3-10)

| Derivation Path: TS 36.508 [7], Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| intraLTE SEQUENCE { |  |  |  |
| nextHopChainingCount | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>