3GPP TSG-RAN WG5 Meeting #99	R5-233395

Incheon, South Korea, 22nd – 26th May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0173 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of R17 Positioning Enhancements default test conditions in TS 37.571-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN#97-e, WI of UE Conformance Test Aspects - NR positioning enhancements has been approved, related contents should be introduced into TS 37.571-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Added Release-17 NR positioning methods associated abbreviations 2. Added sub-test definitions for Release-17 positioning methods in Table 8.1-1 3. Added simulated UL-TDOA environment in the default test condition in clause 8.2 4. Corrected SRS-PosResource for TDD condition 5. Updated default LPP messages and information elements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test spec will not applicable to Release-17 NR positioning methods signal test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3, 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 23.271 [2],

TS 36.305 [3], TS 36.355 [4] , TS 38.305 [31] , TS 37.355 [32] apply.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BDS	BeiDou Navigation Satellite System

DL	Downlink

DL-AoD	Downlink Angle-of-Departure

DL-PRS	Downlink Positioning Reference Signal

DL-TDOA	Downlink Time Difference Of Arrival

LCS	Location Services

LPP	LTE Positioning Protocol

MBS	Metropolitan Beacon System

MO-LR	Mobile Originated Location Request

Multi-RTT	Multi-Round Trip Time

MT-LR	Mobile Terminated Location Request

NAS	Non-Access-Stratum

NI-LR	Network Induced Location Request

NR	New Radio

NR E-CID	NR Enhanced Cell ID (positioning method)

OTA	Over The Air

RSRP	Reference Signal Received Power

RSRPP	Reference Signal Received Path Power

TBS	Terrestrial Beacon System

TEG	Timing Error Group

UL	Uplink

WLAN	Wireless Local Area Network

UL-SRS	Uplink Sounding Reference Signal

< Unchanged sections omitted >

8	Default Conditions for NR

8.1	LCS Sub-Test Cases

Some test cases defined in clause 9 may include several sub-test cases dependent on the positioning method(s) supported by the UE. Each sub-test case is identified by a sub-test case number as defined in Table 8.1-1. The applicable sub-tests for each test case are specified in the test procedure sequence clause of each test case. If no sub-tests are defined for a specific test case it means that this particular test case is not dependent on a specific positioning method and is applicable in all cases, independently of the positioning method(s) supported by the UE.

Table 8.1-1: Sub-Test Case Numbers for NR

| Sub-Test Case Number | Supported Positioning Methods |
| --- | --- |
| 1 | Void |
| 2 | Void |
| 3 | Void |
| 4 | Void |
| 5 | UE supporting OTDOA (LTE) |
| 6 | UE supporting ECID (LTE) |
| 7 | UE supporting GNSS(1) and OTDOA (LTE) |
| 8 | Void |
| 9 | Void |
| 10 | Void |
| 11 | UE supporting WLAN (Rel-13 only) |
| 12 | UE supporting MBS(2) (Rel-13 only) |
| 13 | UE supporting Bluetooth |
| 14 | UE supporting Sensor (Rel-13 only) |
| 15 | UE supporting GNSS(1) |
| 16 | UE supporting MBS(2) (Rel-14 onwards) |
| 17 | UE supporting WLAN (Rel-14 onwards) |
| 18 | UE supporting Sensor (Rel-14 onwards) |
| 19 | UE supporting Multi-RTT (Rel-16 onwards) |
| 20 | UE supporting DL-AoD (Rel-16 onwards) |
| 21 | UE supporting DL-TDOA (Rel-16 onwards) |
| 22 | UE supporting NR E-CID (Rel-16 onwards) |
| 23 | UE supporting MBS(2) (Rel-16 onwards) |
| 24 | UE supporting Sensor (Rel-16 onwards) |
| 25 | UE supporting GNSS(1) (Rel-15 onwards) |
| 26 | UE supporting Multi-RTT (Rel-17 onwards) |
| 27 | UE supporting DL-AoD (Rel-17 onwards) |
| 28 | UE supporting DL-TDOA (Rel-17 onwards) |
| 29 | UE supporting UL-TDOA (Rel-17 onwards) |
| NOTE 1: The GNSS combination of BDS, Galileo, GLONASS, GPS supported by the UE NOTE 2: Metropolitan Beacon System (MBS) is a specific type of Terrestrial Beacon System (TBS) [29] |  |

8.1A	Test Configurations

For ease of use of this document a number of Test Configurations corresponding to Network Deployment Types are defined in Table 8.1A-1.

Table 8.1A-1: Test Configuration

| Test Configuration | Network Deployment Type |
| --- | --- |
| A | EN-DC |
| B | NG-RAN NR |
| C | NE-DC |
| D | NG-RAN E-UTRA |
| E | NGEN-DC |

8.2	Default signal conditions

8.2.1	Simulated GNSS environment

Same as defined in clause 5.2.1.

For Test Configuration B (Table 8.1A-1) the NR Cell 1 frequency to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

For Test Configuration D (Table 8.1A-1) the LTE Cell 1 frequency to be tested and other default conditions are as specified for signalling test cases in 3GPP TS 36.508 [8].

8.2.2	Simulated OTDOA (LTE) environment

For Test Configuration B (Table 8.1A-1) the NR Cell 1 frequency to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

For Test Configuration B (Table 8.1A-1) an additional independent multi cell LTE environment is used with LTE Cell 1 and LTE Cell 2 (where required) . The E-UTRA frequency to be tested and other default conditions are as specified for signalling test cases in 3GPP TS 36.508 [8].

For Test Configuration D (Table 8.1A-1), the LTE Cell 1 and LTE Cell 2 (where required) E-UTRA frequency to be tested and other default conditions are as specified for signalling test cases in 3GPP TS 36.508 [8].

All LTE cells transmit PRS according to the PRS configuration provided in the OTDOA (LTE) assistance data defined in subclause 8.4.1.2. The positioning subframes are low-interference subframes, i.e. contain no PDSCH transmissions.

LTE Cell 1 is the OTDOA reference cell, LTE Cell 2 (where required) is an OTDOA neighbour cell.

Where two LTE cells are required, the two LTE Cells 1 and 2 shall be synchronized, and the timing offset (the RSTD) between the cells, referenced to the UE’s antenna input, shall be set equal to the expectedRSTD value provided in the OTDOA (LTE) assistance data, as defined in subclause 8.4.1.2.

Normal propagation condition is used for all cells.

8.2.3	Simulated ECID (LTE) environment

Same as defined in clause 5.2.3.

For Test Configuration B (Table 8.1A-1) up to and including LPP Rel-15, UE Rx-Tx measurement is not possible and therefore there is little value in testing ECID (LTE) for positioning purposes and ECID (LTE) shall not be tested.

For Test Configuration D (Table 8.1A-1), the LTE Cell 1 and LTE Cell 2 E-UTRA frequency to be tested and other default conditions are as specified for signalling test cases in 3GPP TS 36.508 [8].

8.2.4	Simulated MBS environment

Same as defined in clause 5.2.4.

For Test Configuration B (Table 8.1A-1) the NR Cell 1 frequency to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

For Test Configuration D (Table 8.1A-1) the LTE Cell 1 frequency to be tested and other default conditions are as specified for signalling test cases in 3GPP TS 36.508 [8].

8.2.5	Simulated WLAN environment

Same as defined in clause 5.2.5.

For Test Configuration B (Table 8.1A-1) the NR Cell 1 frequency to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

For Test Configuration D (Table 8.1A-1) the LTE Cell 1 frequency to be tested and other default conditions are as specified for signalling test cases in 3GPP TS 36.508 [8].

8.2.6	Simulated Bluetooth environment

Same as defined in clause 5.2.6.

For Test Configuration B (Table 8.1A-1) the NR Cell 1 frequency to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

For Test Configuration D (Table 8.1A-1) the LTE Cell 1 frequency to be tested and other default conditions are as specified for signalling test cases in 3GPP TS 36.508 [8].

8.2.7	Simulated Sensor environment

Same as defined in clause 5.2.7.

For Test Configuration B (Table 8.1A-1) the NR Cell 1 frequency to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

For Test Configuration D (Table 8.1A-1) the LTE Cell 1 frequency to be tested and other default conditions are as specified for signalling test cases in 3GPP TS 36.508 [8].

8.2.8	Simulated general NR environment

For NR FR2, the connection between the SS and the DUT shall be OTA. The SS shall ensure that the NR cell is suitable throughout the test. The OTA link shall be sufficient to provide stable LPP message transmissions between the SS and the DUT.

For NR FR1 and the other technologies used for the test cases in Chapter 9 (e.g. LTE, GNSS, WLAN…) the connection shall be conducted, following the settings described in this section.

8.2.9	Simulated Multi-RTT environment

For Multi-RTT signalling test cases NR Cell 1 is used, as defined in 3GPP TS 38.508-1 [30].

NR cell transmits DL-PRS according to the DL-PRS configuration provided in the Multi-RTT assistance data defined in subclause 8.4.1.6. The UE transmits UL-SRS according to the UL-SRS configuration provided in the RRC message as defined in subclause 8.3.2.

Normal propagation condition is used for all cells.

The simulated general NR environment as specified in 8.2.8 also applies.

Multi-RTT tests are only applicable for Test Configuration B (Table 8.1A-1). The NR Cell 1 frequency to be used for testing and other default conditions is as specified for signalling test cases in 3GPP TS 38.508-1 [30].

8.2.10	Simulated DL-AoD environment

For DL-AoD signalling test cases NR Cell 1 and NR Cell 2 (if required) are used, as defined in 3GPP TS 38.508-1 [30].

All NR cells transmit DL-PRS according to the DL-PRS configuration provided in the DL-AoD assistance data defined in subclause 8.4.1.7.

Normal propagation condition is used for all cells.

The simulated general NR environment as specified in 8.2.8 also applies.

DL-AoD tests are only applicable for Test Configuration B (Table 8.1A-1). The NR Cell 1 and NR Cell 2 frequencies to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

8.2.11	Simulated DL-TDOA environment

For DL-TDOA signalling test cases a multi cell environment with NR Cell 1, NR Cell 2 and NR Cell 3 (if required) are used, as defined in 3GPP TS 38.508-1 [30].

All NR cells transmit DL-PRS according to the DL-PRS configuration provided in the DL-TDOA assistance data defined in subclause 8.4.1.8.

Normal propagation condition is used for all cells.

NR Cell 1 is the DL-TDOA reference cell. NR Cell 2 and NR Cell 3 are DL-TDOA neighbour cells.

The simulated general NR environment as specified in 8.2.8 also applies.

DL-TDOA tests are only applicable for Test Configuration B (Table 8.1A-1). The NR Cell 1, NR Cell 2 and NR Cell 3 frequencies to be used for testing and other default conditions are as specified for signalling test cases in 3GPP TS 38.508-1 [30].

8.2.12	Simulated NR E-CID environment

For NR E-CID signalling test cases NR Cell 1 is used, as defined in 3GPP TS 38.508-1 [30].

The simulated general NR environment as specified in 8.2.8 also applies.

NR E-CID tests are only applicable for Test Configuration B (Table 8.1A-1). The NR Cell 1 frequency to be used for testing and other default conditions is as specified for signalling test cases in 3GPP TS 38.508-1 [30].

8.2.13	Simulated UL-TDOA environment

For UL-TDOA signalling test cases NR Cell 1 is used, as defined in 3GPP TS 38.508-1 [30].

The simulated general NR environment as specified in 8.2.8 also applies.

UL-TDOA test is only applicable for Test Configuration B (Table 8.1A-1). The NR Cell 1 frequency to be used for testing and other default conditions is as specified for signalling test cases in 3GPP TS 38.508-1 [30].

8.3	Default RRC and NAS message and information elements contents

The default values of common RRC and NAS messages and information elements are used as defined in 3GPP
TS 38.508-1 [30] with the following exceptions:

Table 8.3-1: Void

Table 8.3-2: Void

### -	REGISTRATION ACCEPT

Table 8.3-3: REGISTRATION ACCEPT

| Derivation Path: 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS network feature support |  | Set according to Table 8.3-4 |  |  |

Table 8.3-4: 5GS network feature support

| Derivation Path: 24.501 clause 9.11.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in 38.508-1 Table 4.7.1-7 with the following exceptions: |  |  |  |  |
| Emergency service support indicator for 3GPP access (EMC) (octet 3, bit 3 and bit 4) |  | 01 | Emergency services supported in NR connected to 5GCN only |  |

8.3.1	RRC message contents for measurement gaps

Table 8.3.1-1: RRCReconfiguration for meas gaps

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
|  |  | Not present |  |  |
| measConfig |  | MeasConfig | Table 8.3.1-2 |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.3.1-2: MeasConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList | MeasObjectToAddModList | Table 8.3.1-4 |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList | Not present |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
| s-MeasureConfig | Not present |  |  |
| quantityConfig | Not present |  |  |
| measGapConfig | MeasGapConfig | Table 8.3.1-3 |  |
| measGapSharingConfig | Not present |  |  |
| } |  |  |  |

Table 8.3.1-3: MeasGapConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapFR2 | Not present |  |  |
| gapFR1 | Not present |  |  |
| gapUE CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset | 10 |  |  |
| mgl | ms3 |  |  |
| mgrp | ms160 |  |  |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.1-4: MeasObjectToAddModList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectToAddModList ::=SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 8.3.1-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.1-5: MeasObjectNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::=SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for NR Cell 1 |  |  |
| ssbSubcarrierSpacing | Subcarrier spacing of SSB for NR Cell 1 |  |  |
| smtc1 | Not present |  |  |
| smtc2 | Not present |  |  |
| refFreqCSI-RS | Not present |  |  |
| referenceSignalConfig SEQUENCE { |  |  |  |
| ssb-ConfigMobility | Not present |  |  |
| csi-rs-ResourceConfigMobility | Not present |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| absThreshCSI-RS-Consolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| nrofCSI-RS-ResourcesToAverage | Not present |  |  |
| quantityConfigIndex | 1 |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB0 |  |  |
| rsrqOffsetSSB | dB0 |  |  |
| sinrOffsetSSB | dB0 |  |  |
| rsrpOffsetCSI-RS | dB0 |  |  |
| rsrqOffsetCSI-RS | dB0 |  |  |
| sinrOffsetCSI-RS | dB0 |  |  |
| } |  |  |  |
| cellsToRemoveList | Not present |  |  |
| cellsToAddModList | Not present |  |  |
| excludedCellsToRemoveList | Not present |  |  |
| excludedCellsToAddModList | Not present |  |  |
| allowedCellsToRemoveList | Not present |  |  |
| allowedCellsToAddModList | Not present |  |  |
| freqBandIndicatorNR | Not present |  |  |
| measCycleSCell | Not present |  |  |
| smtc3list-r16 | Not present |  |  |
| rmtc-Config-r16 | Not present |  |  |
| t312-r16 | Not present |  |  |
| associatedMeasGapSSB-r17 | Not present |  |  |
| associatedMeasGapCSIRS-r17 | Not present |  |  |
| smtc4list-r17 | Not present |  |  |
| measCyclePSCell-r17 | Not present |  |  |
| cellsToAddModListExt-v1710 | Not present |  |  |
| associatedMeasGapSSB2-v1720 | Not present |  |  |
| associatedMeasGapCSIRS2-v1720 | Not present |  |  |
| } |  |  |  |

8.3.2	RRC message contents for UL-SRS Configuration

Table 8.3.2-1: RRCReconfiguration

| Derivation Path: TS 38.508-1, table 4.6.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.3.2-2: CellGroupConfig

| Derivation Path: 38.508-1 [30], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| srs-Config CHOICE { |  |  |  |
| setup | srs-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.2-3: SRS-Config

| Derivation Path: TS 38.331, clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-PosResourceSetToReleaseList-r16 | Not present |  |  |
| srs-PosResourceSetToAddModList-r16 SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16 { | 1 entry |  |  |
| SRS-PosResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| srs-PosResourceSetId-r16 | 0 |  |  |
| srs-PosResourceIdList-r16 SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-PosResourceId-r16 { | 1 entry |  |  |
| SRS-PosResourceId-r16[1] | 0 | entry 1 |  |
| } |  |  |  |
| resourceType-r16 CHOICE { |  |  |  |
| periodic-r16 SEQUENCE { |  |  |  |
| } |  |  |  |
| alpha-r16 | Not present |  |  |
| p0-r16 | -100 |  |  |
| pathlossReferenceRS-Pos-r16 CHOICE { |  |  |  |
| ssb-IndexServing-r16 | 1 | Set according to Table 4.4.2-2 in TS 38.508-1. |  |
| } |  |  |  |
| } |  |  |  |
| srs-PosResourceToReleaseList-r16 | Not present |  |  |
| srs-PosResourceToAddModList-r16 SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16 { | 1 entry |  |  |
| SRS-PosResource-r16[1] | SRS-PosResource-r16 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.2-4: SRS-PosResource-r16

| Derivation Path: TS 38.331, clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-PosResource-r16::=SEQUENCE { |  |  |  |
| srs-PosResourceId-r16 | 0 |  |  |
| transmissionComb-r16 CHOICE { |  |  |  |
| n4-r16 SEQUENCE { |  |  |  |
| combOffset-n4-r16 | 0 |  |  |
| cyclicShift-n4-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceMapping-r16 SEQUENCE { |  |  |  |
| startPosition-r16 | 1 |  |  |
| nrofSymbols-r16 | n2 |  |  |
| } |  |  |  |
| freqDomainShift-r16 | 0 |  |  |
| freqHopping-r16 SEQUENCE { |  |  |  |
| c-SRS-r16 | 0 |  |  |
| } |  |  |  |
| groupOrSequenceHopping-r16 | neither |  |  |
| resourceType-r16 CHOICE { |  |  |  |
| periodic-r16 SEQUENCE { |  |  |  |
| periodicityAndOffset-p-r16 CHOICE { |  |  |  |
| sl160 | 20 |  | FR1 |
|  | 23 |  | FR1 AND (HD_FDD OR TDD) AND SCS15Khz |
|  | 27 |  | FR1 AND TDD AND SCS30Khz |
| sl1280 | 163 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sequenceId-r16 | 0 |  |  |
| spatialRelationInfoPos-r16 CHOICE { |  |  |  |
| servingRS-r16 SEQUENCE { |  |  |  |
| servingCellId | 0 |  |  |
| referenceSignal-r16 CHOICE { |  |  |  |
| ssb-IndexServing-r16 | 1 | Set according to Table 4.4.2-2 in TS 38.508-1. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| HD_FDD | pc_halfDuplexFDD_TypeA_RedCap_r17 (i.e HD_FDD UE are performing test on FDD band) |

8.4	Default LPP message and information elements contents

The default values of LPP messages and information elements used, unless indicated otherwise in specific clauses of this specification are as defined in clause 5.4 with the following exceptions:

-	LPP PROVIDE ASSISTANCE DATA

Table 8.4-1: ProvideAssistanceData

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { | Dependent on test case. |  |  |
| initiator |  |  |  |
| transactionNumber |  |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData-r9 SEQUENCE { |  |  |  |
| commonIEsProvideAssistanceData | Not present |  |  |
| a-gnss-ProvideAssistanceData SEQUENCE { |  |  | Sub-tests 7 and 15 only; and as defined in Table 5.4.1.1-1. |
| gnss-CommonAssistData SEQUENCE { |  |  |  |
| gnss-ReferenceTime | As defined in 37.571-5 [12] |  |  |
| gnss-ReferenceLocation | As defined in 37.571-5 [12] |  |  |
| gnss-IonosphericModel | As defined in 37.571-5 [12] |  |  |
| gnss-EarthOrientationParameters | Not present |  |  |
| gnss-RTK-ReferenceStationInfo-r15 | Not present | Rel-15 onwards |  |
| gnss-RTK-CommonObservationInfo-r15 | Not present | Rel-15 onwards |  |
| gnss-RTK-AuxiliaryStationData-r15 | Not present | Rel-15 onwards |  |
| gnss-SSR-CorrectionPoints-r16 | Not present | Rel-16 onwards |  |
| gnss-Integrity-ServiceParameters-r17 | Not present | Rel-17 onwards |  |
| gnss-Integrity-ServiceAlert-r17 | Not present | Rel-17 onwards |  |
| } |  |  |  |
| gnss-GenericAssistData(SIZE(1..4))OF{ | SIZE is dependent on the number of GNSSs supported by the UE. If one GNSS supported by the UE, SIZE = 1 If two GNSSs supported by the UE, SIZE = 2 If three GNSSs supported by the UE, SIZE = 3 If four GNSSs supported by the UE, SIZE = 4 |  |  |
| gnss-ID | For each GNSS supported by the UE. |  |  |
| sbas-ID | Not present |  |  |
| gnss-TimeModels | As defined in 37.571-5 [12] |  |  |
| gnss-DifferentialCorrections | Not present |  |  |
| gnss-NavigationModel | As defined in 37.571-5 [12] |  |  |
| gnss-RealTimeIntegrity | Not present |  |  |
| gnss-DataBitAssistance | Not present |  |  |
| gnss-AcquisitionAssistance | As defined in 37.571-5 [12] |  |  |
| gnss-Almanac | As defined in 37.571-5 [12] |  |  |
| gnss-UTC-Model | As defined in 37.571-5 [12] |  |  |
| gnss-AuxiliaryInformation | As defined in 37.571-5 [12] |  |  |
| bds-DifferentialCorrections-r12 | Not present | Rel-12 onwards |  |
| bds-GridModel-r12 | Not present | Rel-12 onwards |  |
| gnss-RTK-Observations-r15 | Not present | Rel-15 onwards |  |
| glo-RTK-BiasInformation-r15 | Not present | Rel-15 onwards |  |
| gnss-RTK-MAC-CorrectionDifferences-r15 | Not present | Rel-15 onwards |  |
| gnss-RTK-Residuals-r15 | Not present | Rel-15 onwards |  |
| gnss-RTK-FKP-Gradients-r15 | Not present | Rel-15 onwards |  |
| gnss-SSR-OrbitCorrections-r15 | Not present | Rel-15 onwards |  |
| gnss-SSR-ClockCorrections-r15 | Not present | Rel-15 onwards |  |
| gnss-SSR-CodeBias-r15 | Not present | Rel-15 onwards |  |
| gnss-SSR-URA-r16 | Not present | Rel-16 onwards |  |
| gnss-SSR-PhaseBias-r16 | Not present | Rel-16 onwards |  |
| gnss-SSR-STEC-Correction-r16 | Not present | Rel-16 onwards |  |
| gnss-SSR-GriddedCorrection-r16 | Not present | Rel-16 onwards |  |
| navic-DifferentialCorrections-r16 | Not present | Rel-16 onwards |  |
| navic-GridModel-r16 | Not present | Rel-16 onwards |  |
| } |  |  |  |
| gnss-Error | Not present |  |  |
| } |  |  |  |
| otdoa-ProvideAssistanceData SEQUENCE { |  |  | Sub-tests 5 and 7 only |
| otdoa-ReferenceCellInfo | As defined in Table 8.4.1.2-1 |  |  |
| otdoa-NeighbourCellInfo | As defined in Table 8.4.1.2-2 |  |  |
| otdoa-Error | Not present |  |  |
| otdoa-ReferenceCellInfoNB-r14 | Not present | Rel-14 onwards |  |
| otdoa-NeighbourCellInfoNB-r14 | Not present | Rel-14 onwards |  |
| } |  |  |  |
| epdu-Provide-AssistanceData | Not present |  |  |
| sensor-ProvideAssistanceData-r14 SEQUENCE { |  | Rel-14 onwards | Sub-test 18 only as defined in clause 5.4.1.5 |
| sensor-AssistanceDataList-r14 | As defined in Table 5.4.1.5-2 |  |  |
| sensor-Error-r14 | Not present |  |  |
| } |  |  |  |
| tbs-ProvideAssistanceData-r14 SEQUENCE { |  | Rel-14 onwards | Sub-test 16 only as defined in clause5.4.1.3 |
| tbs-AssistanceDataList-r14 SEQUENCE { |  |  |  |
| mbs-AssistanceDataList-r14 SEQUENCE (SIZE(1..n)) OF SEQUENCE{ |  |  |  |
| mbs-AlmanacAssistance-r14 | As defined in Table 5.4.1.3-2 |  |  |
| mbs-AcquisitonAssistance-r14 | As defined in Table 5.4.1.3-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| tbs-Error-r14 | Not present |  |  |
| } |  |  |  |
| wlan-ProvideAssistanceData-r14 SEQUENCE { |  | Rel-14 onwards | Sub-test 17 only as defined in clause 5.4.1.4 |
| wlan-DataSet-r14 | As defined in Table 5.4.1.4-2 |  |  |
| wlan-Error-r14 | Not present |  |  |
| } |  |  |  |
| nr-Multi-RTT-ProvideAssistanceData-r16 SEQUENCE { |  | Rel-16 onwards | Sub-test 19 and Sub-test 26 |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 8.4.1.6-1 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | As defined in Table 8.4.1.6-2 |  |  |
| nr-Multi-RTT-Error-r16 | Not present |  |  |
| nr-On-Demand-DL-PRS-Configurations-r17 | Not present | Rel-17 onwards |  |
| nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17 | Not present | Rel-17 onwards |  |
| assistanceDataValidityArea-r17 | Not present | Rel-17 onwards |  |
| } |  |  |  |
| nr-DL-AoD-ProvideAssistanceData-r16 SEQUENCE { |  | Rel-16 onwards | Sub-test 20 and Sub-test 27 |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 8.4.1.6-1 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | As defined in Table 8.4.1.6-2 |  |  |
| nr-PositionCalculationAssistance-r16 | Present or not present dependent on pc_UEB_DL_AoD. As defined in Table 8.4.1.7-1 |  |  |
| nr-DL-AoD-Error-r16 | Not present |  |  |
| nr-DL-PRS-BeamInfo-r17 | Not present | Rel-17 onwards |  |
| nr-On-Demand-DL-PRS-Configurations-r17 | Not present | Rel-17 onwards |  |
| nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17 | Not present | Rel-17 onwards |  |
| assistanceDataValidityArea-r17 | Not present | Rel-17 onwards |  |
| } |  |  |  |
| nr-DL-TDOA-ProvideAssistanceData-r16 SEQUENCE { |  | Rel-16 onwards | Sub-test 21 and Sub-test 28 |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 8.4.1.6-1 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | As defined in Table 8.4.1.6-2 |  |  |
| nr-PositionCalculationAssistance-r16 | Present or not present dependent on pc_UEB_DL_TDOA. As defined in Table 8.4.1.7-1 |  |  |
| nr-DL-TDOA-Error-r16 | Not present |  |  |
| nr-On-Demand-DL-PRS-Configurations-r17 | Not present | Rel-17 onwards |  |
| nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17 | Not present | Rel-17 onwards |  |
| assistanceDataValidityArea-r17 | Not present | Rel-17 onwards |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-	LPP REQUEST CAPABILITIES

Table 8.4-2: RequestCapabilities

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| Initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestCapabilities SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestCapabilities-r9 SEQUENCE { |  |  |  |
| commonIEsRequestCapabilities SEQUENCE { | Present | Rel-14 onwards |  |
| lpp-message-segmentation-req-r14 | 00 | Server is not able to send segmented LPP messages. Server is not able to receive segmented LPP messages. Rel-14 onwards |  |
| } |  |  |  |
| a-gnss-RequestCapabilities SEQUENCE { |  |  |  |
| gnss-SupportListReq | TRUE |  |  |
| assistanceDataSupportListReq | TRUE |  |  |
| locationVelocityTypesReq | TRUE |  |  |
| } |  |  |  |
| otdoa-RequestCapabilities SEQUENCE { | Present |  |  |
| } |  |  |  |
| ecid-RequestCapabilities SEQUENCE { | Present |  |  |
| } |  |  |  |
| epdu-RequestCapabilities | Not present |  |  |
| sensor-RequestCapabilities-r13 SEQUENCE { | Present | Rel-13 onwards |  |
| } |  |  |  |
| tbs-RequestCapabilities-r13 SEQUENCE { | Present | Rel-13 onwards |  |
| } |  |  |  |
| wlan-RequestCapabilities-r13 SEQUENCE { | Present | Rel-13 onwards |  |
| } |  |  |  |
| bt-RequestCapabilities-r13 SEQUENCE { | Present | Rel-13 onwards |  |
| } |  |  |  |
| nr-ECID-RequestCapabilities-r16 SEQUENCE { | Present | Rel-16 onwards |  |
| } |  |  |  |
| nr-Multi-RTT-RequestCapabilities-r16 SEQUENCE { | Present | Rel-16 onwards |  |
| } |  |  |  |
| nr-DL-AoD-RequestCapabilities-r16 SEQUENCE { | Present | Rel-16 onwards |  |
| } |  |  |  |
| nr-DL-TDOA-RequestCapabilities-r16 SEQUENCE { | Present | Rel-16 onwards |  |
| } |  |  |  |
| nr-UL-RequestCapabilities-r16 SEQUENCE { | Present | Rel-16 onwards |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-	LPP REQUEST LOCATION INFORMATION

Table 8.4-3: RequestLocationInformation

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsRequestLocationInformation SEQUENCE { |  |  |  |
| locationInformationType | Dependent on test case |  |  |
| triggeredReporting | Not present |  |  |
| periodicalReporting | Not present |  |  |
| additionalInformation | onlyReturnInformationRequested |  |  |
| qos SEQUENCE { |  |  |  |
| horizontalAccuracy | Not present |  |  |
| verticalCoordinateRequest | FALSE |  |  |
| verticalAccuracy | Not present |  |  |
| responseTime SEQUENCE { |  |  |  |
| time | 32 |  |  |
| responseTimeEarlyFix-r12 | Not present | Rel-12 onwards |  |
| } |  |  |  |
| velocityRequest | FALSE |  |  |
| messageSizeLimitNB-r14 | Not present | Rel-14 onwards |  |
| segmentationInfo-r14 | Not present | Rel-14 onwards |  |
| scheduledLocationTime-r17 | Not present | Rel-17 onwards |  |
| targetIntegrityRisk-r17 | Not present | Rel-17 onwards |  |
| } |  |  |  |
| environment | Not present |  |  |
| locationCoordinateTypes | Not present |  |  |
| velocityTypes | Not present |  |  |
| } |  |  |  |
| a-gnss-RequestLocationInformation | As defined in Table 5.4-4 |  | Sub-tests 7 and 15 |
| otdoa-RequestLocationInformation | As defined in Table 5.4-5 |  | Sub-test 5 and 7 |
| ecid-RequestLocationInformation | As defined in Table 5.4-6 |  | Sub-test 6 |
| epdu-RequestLocationInformation | Not Present |  |  |
| sensor-RequestLocationInformation-r13 | As defined in Table 5.4-10 | Rel-13 onwards | Sub-test 14, 18 |
| tbs-RequestLocationInformation-r13 | As defined in Table 5.4-7 | Rel-13 onwards | Sub-tests 12, 16 |
| wlan-RequestLocationInformation-r13 | As defined in Table 5.4-8 | Rel-13 onwards | Sub-test 11, 17 |
| bt-RequestLocationInformation-r13 | As defined in Table 5.4-9 | Rel-13 onwards | Sub-test 13 |
| nr-ECID-RequestLocationInformation-r16 | As defined in Table 8.4-4 | Rel-16 onwards | Sub-test 22 |
| nr-Multi-RTT-RequestLocationInformation-r16 | As defined in Table 8.4-5 | Rel-16 onwards | Sub-test 19 and Sub-test 26 |
| nr-DL-AoD-RequestLocationInformation-r16 | As defined in Table 8.4-6 | Rel-16 onwards | Sub-test 20 and Sub-test 27 |
| nr-DL-TDOA-RequestLocationInformation-r16 | As defined in Table 8.4-7 | Rel-16 onwards | Sub-test 21 and Sub-test 28 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-	NR E-CID REQUEST LOCATION INFORMATION

Table 8.4-4: NR-ECID-RequestLocationInformation

| Derivation Path: 37.355 clause 6.5.9.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-ECID-RequestLocationInformation-r16 ::= SEQUENCE { |  |  |  |
| requestedMeasurements-r16 | All measurements supported by the UE |  |  |
| } |  |  |  |

-	NR Multi-RTT REQUEST LOCATION INFORMATION

Table 8.4-5: NR-Multi-RTT-RequestLocationInformation

| Derivation Path: 37.355 clause 6.5.12.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-Multi-RTT-RequestLocationInformation-r16 ::= SEQUENCE { |  |  |  |
| nr-UE-RxTxTimeDiffMeasurementInfoRequest-r16 | Not present |  |  |
| nr-RequestedMeasurements-r16 | bit 0 = 1 (prsrsrpReq) |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-Multi-RTT-ReportConfig-r16 SEQUENCE { |  |  |  |
| maxDL-PRS-RxTxTimeDiffMeasPerTRP-r16 | Not present |  |  |
| timingReportingGranularityFactor-r16 | Not present |  |  |
| } |  |  |  |
| additionalPaths-r16 | Not present |  |  |
| nr-UE-RxTxTEG-Request-r17 | Not present | Rel-17 onwards |  |
| measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 | Not present | Rel-17 onwards |  |
| measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 | Not present | Rel-17 onwards |  |
| reducedDL-PRS-ProcessingSamples-r17 | Not present | Rel-17 onwards |  |
| nr-los-nlos-IndicatorRequest-r17 | Not present | Rel-17 onwards |  |
| additionalPathsExt-r17 | Not present | Rel-17 onwards |  |
| additionalPathsDL-PRS-RSRP-Request-r17 | Not present | Rel-17 onwards |  |
| multiMeasInSameReport-r17 | Not present | Rel-17 onwards |  |
| lowerRxBeamSweepingFactor-FR2-r17 | Not present | Rel-17 onwards |  |
| } |  |  |  |

-	NR DL-AoD REQUEST LOCATION INFORMATION

Table 8.4-6: NR-DL-AoD-RequestLocationInformation

| Derivation Path: 37.355 clause 6.5.11.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-AoD-RequestLocationInformation-r16 ::= SEQUENCE { |  |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-DL-AoD-ReportConfig-r16 SEQUENCE { |  |  |  |
| maxDL-PRS-RSRP-MeasurementsPerTRP-r16 | Not present |  |  |
| maxDL-PRS-RSRP-MeasurementsPerTRP-r17 | Not present | Rel-17 onwards |  |
| maxDL-PRS-RSRPP-MeasurementsPerTRP-r17 | Not present | Rel-17 onwards |  |
| nr-los-nlos-IndicatorRequest-r17 | Not present | Rel-17 onwards |  |
| reducedDL-PRS-ProcessingSamples-r17 | Not present | Rel-17 onwards |  |
| lowerRxBeamSweepingFactor-FR2-r17 | Not present | Rel-17 onwards |  |
| } |  |  |  |
| multiMeasInSameReport-r17 | Not present | Rel-17 onwards |  |
| } |  |  |  |

-	NR DL-TDOA REQUEST LOCATION INFORMATION

Table 8.4-7: NR-DL-TDOA-RequestLocationInformation

| Derivation Path: 37.355 clause 6.5.10.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-RstdMeasurementInfoRequest-r16 | Not present |  |  |
| nr-RequestedMeasurements-r16 | bit 0 = 1 (prsrsrpReq) |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-DL-TDOA-ReportConfig-r16 SEQUENCE { |  |  |  |
| maxDL-PRS-RSTD-MeasurementsPerTRPPair-r16 | Not present |  |  |
| timingReportingGranularityFactor-r16 | Not present |  |  |
| measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 | Not present | Rel-17 onwards |  |
| reducedDL-PRS-ProcessingSamples-r17 | Not present | Rel-17 onwards |  |
| lowerRxBeamSweepingFactor-FR2-r17 | Not present | Rel-17 onwards |  |
| } |  |  |  |
| additionalPaths-r16 | Not present |  |  |
| nr-UE-RxTEG-Request-r17 | Not present | Rel-17 onwards |  |
| nr-los-nlos-IndicatorRequest-r17 | Not present | Rel-17 onwards |  |
| additionalPathsExt-r17 | Not present | Rel-17 onwards |  |
| additionalPathsDL-PRS-RSRP-Request-r17 | Not present | Rel-17 onwards |  |
| multiMeasInSameReport-r17 | Not present | Rel-17 onwards |  |
| } |  |  |  |

8.4.1	Default assistance data information elements

8.4.1.1	GNSS Assistance Data Elements

The GNSS assistance data elements which shall be provided to the UE in the tests in LPP Provide Assistance Data messages in the absence of a corresponding LPP Request Assistance Data message are as defined in clause 5.4.1.1.

8.4.1.2	OTDOA (LTE) Assistance Data Elements

This clause defines the OTDOA (LTE) assistance data elements which shall be provided to the UE in the tests in LPP Provide Assistance Data messages.

-	OTDOA (LTE) REFERENCE CELL INFO

Table 8.4.1.2-1: OTDOA-ReferenceCellInfo

| Derivation Path: 37.355 clause 6.5.1.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| OTDOA-ReferenceCellInfo ::= SEQUENCE { |  | LTE Cell 1 |  |
| physCellId | 0 |  |  |
| cellGlobalId SEQUENCE { |  |  |  |
| mcc | As defined for Cell 1 in 36.508 [8] |  |  |
| mnc | As defined for Cell 1 in 36.508 [8] |  |  |
| cellidentity | As defined for E-UTRAN Cell Identifier for Cell 1 in 36.508 [8] |  |  |
| } |  |  |  |
| earfcnRef | As defined for Cell 1 in 36.508 [8] |  |  |
| antennaPortConfig | As defined for Cell 1 in 36.508 [8] |  |  |
| cpLength | Normal |  |  |
| prsInfo SEQUENCE { |  |  |  |
| prs-Bandwidth | PRS are transmitted over the used system bandwidth (see clause 5.2.2) |  |  |
| prs-ConfigurationIndex | FDD: 2 TDD: 4 |  |  |
| numDL-Frames | sf-1 |  |  |
| prs-MutingInfo-r9 | Not present | PRS muting is not used. |  |
| prsID-r14 | Not present | PRS-ID not used | Rel-14 onwards |
| add-numDL-Frames-r14 | Not present | Not required | Rel-14 onwards |
| prsOccGroupLen-r14 | Not present | No PRS occasion group configured | Rel-14 onwards |
| prsHoppingInfo-r14 | Not present | PRS frequency hopping not used | Rel-14 onwards |
| } |  |  |  |
| earfcnRef-v9a0 | As defined for Cell 1 in 36.508 [8] |  |  |
| tpId-r14 | Not present | Transmission Points not used | Rel-14 onwards |
| cpLengthCRS-r14 | Normal |  | Rel-14 onwards |
| sameMBSFNconfigRef-r14 | FALSE | Not the same as the serving cell | Rel-14 onwards |
| dlBandwidth-r14 | Not present | PRS frequency hopping not used | Rel-14 onwards |
| addPRSconfigRef-r14 | Not present | No additional PRS configuration(s) | Rel-14 onwards |
| nr-LTE-SFN-Offset-r15 | Not present |  | Rel-15 onwards |
| tdd-config-v1520 | Not present |  | Rel-15 onwards |
| nr-LTE-fineTiming-Offset-r15 | Not present |  | Rel-15 onwards |
| } |  |  |  |

-	OTDOA (LTE) NEIGHBOUR CELL INFO LIST

Table 8.4.1.2-2: OTDOA-NeighbourCellInfoList

| Derivation Path: 37.355 clause 6.5.1.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| OTDOA-NeighbourCellInfoList ::= SEQUENCE (SIZE(1)) OF SEQUENCE { |  |  |  |
| SEQUENCE (SIZE(2)) OF SEQUENCE { |  | Cell 2 |  |
| physCellId | 2 |  |  |
| cellGlobalId SEQUENCE { |  |  |  |
| mcc | As defined for Cell 2 in 36.508 [8] |  |  |
| mnc | As defined for Cell 2 in 36.508 [8] |  |  |
| cellidentity | As defined for E-UTRAN Cell Identifier for Cell 2 in 36.508 [8] |  |  |
| } |  |  |  |
| earfcn | Not present | Same as for the reference cell |  |
| cpLength | Not present | Same as for the reference cell |  |
| prsInfo | Not present | Same as for the reference cell |  |
| antennaPortConfig | Not present | Same as for the reference cell |  |
| slotNumberOffset | Not present | Slot timing is the same as for reference cell |  |
| prs-SubframeOffset | Not present |  |  |
| expectedRSTD | 8192 | Value 0 |  |
| expectedRSTD-Uncertainty | 10 | About 1 s |  |
| earfcn-v9a0 | Not present | Same as for the reference cell |  |
| tpId-r14 | Not present | Transmission Points not used | Rel-14 onwards |
| prs-only-tp-r14 | Not present | Not required | Rel-14 onwards |
| cpLengthCRS-r14 | Not present | Not required | Rel-14 onwards |
| sameMBSFNconfigNeighbour-r14 | TRUE | Same as for the reference cell | Rel-14 onwards |
| dlBandwidth-r14 | Not present | Same as for the reference cell and PRS frequency hopping not used | Rel-14 onwards |
| addPRSconfigNeighbour-r14 | Not present | No additional PRS configuration(s) | Rel-14 onwards |
| tdd-config-v1520 | Not present |  | Rel-15 onwards |
| } |  |  |  |
| SEQUENCE { |  | Cell 4 |  |
| physCellId | 4 |  |  |
| cellGlobalId SEQUENCE { |  |  |  |
| mcc | As defined for Cell 4 in 36.508 [8] |  |  |
| mnc | As defined for Cell 4 in 36.508 [8] |  |  |
| cellidentity | As defined for E-UTRAN Cell Identifier for Cell 4 in 36.508 [8] |  |  |
| } |  |  |  |
| earfcn | Not present | Same as for the reference cell |  |
| cpLength | Not present | Same as for the reference cell |  |
| prsInfo | Not present | Same as for the reference cell |  |
| antennaPortConfig | Not present | Same as for the reference cell |  |
| slotNumberOffset | Not present | Slot timing is the same as for reference cell |  |
| prs-SubframeOffset | Not present |  |  |
| expectedRSTD | 8192 | Value 0 |  |
| expectedRSTD-Uncertainty | 10 | About 1 s |  |
| earfcn-v9a0 | Not present | Same as for the reference cell |  |
| tpId-r14 | Not present | Transmission Points not used | Rel-14 onwards |
| prs-only-tp-r14 | Not present | Not required | Rel-14 onwards |
| cpLengthCRS-r14 | Not present | Not required | Rel-14 onwards |
| sameMBSFNconfigNeighbour-r14 | TRUE | Same as for the reference cell | Rel-14 onwards |
| dlBandwidth-r14 | Not present | Same as for the reference cell and PRS frequency hopping not used | Rel-14 onwards |
| addPRSconfigNeighbour-r14 | Not present | No additional PRS configuration(s) | Rel-14 onwards |
| tdd-config-v1520 | Not present |  | Rel-15 onwards |
| } |  |  |  |
| } |  |  |  |

8.4.1.3	MBS Assistance Data Elements

The MBS assistance data elements which shall be provided to the UE in sub-test 16 via LPP Provide Assistance Data messages in the absence of a corresponding LPP Request Assistance Data message are as defined in clause 5.4.1.3.

8.4.1.4	WLAN Assistance Data Elements

The WLAN assistance data elements which shall be provided to the UE in sub-test 17 via LPP Provide Assistance Data messages in the absence of a corresponding LPP Request Assistance Data message are as defined in clause 5.4.1.4.

8.4.1.5	Sensor Assistance Data Elements

The Sensor assistance data elements which shall be provided to the UE in sub-test 18 via LPP Provide Assistance Data messages in the absence of a corresponding LPP Request Assistance Data message are as defined in clause 5.4.1.5.

8.4.1.6	Multi-RTT Assistance Data Elements

This clause defines the Multi-RTT assistance data elements which shall be provided to the UE in the tests in LPP Provide Assistance Data messages.

-	NR DL-PRS ASSISTANCE DATA

Table 8.4.1.6-1: NR-DL-PRS-AssistanceData

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | SubcarrierSpacing | 38.508-1 [30] Table 4.6.3-188 |  |
| dl-PRS-ResourceBandwidth-r16 | 1 | 24 PRBs |  |
| dl-PRS-StartPRB-r16 | same value as ‘offsetToCarrier’ as defined for the DL frequency of NR Cell 1 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the NR Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | 0 |  |  |
| nr-CellGlobalID-r16 SEQUENCE { |  |  |  |
| mcc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 1 |  |  |
| mnc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 1 |  |  |
| nr-cellidentity-r15 | Set to NR Cell 1 Identifier defined in TS 38.508-1 Table 4.4.2-2 |  |  |
| } |  |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 1 frequency |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 4 | About 1 s |  |
| nr-DL-PRS-Info-r16 | NR-DL-PRS-Info-r16 as specified in Table 8.4.1.6-3 |  |  |
| prs-OnlyTP-r16 | Not present |  |  |
| nr-DL-PRS-ExpectedAoD-or-AoA-r17 | Not present | Rel-17 onwards |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | In case of sub-test 20/27 UE-based DL-AoD or sub-test 21/28 DL-TDOA method supported by the UE as defined in clause 8.2.10 and clause 8.2.11. |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | 2 |  |  |
| nr-CellGlobalID-r16 SEQUENCE { |  |  |  |
| mcc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 2 |  |  |
| mnc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 2 |  |  |
| nr-cellidentity-r15 | Set to NR Cell 2 Identifier defined in TS 38.508-1 Table 4.4.2-2 |  |  |
| } |  |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 4 | About 1 s |  |
| nr-DL-PRS-Info-r16 | NR-DL-PRS-Info-r16 as specified in Table 8.4.1.6-3 |  |  |
| prs-OnlyTP-r16 | Not present |  |  |
| nr-DL-PRS-ExpectedAoD-or-AoA-r17 | Not present | Rel-17 onwards |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[2] SEQUENCE { |  | entry 2 | In case of sub-test 21 UE-based DL-TDOA method supported by the UE as defined in clause 8.2.11 |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | SubcarrierSpacing | 38.508-1 [30] Table 4.6.3-188 |  |
| dl-PRS-ResourceBandwidth-r16 | 1 | 24 PRBs |  |
| dl-PRS-StartPRB-r16 | same value as ‘offsetToCarrier’ as defined for the DL frequency of NR Cell 3 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the NR Cell 3 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 | 2 |  |  |
| nr-PhysCellID-r16 | 3 |  |  |
| nr-CellGlobalID-r16 SEQUENCE { |  |  |  |
| mcc-r15 | As defined in TS 38.508-1 [30] Table 4.4.2-3 for NR Cell 3 |  |  |
| mnc-r15 | As defined in TS 38.508-1 [30] Table 4.4.2-3 for NR Cell 3 |  |  |
| nr-cellidentity-r15 | Set to NR Cell 3 Identifier defined in TS 38.508-1 [30] Table 4.4.2-2 |  |  |
| } |  |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 3 frequency |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 4 | About 1 s |  |
| nr-DL-PRS-Info-r16 | NR-DL-PRS-Info-r16 as specified in Table 8.4.1.6-3 |  |  |
| prs-OnlyTP-r16 | Not present |  |  |
| nr-DL-PRS-ExpectedAoD-or-AoA-r17 | Not present | Rel-17 onwards |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-SSB-Config-r16 SEQUENCE (SIZE (1..nrMaxTRPs-r16)) OF NR-SSB-Config-r16 { | 1 entry |  | In case of sub-test 21 UE-based DL-TDOA method supported by the UE as defined in clause 8.2.11 |
| NR-SSB-Config-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-PhysCellID-r16 | 3 |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 3 frequency | As defined in 38.508 -1 [30] subclause 6.2.3. |  |
| ss-PBCH-BlockPower-r16 | 0 |  |  |
| halfFrameIndex-r16 | 0 |  |  |
| ssb-periodicity-r16 | ms20 |  |  |
| ssb-PositionsInBurst-r16 | Not present |  |  |
| ssb-SubcarrierSpacing-r16 | Subcarrier spacing of SSB for NR Cell 3 |  |  |
| sfn-SSB-Offset-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-	NR-SelectedDL-PRS-IndexList

Table 8.4.1.6-2: NR-SelectedDL-PRS-IndexList

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-SelectedDL-PRS-IndexList-r16 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-SelectedDL-PRS-PerFreq-r16 { | 2 entries |  |  |
| NR-SelectedDL-PRS-PerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-SelectedDL-PRS-FrequencyLayerIndex-r16 | 0 |  |  |
| nr-SelectedDL-PRS-IndexListPerFreq-r16 | Not present |  |  |
| } |  |  |  |
| NR-SelectedDL-PRS-PerFreq-r16[2] SEQUENCE { |  | entry 2 | Sub-test 21 UE-Based only as defined in clause 8.2.11 |
| nr-SelectedDL-PRS-FrequencyLayerIndex-r16 | 1 |  |  |
| nr-SelectedDL-PRS-IndexListPerFreq-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

-	NR-DL-PRS-Info

Table 8.4.1.6-3: NR-DL-PRS-Info

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |  |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { |  | 1 entry |  |  |  |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  |  |  | entry 1 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  | 0 |  |  |  |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 |  | The periodicity is 160ms and the resource set slot offset is 11 ms for any SCS configuration |  |  |  |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 |  | Not present |  |  |  |  |  |
| dl-PRS-ResourceTimeGap-r16 |  | Not present |  |  |  |  |  |
| dl-PRS-NumSymbols-r16 |  | N4 |  |  |  |  |  |
| dl-PRS-MutingOption1-r16 |  | Not present |  |  |  |  |  |
| dl-PRS-MutingOption2-r16 |  | Not present |  |  |  |  |  |
| dl-PRS-ResourcePower-r16 |  | 0 |  |  |  |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { |  | 1 entry |  |  |  |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  |  |  | entry 1 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  | 0 |  |  |  |  |  |
| dl-PRS-SequenceID-r16 |  | 0 |  |  |  | NR-DL-PRS-AssistanceDataPerFreq-r16[1] NR-DL-PRS-AssistanceDataPerTRP-r16[1] Table 8.4.1.6-1 |  |
|  |  | 1 |  |  |  | NR-DL-PRS-AssistanceDataPerFreq-r16[1] NR-DL-PRS-AssistanceDataPerTRP-r16[2], Table 8.4.1.6-1 |  |
|  |  | 0 |  |  |  | NR-DL-PRS-AssistanceDataPerFreq-r16[2] NR-DL-PRS-AssistanceDataPerTRP-r16[1] Table 8.4.1.6-1 |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |  |  |  |  |
| n2-r16 |  | 0 |  |  |  | NR-DL-PRS-AssistanceDataPerFreq-r16[1] NR-DL-PRS-AssistanceDataPerTRP-r16[1] Table 8.4.1.6-1 |  |
|  |  | 1 |  |  |  | NR-DL-PRS-AssistanceDataPerFreq-r16[1] NR-DL-PRS-AssistanceDataPerTRP-r16[2] Table 8.4.1.6-1 |  |
|  |  | 0 |  |  |  | NR-DL-PRS-AssistanceDataPerFreq-r16[2] NR-DL-PRS-AssistanceDataPerTRP-r16[1] Table 8.4.1.6-1 |  |
| } |  |  |  |  |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 |  | 0 |  |  |  |  |  |
| dl-PRS-ResourceSymbolOffset-r16 |  | 0 |  |  |  |  |  |
| dl-PRS-QCL-Info-r16 |  | Not present |  |  |  |  |  |
| dl-PRS-ResourcePrioritySubset-r17 |  | Not present |  | Rel-17 onwards |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

8.4.1.7	DL-AoD Assistance Data Elements

This clause defines the DL-AoD assistance data elements which shall be provided to the UE in the tests in LPP Provide Assistance Data messages in addition to those defined in Table 8.4.1.6-1 and Table 8.4.1.6-2 and Table 8.4.1.6-3.

-	NR-PositionCalculationAssistance

Table 8.4.1.7-1: NR-PositionCalculationAssistance

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-PositionCalculationAssistance-r16 ::= SEQUENCE { |  |  |  |
| nr-TRP-LocationInfo-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-TRP-LocationInfoPerFreqLayer-r16 { | 2 entries |  |  |
| NR-TRP-LocationInfoPerFreqLayer-r16[1] SEQUENCE { |  | entry1 |  |
| referencePoint-r16 |  |  |  |
| trp-LocationInfoList-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF TRP-LocationInfoElement-r16 { | 2 entries |  |  |
| TRP-LocationInfoElement-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Not present |  |  |
| nr-CellGlobalID-r16 | Not present |  |  |
| nr-ARFCN-r16 | Not present |  |  |
| associated-DL-PRS-ID-r16 | Not present |  |  |
| trp-Location-r16 | Not present | Same as the reference point location |  |
| trp-DL-PRS-ResourceSets-r16 | Not present |  |  |
| } |  |  |  |
| TRP-LocationInfoElement-r16[2] SEQUENCE { |  | entry 2 | In case of sub-test 20/27 UE-based DL-AoD or sub-test 21/28 DL-TDOA method supported by the UE as defined in clause 8.2.10 and clause 8.2.11. |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Not present |  |  |
| nr-CellGlobalID-r16 | Not present |  |  |
| nr-ARFCN-r16 | Not present |  |  |
| associated-DL-PRS-ID-r16 | Not present |  |  |
| trp-Location-r16 | trp-Location-r16 for NR cell 2 as defined in 37.571-5 [12] |  |  |
| trp-DL-PRS-ResourceSets-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NR-TRP-LocationInfoPerFreqLayer-r16[2] SEQUENCE { |  | entry 2 |  |
| referencePoint-r16 | Not present | Same as in the previous entry of the NR-TRP-LocationInfoPerFreqLayer list. |  |
| trp-LocationInfoList-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF TRP-LocationInfoElement-r16 { | 1 entry |  |  |
| TRP-LocationInfoElement-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 | 2 |  |  |
| nr-PhysCellID-r16 | Not present |  |  |
| nr-CellGlobalID-r16 | Not present |  |  |
| nr-ARFCN-r16 | Not present |  |  |
| associated-DL-PRS-ID-r16 | Not present |  |  |
| trp-Location-r16 | trp-Location-r16 for NR cell 3 as defined in TS 37.571-5 [12] |  |  |
| trp-DL-PRS-ResourceSets-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-DL-PRS-BeamInfo-r16 | Not present |  |  |
| nr-RTD-Info-r16 | Not present |  |  |
| nr-TRP-BeamAntennaInfo-r17 | Not present | Rel-17 onwards |  |
| nr-DL-PRS-Expected-LOS-NLOS-Assistance-r17 | Not present | Rel-17 onwards |  |
| nr-DL-PRS-TRP-TEG-Info-r17 | Not present | Rel-17 onwards |  |
| } |  |  |  |

8.4.1.8	DL-TDOA Assistance Data Elements

The DL-TDOA assistance data elements which shall be provided to the UE in the tests in LPP Provide Assistance Data messages are defined in Table 8.4.1.6-1, Table 8.4.1.6-2, Table 8.4.1.6-3 and Table 8.4.1.7-1.