3GPP TSG-RAN5 Meeting #99	R5-233492

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0312 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to SDP Message from the UE for MCData |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2023-05-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MCData SDP from the UE was missing elements to enable pre-established session functionality |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the PRE_ESTABLISHED_SESSION condition to Table 5.5.3.1.1-3. Added IEs to enable pre-established session functionality to Table 5.5.3.1.1-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SDP used in MCData may not be correct and cause multiple test cases to have false results |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Final version of R5-232603 |  |  |  |  |  |  |  |  |

<START OF CHANGES>

Table 5.5.3.1.1-3: SDP Message from the UE for MCData

| Derivation Path: RFC 4566 [27] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Session description: |  |  |  |  |
| Protocol Version | "0" | v= line |  |  |
| Origin | Same o=line as in the previous SDP message sent by the UE except that sess-version is incremented by one | o= line |  |  |
| Origin |  | o= line |  | FIRST_SDP_FROM_UE |
| username | any allowed value |  |  |  |
| sess-id | any allowed value | A numeric string such that the tuple of <username>, <sess-id>, <nettype>, <addrtype>, and <unicast-address> forms a globally unique identifier for the session. |  |  |
| sess-version | any allowed value |  |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| unicast-address | IP address of the UE | IP address assigned at initial registration |  |  |
| Session Name | at least one UTF-8-encoded character, or if no name is given, a single empty space | s= line |  |  |
| Connection Data | not required if included in all media | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| Session attribute | optional (NOTE 1) | a=line attribute=”ice-lite” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| ice-lite |  |  |  |  |
| Time description |  |  |  |  |
| Timing |  | t= line |  |  |
| start-time | "0" |  |  |  |
| stop-time | "0" |  |  |  |
| Session attribute | present only if there is no key-mgmt media attribute in the media description for audio | a= line attribute = key-mgmt |  | SDP_OFFER AND MCD_1to1 |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A |  | RFC 4567 [44] |  |
| Media description[1] |  | Media description for data |  |  |
| media description |  | m= line media = message | RFC 4867 [59] TS 24.282 [31] |  |
| media | "message" |  |  |  |
| port | any allowed value | The transport port to which the media stream is sent |  |  |
| proto | "TCP/MSRP" |  |  |  |
| fmt | “*” |  |  |  |
| Connection Data | present if session description does not contain a c=line; optional otherwise | c= line |  |  |
| nettype | "IN" |  |  |  |
| Addrtype | "IP4" or "IP6" depending on IP address" |  |  |  |
| connection-address | IP address of the UE |  |  |  |
| media attribute |  | a= line attribute = sendonly |  | SDP_OFFER AND NOT SDS_SESSION |
| sendonly |  | No parameters associated with this line |  |  |
| media attribute |  | a= line attribute = recvonly |  | SDP_ANSWER AND NOT SDS_SESSION |
| recvonly |  | No parameters associated with this line |  |  |
| media attribute |  | a= line attribute = sendrecv |  | SDS_SESSION |
| sendrecv |  | No parameters associated with this line |  |  |
| media attribute |  | a= line attribute = path |  |  |
| path | MSRP URI according to RFC 4975 [120] clause 6 and 9 | attribute containing its own MSRP URI. An example: msrp://mcdata.example.com:7654/abcde1; tcp | TS 24.282 [31] |  |
| scheme | "msrp" |  |  |  |
| authority |  |  | RFC 3986 [123] clause 3.2 |  |
| userinfo | any value if present |  |  |  |
| host | any allowed value | domain name or IP address of the UE |  |  |
| port | same value as in the media line if present | port at which the UE may be connected to for MSRP; mandatory when hostname is an IP address |  |  |
| session id | any allowed value if present |  |  |  |
| transport | "tcp" | mandatory for MSRP according to RFC 4975 [120] clause 6 |  |  |
| URI-parameter | not present |  |  |  |
| media attribute |  | a= line attribute = accept-types | RFC 4975 [120] |  |
| accept-types |  |  |  |  |
| format-entry[1] | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| format-entry[2] | "application/vnd.3gpp.mcdata-payload" |  |  | MCDATA_SDS |
| media attribute |  | a=line attribute=”candidate” | RFC 5245 [115] | PRE_ESTABLISHED_SESSION |
| candidate |  | candidate for TCP/MSRP |  |  |
| foundation | any value |  |  |  |
| component-id | 1 | according to RFC 5245 [115] clause 4.1.1.1 |  |  |
| transport | "TCP/MSRP" |  |  |  |
| priority | any value |  |  |  |
| connection-address | same IP address as in media's c= line or in the session's c= line if the media does not have a c= line | default candidate |  |  |
| port | same port number as in the m= line |  |  |  |
| cand-type | "host" |  |  |  |
| media attribute |  | a= line attribute = setup | RFC 4145 [119] |  |
| setup | “actpass” |  |  | SDP_OFFER |
|  | "active" or "passive" |  |  | SDP_ANSWER |
| media attribute |  | a= line attribute = file-transfer-id | RFC 5547 [124] | MCDATA_FD |
| file-transfer-id | any allowed value |  | RFC 5547 [124] clause 8.2.1 | SDP_OFFER |
|  | same value as in the sdp offer |  | RFC 5547 [124] clause 8.2.2 | SDP_ANSWER |
| media attribute |  | a= line attribute = file-selector | RFC 5547 [124] | MCDATA_FD |
| file-selector |  |  |  | SDP_OFFER |
| selector[1] |  |  |  |  |
| filename | any allowed value | e.g. "TestFile.txt" |  |  |
| filesize | size of the file to be transferred |  |  |  |
| filetype | any allowed value | e.g. "text/plain" |  |  |
| hash |  |  |  |  |
| algorithm | "sha-1" |  |  |  |
| value | hash value of the file to be transferred |  |  |  |
| file-selector | same value as in the sdp offer |  |  | SDP_ANSWER |
| media attribute |  | a= line attribute = file-date | RFC 5547 [124] | MCDATA_FD AND SDP_OFFER |
| file-date |  |  |  |  |
| date-param | at least one entry with an allowed value |  |  |  |
| media attribute | present only if there is no key-mgmt attribute at session level | a= line attribute = key-mgmt |  | SDP_OFFER AND MCD_1to1 |
| key-mgmt |  |  | TS 24.379 [9] clause 6.2.1 |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 5.5.9.1-2A | Use condition MCDATA | RFC 4567 [44] |  |

<END OF CHANGES>