3GPP TSG-RAN5 Meeting #99	R5-233532

# Incheon, South Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2299 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for CA_n66A-n71A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Band n71 supports a maximum CHBW of 20 MHz. However, due to a typo in the test point analysis for CA_n66A-n71A, 216 RBs (40 MHz) is specified in the test configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The SCC NRB for CA_n66A-n71A has been corrected in Table 6.5A.3.2.1.4.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spurious emissions for UE co-existence for CA_n66A-n71A cannot be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.3.2.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0781 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | associated test point analysis update in CR R5-233530. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final outcome of one revision of R5-233166. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 6.5A.3.2.1	Spurious emissions for UE co-existence for CA (2UL CA)

6.5A.3.2.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions for 2UL CA.

6.5A.3.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support 2UL CA.

6.5A.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.3.2.0.

6.5A.3.2.1.4	Test description

6.5A.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.3.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.2.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  | Normal |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  | Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  |  |  |  |  |  |  | Lowest SCS for default test points. CA configuration specific test points: Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  |  |  |  |  |  |  | DL Allocation |  |  |  |  |  | UL Allocation |  |  |  |  |  |
|  | CA Configuration |  |  |  |  |  | PCC NRB @SCS | SCC NRB@SCS | CC MOD | PCC & SCC RB allocation |  |  |  |  | CC MOD |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
|  | PCC |  | SCC |  |  |  |  |  |  | PCC |  |  | SCC |  |  |  |  |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n1A-n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | Low | n8 | Low |  | 25@15kHz |  | 25@15kHz | CP-OFDM QPSK | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| Test Settings for CA_n1A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | Low | n78 | Low | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@105 |  | 1@272 |
| 2 | n1 | Low | n78 | 3421,14 MHz (NOTE 2) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| 3 | n1 | Low | n78 | High | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| Test Settings for CA_n1A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | LOW | n79 | MID | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@181 |
| 2 | n1 | LOW | n79 | 4618,41 MHz (NOTE 3) | 79@15kHz |  |  | 216@15kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| 3 | n1 | LOW | n79 | MID | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@105 |  | 1@245 |
| 4 | n1 | LOW | n79 | 4785,54 MHz (NOTE 4) | 106@15kHz |  |  | 216@15kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| Test Settings for CA_n2A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | Low | n5 | low | 20/106 |  |  | 20/106 | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| 2 | n2 | High | n5 | High | 20/106 |  |  | 20/106 | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@105 |  | 1@105 |
| Test Settings for CA_n2A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N/A (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n2A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | Low | n77 | Mid | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@272 |
| 2 | n2 | High | n77 | 3850,23 MHz (NOTE 5) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  |  | NA |  |  | CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| 3 | n2 | High | n77 | 3965,25 MHz (NOTE 6) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@0 |  | 1@0 |
| Test Settings for CA_n3A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | HIGH | n41 | HIGH | 160@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@0 |  | 1@151 |
| 2 | n3 | LOW | n41 | HIGH | 160@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@149 |  | 1@18 |
| 3 | n3 | LOW | n41 | HIGH | 133@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@132 |  | 1@155 |
| 4 | n3 | HIGH | n41 | LOW | 160@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@0 |  | 1@151 |
| 5 (NOTE 7) | n3 | HIGH | n41 | HIGH | 160@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@159 |  | 1@105 |
| 6 (NOTE 7) | n3 | LOW | n41 | HIGH | 79@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  |  | NA |  |  | - CP-OFDM QPSK |  |  | 1@78 |  | 1@153 |
| Test Settings for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | LOW | n78 | HIGH | 160@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@159 |  |  | 1@272 |  |
| 2 | n3 | HIGH | n78 | LOW | 160@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@159 |  |  | 1@0 |  |
| 3 | n3 | LOW | n78 | MID | 133@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@262 |  |
| 4 | n3 | LOW | n78 | MID | 160@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@41 |  |
| Test Settings for CA_n5A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | HIGH | n77 | MID | 79@15kHz |  |  | 162@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@161 |  |
| 2 | n5 | HIGH | n77 | MID | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@17 |  |
| 3 | n5 | LOW | n77 | 3458,79 MHz (NOTE 8) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 4 | n5 | HIGH | n77 | 3593,79 MHz (NOTE 9) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 5 | n5 | LOW | n77 | 4021,89 MHz (NOTE 10) | 52@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@51 |  |  | 1@0 |  |
| 6 | n5 | HIGH | n77 | HIGH | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@272 |  |
| 7 | n5 | LOW | n77 | 3538,29 MHz (NOTE 11) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n8 | HIGH | n78 | LOW | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 2 | n8 | LOW | n78 | MID | 79@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@192 |  |
| 3 | n8 | HIGH | n78 | HIGH | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@35 |  |
| 4 | n8 | LOW | n78 | LOW | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@0 |  |
| 5 | n8 | LOW | n78 | HIGH | 106@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@139 |  |
| Test Settings for CA_n24A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n24 | High | n41 | Low | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@136 |  |
| 2 | n24 | Low | n41 | Low | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@52 |  |  | 1@108 |  |
| 3 | n24 | High | n41 | Low | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@136 |  |
| 4 | n24 | Low | n41 | Low | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@94 |  |
| 5 | n24 | High | n41 | High | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@192 |  |
| Test Settings for CA_n24A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n24 | High | n48 | Mid | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@136 |  |
| 2 | n24 | High | n48 | High | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n24A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n24 | Low | n77 | Mid | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@136 |  |
| 2 | n24 | Low | n77 | 3930 MHz | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@273 |  |
| 3 | n24 | Low | n77 | 3930 MHz | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@262 |  |
| 4 | n24 | Low | n77 | 3930 MHz | 52@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@52 |  |  | 1@196 |  |
| Test Settings for CA_n26A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n26 | High | n66 | High | 106@15kHz |  |  | 216@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@0 |  |
| Test Settings for CA_n26A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n26 | Low | n70 | Low | 106@15kHz |  |  | 79@15kHz | QPSK/CP-OFDM QPSK |  | NA |  |  | QPSK / CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n28A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | LOW | n41 | HIGH | 79@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@78 |  |  | 1@157 |  |
| 2 | n28 | HIGH | n41 | HIGH | 79@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@78 |  |  | 1@115 |  |
| 3 | n28 | HIGH | n41 | LOW | 79@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@194 |  |
| Test Settings for CA_n39A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n39 | LOW | n41 | HIGH | 216@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@137 |  |
| Test Settings for CA_n41A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | LOW | n79 | LOW | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@272 |  |  | 1@0 |  |
| 2 | n41 | HIGH | n79 | LOW | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@272 |  |  | 1@259 |  |
| 3 | n41 | LOW | n79 | MID | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@272 |  |  | 1@150 |  |
| 4 | n41 | HIGH | n79 | LOW | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 5 | n41 | HIGH | n79 | LOW | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@129 |  |  | 1@0 |  |
| 6 | n41 | HIGH | n79 | LOW | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 7 | n41 | HIGH | n79 | HIGH | 273@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@272 |  |  | 1@272 |  |
| 8 | n41 | HIGH | n79 | MID | 217@30kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@216 |  |  | 1@133 |  |
| Test Settings for CA_n48A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | High | n66 | Low | 216@15kHz |  |  | 216@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n48A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | High | n70 | High | 216@15kHz |  |  | 79@15kHz | QPSK |  | NA |  |  | QPSK / CP-OFDM QPSK |  | 1@215 |  |  | 1@78 |  |
| 2 | n48 | High | n70 | Low | 216@15kHz |  |  | 79@15kHz | QPSK |  | NA |  |  | QPSK / CP-OFDM QPSK |  | 1@215 |  |  | 1@0 |  |
| Test Settings for CA_n48A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | HIGH | n71 | LOW | 106@15kHz |  |  | 106@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 2 | n48 | LOW | n71 | HIGH | 216@15kHz |  |  | 106@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@105 |  |
| Test Settings for CA_n66A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | Low | n71 | Low | 216@15kHz |  |  | 106@15kHz | QPSK/CP-OFDM QPSK |  | NA |  |  | QPSK / CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| Test Settings for CA_n66A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | LOW | n77 | 3825,24 MHz (NOTE 12) | 106@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 2 | n66 | LOW | n77 | HIGH | 216@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@151 |  |
| 3 | n66 | LOW | n77 | 4065,18 MHz (NOTE 13) | 79@15kHz |  |  | 52@15kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| 4 | n66 | LOW | n77 | HIGH | 216@15kHz |  |  | 217@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@48 |  |
| 5 | n66 | LOW | n77 | HIGH | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@217 |  |
| 6 (NOTE 14) | n66 | LOW | n77 | HIGH | 79@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@214 |  |
| 7 | n66 | LOW | n77 | HIGH | 216@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@211 |  |
| 8 | n66 | LOW | n77 | MID | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@0 |  |  | 1@13 |  |
| 9 | n66 | HIGH | n77 | MID | 106@15kHz |  |  | 273@30kHz | CP-OFDM QPSK |  | NA |  |  | CP-OFDM QPSK |  | 1@105 |  |  | 1@24 |  |
| Test Settings for CA_n70A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n70 | Low | n71 | Low | 79@15kHz |  |  | 106@15kHz | QPSK/CP-OFDM QPSK |  | NA |  |  | QPSK / CP-OFDM QPSK |  | 1@0 |  |  | 1@0 |  |
| NOTE 1: No impact on the protected bands by 2nd and 3rd order IM products caused by the NR CA band combination. NOTE 2: Test Point ID 2 for CA_n1A-n78A and Band n78 is defined as a carrier with CBW=10MHz and centre frequency at 3421.14MHz (NR ARFCN=628067). NOTE 3: Test Point ID 2 for CA_n1A-n79A and Band n79 is defined as a carrier with CBW=40MHz and centre frequency at 4618.41MHz (NR ARFCN=707867). NOTE 4: Test Point ID 4 for CA_n1A-n79A and Band n79 is defined as a carrier with CBW=40MHz and centre frequency at 4785.54MHz (NR ARFCN=719000). NOTE 5: Test Point ID 2 for CA_n2A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3850.23MHz (NR ARFCN=656667). NOTE 6: Test Point ID 3 for CA_n2A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3965.25MHz (NR ARFCN=664333). NOTE 7: Test point ID 5 and 6 for CA_n3A-n41A test protected NR band 100 introduced in Rel-17. NOTE 8: Test Point ID 3 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3458.79MHz (NR ARFCN=630533). NOTE 9: Test Point ID 4 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3593.79MHz (NR ARFCN=639533). NOTE 10: Test Point ID 5 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 4021.89MHz (NR ARFCN=668067). NOTE 11: Test Point ID 7 for CA_n5A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3538.29MHz (NR ARFCN=635867). NOTE 12: Test Point ID 1 for CA_n66A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3825.24MHz (NR ARFCN=655000). NOTE 13: Test Point ID 3 for CA_n66A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 4065.18MHz (NR ARFCN=671000). NOTE 14: Test point ID 6 for CA_n66A-n77A tests protected E-UTRA band 103 introduced in Rel-17. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] subclause4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.3.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.3.2.1.4.3.

< End of changes >