3GPP TSG-RAN5 Meeting #99	R5-233362

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2365 | rev | 1 | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to TRS based UE Rx-Tx measurement FR1 SA test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TRS based UE Rx-Tx measurement test case has several missing parts and need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated core requirements. - Completed the editor’s notes. - Added TTs to test case 6.6.21. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case implementation will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.21, F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.903 CR 0523 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Corrected the missing clauses based on TS 38.133 |  |  |  |  |  |  |  |  |

## 6.6	Measurement procedures

-------- Start of First change --------

### 6.6.21	UE Rx-Tx time difference measurement for propagation delay compensation with TRS

##### 6.6.21.0	Minimum conformance requirements

[38.133 clause 9.12.4.2, 9.12.5 and 10.1.25.3.1] for measurement time

When UE is configured with csi-RS-Ref-r17 in MeasObjectRxTxDiff-r17 defined in TS 38.331 [2] and provided with TRS resource configuration, the UE shall be able to measure UE Rx-Tx time difference on PCell within the measurement period TUERx-Tx_TRS, where:

TUERxTx,TRS=Nsample *Klayer3*LCM(TTRS, TDRX)

Where

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample is [4],

TTRS is the TRS resource periodicity specific for RTT-based propagation delay compensation,

TDRX is the DRX cycle length when DRX is in use, 1ms otherwise.

Klayer3 is

Ntotal / Navailable when Navailable>0

Where,

-	For a window W of duration LCM(TTRS, MGRP_max, TSMTC) and starting at the beginning of any TRS resource occasion, where TSMTC is the periodicity of SMTC for intra-frequency measurement without gap, and MGRP_max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as PCell for a UE supporting concurrentMeasGap-r17and is configured with concurrent measurement gaps, otherwise MGRP_max is the MGRP of configured per-UE measurement gap or per-FR measurement gap within the same FR as PCell:

-	Ntotal is the total number of TRS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Navailable is the number of TRS resource occasions that are not overlapped with

-	For FR1: any measurement gap occasions after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.8.3 within the window W

No requirements shall apply if Navailable=0.

UE is only required to perform UE Rx-Tx time difference on TRS within the active DL BWP. When UE is configured to perform UE Rx-Tx time difference measurement based on TRS, the requirements apply provided that the SCS of the TRS is same as that of the active BWP on PCell.

If TRS resources overlap with Type 1A/1B/2 PPW, the UE is allowed longer measurement period to measure UE Rx-Tx time difference on PCell.

The UE Rx-Tx time difference measurement values contained in measurement report shall be based on the measurement report mapping requirements specified in below.

The reporting range for the absolute UE Rx-Tx time difference measurement (TUE Rx-Tx) is defined from -985024×Tc to 985024×Tc with the resolution step of 2k×Tc.

Table 6.6.21.0-1: Absolute UE Rx-Tx time difference measurement report mapping for k=5

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -984992 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -984992  TUE Rx-Tx < -984960 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_30782 | -32  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_30783 | 0  TUE Rx-Tx < 32 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_61563 | 984960  TUE Rx-Tx < 984992 | Tc |
| RX-TX_TIME_DIFFERENCE_61564 | 984992  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_61565 | 985024  TUE Rx-Tx | Tc |

[TS 38.133 clause 10.1.39.3] for measurement accuracy

The error in the reported value of UE Rx-Tx time difference measurement, including both the measurement error and the reporting quantization error, should be within the accuracy requirements specified in this clause.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall not apply, if:

-	NTA_offset defined in Table 7.1.2-2 changes during the UE Rx-Tx measurement period or

-	if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the network-configured Timing Advance.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that:

-	The UE transmits SRS within [-160, 160] msec of at least one PDC TRS resource from the serving cell (PCell).

When a serving cell change occurs during the UE Rx-Tx measurement period, the UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx measurement.

The accuracy requirements in Table 6.6.21.0-2 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to Annex [TBD] for a corresponding Band.

AWGN propagation condition.

Table 6.6.21.0-2: UE Rx-Tx time difference measurement accuracy in FR1 in AWGN

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | TRS Ês/Iot | Minimum TRS bandwidth | TRS SCS | NR operating band groupsNote 2 | IoNote 3 range |  |
|  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 4 | dB | RB | kHz |  | dBm / SCSTRS | dBm/BW |
| [116++ℇ] | -3 | ≥[24] | 15 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -120.5 |  |
|  |  |  |  | NR_TDD_FR1_C | -120 |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  | NR_FDD_FR1_G | -118 |  |
|  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
| [60++ℇ] |  | ≥[52] |  | Note 5 | NOTE 5 | NOTE 5 |
| [29++ℇ] |  | >[104] |  | Note 5 | NOTE 5 | NOTE 5 |
| [56++ℇ] |  | ≥[24] | 30 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -118 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -117.5 |  |
|  |  |  |  | NR_TDD_FR1_C | -117 |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -116.5 |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -116 |  |
|  |  |  |  | NR_FDD_FR1_F | -115.5 |  |
|  |  |  |  | NR_FDD_FR1_G | -115 |  |
|  |  |  |  | NR_FDD_FR1_H | -114.5 |  |
| [29++ℇ] |  | ≥[48] |  | NOTE 5 | NOTE 5 | NOTE 5 |
| [15++ℇ] |  | ≥[132] |  | NOTE 5 | NOTE 5 | NOTE 5 |
| [29++ℇ] |  | ≥[24] | 60 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -115 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -114.5 |  |
|  |  |  |  | NR_TDD_FR1_C | -114 |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -113.5 |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -113 |  |
|  |  |  |  | NR_FDD_FR1_F | -113.5 |  |
|  |  |  |  | NR_FDD_FR1_G | -113 |  |
|  |  |  |  | NR_FDD_FR1_H | -111.5 |  |
| [14++ℇ] |  | ≥[64] |  | NOTE 5 | NOTE 5 | NOTE 5 |
| [7++ℇ] |  | ≥[132] |  | NOTE 5 | NOTE 5 | NOTE 5 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: The Io is defined in TRS slots. The same Io range applies to TRS and non-TRS symbols. Io levels are different in TRS and non-TRS symbols within the same slot. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the TRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: is the margin determined from Table 10.1.39.3-3. NOTE 7: ℇ is the margin for reporting quantitization error and ℇ=16 Tc. |  |  |  |  |  |  |

Table 6.6.21.0-3: Margin for UE Rx-Tx time difference measurement accuracy in FR1

| Min(TRS BW, SRS BW) (RB) |  |  | Margin (Tc Note 1) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 24 | N/A | N/A | [160] |
| ≥ 52 | ≥ 24 | N/A | [80] |
| ≥ 104 | ≥ 48 | ≥ 24 | [56] |
| N/A | ≥ 132 | ≥ 64 | [24] |
| N/A | N/A | ≥ 132 | [24] |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and TRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |  |

##### 6.6.21.1	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA

6.6.21.1.1	Test purpose

The purpose of the test is to verify that the UE Rx-Tx measurement with TRS for RTT-based PDC meets the requirements specified in clause 9.12.4.2 for measurement delay and clause 10.1.X.2 for measurement accuracy in AWGN propagation condition in FR1 in standalone scenario.

6.6.21.1.2	Test applicability

This test applies to all types of NR UE release 17 onwards supporting rtt-BasedPDC-CSI-RS-ForTracking-r17 or gNB-SideRTT-BasedPDC-r17.

6.6.21.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.21.0.

The normative reference for this requirement is TS 38.133 [6] clauses 9.12.4.2, 9.12.5, 10.1.39.3 and A.6.6.21.

6.6.21.1.4	Test description

6.6.21.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.21.1.4.1-1

Table 6.6.21.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.21-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.21-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.21-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.21.1.4.1-2.

Table 6.6.21.1.4.1-2: Initial conditions for UE Tx-Rx time difference measurement for FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.21.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |

1. The general test parameter settings are set up according to Table 6.6.21.1.4.1-3.

2. Message contents are defined in clause 6.6.21.1.4.3.

3. The test scenario comprises of 1 NR Cell. The connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.21.1.4.1-3: General test parameters

| Parameter | Unit | Test configuration | Value |
| --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |
| RF Channel Number |  | 1, 2, 3 | 1 |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |
|  |  | 2 | 10: NRB,c = 52 |
|  |  | 3 | 40: NRB,c = 106 |
| SSB configuration |  | 1 | SSB.1 FR1 |
|  |  | 2 | SSB.1 FR1 |
|  |  | 3 | SSB.2 FR1 |
| SMTC configuration |  | 1 | SMTC.2 |
|  |  | 2 | SMTC.1 |
|  |  | 3 | SMTC.1 |
| CP length |  | 1, 2, 3 | Normal |
| DRX |  | 1, 2, 3 | OFF |
| T1 | s | 1, 2, 3 | 1 |
| T2 | s | 1, 2, 3 | 1 |

6.6.21.1.4.2	Test procedure

The test is considered with one cell (Cell 1) in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 does not have TRS transmission during T1 and transmits TRS during T2.

The measurement control information with MeasObjectRxTxDiff set to ‘csi-RS-Ref’ as defined in table 6.6.21.1.4.3-1, shall be provided to the UE during T1. The last TTI containing the RRC message shall be provided to the UE T ms before the start of T2, where T = 10 ms is the maximum processing time of the measurement request. The UE is configured to transmit SRS during T2.

The general and cell specific test parameters for PCell is given in Table 6.6.21.1.4.1-3 and Table 6.6.21.1.5-1, respectively. The test consists of two sub-tests with different TRS BW.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.21.1.5-1 as appropriate.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.21.1.5-1. T2 starts.

6. The UE shall transmit periodically MeasurementReport messages.

7. After 10s wait from Step 3, the SS shall check the result-k5-r17 reported values in the periodic MeasurementReport. If the result-k5 value is outside the limits in Table 6.6.21.0-2 or the UE fails to report the measurement value for serving cell, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

8. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6.6.21.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table 4.6.3-76B in 38.508-1 [14] with condition CSI-RS_REF |
| Specific message contents exceptions for Test Configuration 6.6.21-1 and 6.6.21-2 | Table 4.6.3-84AC in 38.508-1 [14] with condition SCS15 |
| Specific message contents exceptions for Test Configuration 6.6.21-3 | Table 4.6.3-84AC in 38.508-1 [14] with condition SCS30 |

6.6.21.1.5	Test requirements

Table 6.6.21.1.4.1-3 and Table 6.6.21.1.5-1 define the primary level settings including test tolerances for UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA.

Table 6.6.21.1.5-1: Cell specific test parameters

| Parameter | Unit | Test configuration | Values |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | N/A |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  |
| SRS configuration |  | 1 | PDC-SRS.1 |  |
|  |  | 2 | PDC-SRS.1 |  |
|  |  | 3 | PDC-SRS.2 |  |
| Note 2 | dBm/SCS | 1 | -98+TT |  |
|  |  | 2 | -98+TT |  |
|  |  | 3 | -95+TT |  |
| Note 2 | dBm/15 kHz | 1 | -98+TT |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| TRS | dB | 1 | -Infinity | -3+TT |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| TRS | dB | 1 | -Infinity | -3+TT |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| TRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101+TT |
|  |  | 2 | -Infinity | -101+TT |
|  |  | 3 | -Infinity | -98+TT |
| Io | dBm/9.36 MHz | 1 | N/A | -68.28+TT |
|  | dBm/9.36 MHz | 2 |  | -68.28+TT |
|  | dBm/38.16 MHz | 3 |  | -62.19+TT |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: TRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table 6.6.21.1.5-2: SRS Configuration for FR1 Propagation Delay Compensation

|  | PDC-SRS.1 | PDC-SRS.2 |  |
| --- | --- | --- | --- |
| Field |  |  | Comment |
| c-SRS | Same as NRB,c in the test case | Same as NRB,c in the test case |  |
| b-SRS | 0 | 0 |  |
| b-hop | 0 | 0 | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | neither | No group or sequence hopping |
| freqDomainPosition | 0 | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | 0 | SSB #0 is used for SRS path loss estimation |
| usage | usagePDC-r17 | usagePDC-r17 |  |
| startPosition | 5 | 5 | resourceMapping setting |
| nrofSymbols | 4 | 4 |  |
| repetitionFactor | n.a. | n.a. | without repetition. |
| transmissionComb | n4 | n4 |  |
| combOffset-n2 | 0 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 | 0 |  |
| nrofSRS-Ports | port1 | port1 | Number of antenna ports used for SRS transmission |
| resourceType | Periodic | Periodic |  |
| periodicityAndOffset-p | sl160, 20 | Sl320, 40 | SRS transmission periodicity |

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 6.6.21.0. The UE shall be able to report the Rx-Tx time difference measurement results no later than TBD ms from the beginning of T2.

The UE Rx-Tx time difference measurement time fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 6.6.21.0.

For the test to pass, the ratio of successful reported values shall be more than 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

----- End of First change ------

----Unchanged Clauses Skipped -----

-------- Start of Second change --------

# F.1	Measurement uncertainties and test tolerances for FR1 and FR2

### F.1.1.2	Measurement of RRM requirements

----Unchanged Clauses Skipped -----

Table F.1.1.2-4 Maximum test system uncertainty for RRM requirements for SA FR1 test cases

------ Start of Second change ------

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.6.15.1 NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on LTE freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.6.21.1 UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB meas PRB Ês1 / Noc ±0.3 dB meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês1 / Noc is the cell 1 SNR Ês2 / Noc is the cell 2 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |  |
| 6.7.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB meas PRB Ês1 / Noc1 ±0.3 dB meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the cell 2 SNR Ês2 / Noc2 is the cell 3 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.7.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 6.7.2.1 NR SA FR1 SS-RSRQ measurement accuracy | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 6.7.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 6.7.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |

----- End of Second change ------

----Unchanged Clauses Skipped -----

------ Start of Third change ------

## F.1.3	Test Tolerance and Derivation of Test Requirements (informative)

### F.1.3.2	Measurement of RRM requirements

Table F.1.3.2-2: Derivation of test requirements for NR SA FR1 RRM tests

| Test |  | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- | --- |
| 6.6.15.1 NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: -3dB During T4: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +8dB During T5: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB Thresh_X_HighP 48dB Reported SS-RSRP and SS-RSRQ |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB During T4: 0dB 0dB 0dB 0dB During T5: 0dB 0dB 0dB 0dB +4dB Via Mapping | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: -3dB During T4: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +8dB During T5: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB 52dB Configuration 1 and 2: Cell 1: RSRP_52 to RSRP_75 RSRQ_54 to RSRQ_71 Cell 2: RSRP_39 to RSRP_62 RSRQ_6 to RSRQ_27 Configuration 3: Cell 1: RSRP_55 to RSRP_78 RSRQ_54 to RSRQ_71 Cell 2: RSRP_39 to RSRP_62 RSRQ_6 to RSRQ_27 |
| 6.6.21.1 UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA |  | TEST CONFIGURATION 1, 2 |  |  |
|  |  | During T1: Noc : -98 dBm/15kHz Ês / Noc: infinity During T2: Noc: -98 dBm/15kHz Ês / Noc: -3 dB | During T1: 0 dB 0 dB During T2: 0 dB 0 dB | During T1: Noc : -98 dBm/15kHz Ês / Noc: infinity During T2: Noc: -98 dBm/15kHz Ês / Noc: -3 dB |
|  |  | TEST CONFIGURATION 3 |  |  |
|  |  | During T1: Noc: -95 dBm/30kHz Ês / Noc: infinity During T2: Noc1: -95 dBm/30kHz Ês1 / Noc1: -3 dB | During T1: 0 dB 0 dB During T2: 0 dB 0 dB | During T1: Noc: -95 dBm/30kHz Ês / Noc: infinity During T2: Noc1: -95 dBm/30kHz Ês1 / Noc1: -3 dB |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy |  | TEST CONFIGURATION 1, 2, |  |  |
|  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping | Test 1: Noc: -107.5dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_34 to RSRP_46 RSRP_34 to RSRP_46 RSRP_35 to RSRP_47 RSRP_35 to RSRP_47 RSRP_36 to RSRP_48 RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |
|  |  | TEST CONFIGURATION 3 |  |  |
|  |  | Test 1: Noc: -110dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 1: -0.8dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping | Test 1: Noc: -110.8dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_41 to RSRP_53 Test 2: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 RSRP_38 to RSRP_50 RSRP_38 to RSRP_50 RSRP_39 to RSRP_51 RSRP_40 to RSRP_52 RSRP_40 to RSRP_52 depending on operating band |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |  |

----- End of Third change ------