# Title:	Test Tolerances for UE UL carrier RRC reconfiguration delay test case

# Source:			Nokia, Nokia Shanghai Bell

# 1.	Introduction

The purpose of` this document is to present the methodology and principles used for the development of Test Tolerances of UE UL carrier RRC Reconfiguration delay for FR2 in TS 38.533.

The test case in question is:

- 7.5.11.1, UE UL carrier RRC reconfiguration delay

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Prototype test case in Annex A of TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v17.9.0 Annex A.

### A.7.5.11	UE UL carrier RRC reconfiguration Delay

#### A.7.5.11.1	UE UL carrier RRC reconfiguration Delay

##### A.7.5.11.1.1	Test Purpose and Environment

The purpose of this test is to verify that when the UE receives a RRC message implying NR UL carrier configuration, the UE shall be ready to start transmission on the newly configured carrier within the time limits specified in clause 8.4.2 and 8.4.3 for configuring and DE configuring, respectively. The test will also verify the interruption at UL carrier configuration requirements on PCell in clause 8.2.2.2.4.

There are two cells: FR2 PCell (Cell 1) on NR RF channel 1 and FR2 SCell (Cell 2) on NR RF channel 2. NR uplink is broadcast by ServingCellConfigCommonSIB. The test parameters for PCell and SCell are given in Table A.7.5.11.1.1-1, Table A.7.5.11.1.1-2 and Table A.7.5.11.1.1-3 below.  The test consists of three time periods, with duration of T1, T2 and T3 respectively. During time duration T1, NR uplink of Cell 2 is not configured to UE. At the start of T2, a NR uplink of Cell 2 is configured to UE through RRCReconfiguration, then UE shall start transmission on the NR uplink of Cell 2. At the start of T3, the NR uplink is released through RRCReconfiguration.

The test equipment also verifies that potential interruption of PCell due to UL carrier configuration on SCell is carried out within the correct time span and within the correct length by monitoring ACK/NACK sent in PCell.

Table A.7.5.11.1.1-1: Supported test configurations

| Configuration | PCell (Cell 1) | SCell (Cell 2) |
| --- | --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | DL and UL: 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.11.1.1-2: General test parameters for SA UE UL carrier RRC reconfiguration Delay for FR2

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2 | Two NR RF channels |
| Active cell |  | Config 1 | Cell 1: FR2 PCell Cell 2: FR2 SCell | PCell on RF channel number 1 FR2 SCell on RF channel number 2 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| Cell 2 timing offset to Cell 1 | s | Config 1 | 3 | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 5 |  |
| T3 | s | Config 1 | 5 |  |

Table A.7.5.11.1.1-3: NR Cell specific test parameters for SA UE UL carrier RRC reconfiguration Delay for FR2

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.3.15 |  |  |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  |  | Rough |  |  |
| Frequency Range |  |  | Config 1 | FR2 |  |  | FR2 |  |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | TDD |  |  | TDD |  |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  |
| Data RBs allocated |  |  | Config 1 | 66 |  |  | 66 |  |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | DLBWP.1.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | ULBWP.1.1 |  |  |
| OCNG Patterns |  |  | Config 1 | OP.1 |  |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| SSB configuration |  |  | Config 1 | SSB.1 FR2 |  |  | SSB.1 FR2 |  |  |
| SMTC configuration |  |  | Config 1 | SMTC.1 |  |  | SMTC.1 |  |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  |  | TRS.2.1 TDD |  |  |
| TCI state |  |  | Config 1 | TCI.State.0 |  |  | TCI.State.0 |  |  |
| PUSCH parameters for NR UL carrier |  |  | Config 1 | N/A |  |  | N/A | G-FR2-A3-4 in [13] | N/A |
| PUCCH parameters for NR UL carriers |  |  | Config 1 | N/A |  |  | N/A | Table 11.3.2.3.1.2-2 in [13] | N/A |
| EPRE ratio of PSS to SSS |  |  | Config 1 | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 | Config 1 | -104.7 |  |  | -104.7 |  |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -95.7 |  |  | -95.7 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -88.7 |  |  | -88.7 |  |  |
|  |  | dB | Config 1 | 7 |  |  | 7 |  |  |
|  |  | dB | Config 1 | 7 |  |  | 7 |  |  |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.92 |  |  | -58.92 |  |  |
| Propagation Condition |  |  | Config 1 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |

##### A.7.5.11.1.2	Test Requirements

The UE shall be ready to start transmission on the NR uplink carrier on SCell within 20ms from the start of T2.

The UE shall stop the transmission on the NR uplink carrier on SCell within 20ms from the start of T3.

During T2 and T3, the start of interruption of PCell due to UL carrier configuration or de-configuration on SCell shall not happen outside the UL carrier configuration delay which is 20ms in this test, and the length of interruption shall not exceed the length specified in clause 8.2.2.2.4.

All of the above test requirements shall be fulfilled in order for the observed UE UL carrier configuration delay and UE UL carrier release delay to be counted as correct. The rate of correct observed UE UL carrier configuration delay and UE UL carrier release delay during repeated tests shall be at least 90%.

# 3. Discussion

The test consists of three time periods, with duration of T1, T2 and T3 respectively. During time duration T1, NR uplink of Cell 2 is not configured to UE. At the start of T2, a NR uplink of Cell 2 is configured to UE through RRCReconfiguration, then UE shall start transmission on the NR uplink of Cell 2. At the start of T3, the NR uplink is released through RRCReconfiguration.

The test equipment also verifies that potential interruption of PCell due to UL carrier configuration on SCell is carried out within the correct time span and within the correct length by monitoring ACK/NACK sent in PCell.

There is one test configuration according to Table A.7.5.11.1.1-1.

The design of the test case relies on the UE being able to start transmission on the NR uplink carrier on SCell within 20ms from the start of T2. The UE shall stop the transmission on the NR uplink carrier on SCell within 20ms from the start of T3.

A detailed analysis of the test case is given in section 5 of this document.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Tables A.7.5.11.1.1-1 and A.7.5.11.1.1-3 in TS 38.133. Key parameters relevant to the FR2 test case are copied from TS 38.101-2 and TS 38.101-4. These parameters are then used to calculate the UE internal noise which will then be used to calculate the Es/Iot at UE baseband. Both Spherical coverage and beam peak direction parameters are included but Spherical coverage parameters are “greyed out” as this test case is specified for beam peak direction only.

The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

For this test, the key parameters are:

•	Noc

•	Cell 1 Es / Noc

•	Cell 2 Es / Noc

The key parameters appear in section a) of the accompanying spreadsheet. AWGN is given a separate set of columns. This allows the spreadsheet calculations to be done in a consistent way.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

Note that there is negligible difference in the way RAN4 calculates the value. Applied Io in the excel sheet is calculated as nominal value in Ch BW at reference point resulting the value -58.89 dBm/100 kHz. TS 38.133 specification value is defined as -58.92 dBm/95.04 MHz.

c) Uncertainties

The Test system provides 2 cells on 2 frequencies, with AWGN. We propose to control the following parameters:

- AWGN absolute power on carrier frequency, Noc uncertainty: ±5.65 dB

- Ratio of cell 1 signal / AWGN, Ês1 / Noc ±0.3 dB averaged over BWConfig

- Ratio of cell 2 signal / AWGN, Ês2 / Noc ±0.3 dB averaged over BWConfig

UE SS-RSRP relative accuracy and UE gain G variation are also included. The uncertainties appear in section c) of the accompanying spreadsheet.

d) Controlled parameters critical to verdict

In this test case there are two aspects to consider:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE response (SS-RSRP reports)

The stimulus set by the Test system should be within the constraints (side conditions) for the UE measurement. For each cell they are the Es/Iot range, the SS-RSRP power range, and the Io power range, over which the UE meets the specified SS-RSRP reporting accuracy.

Controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given in the “Requirement” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. In some RRM test case there is usually a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict. The sensitivity factors are therefore derived by inspection as one or zero.

In some cases, the sensitivity factor is an intermediate value. For example, the Cell 1 Es/Noc depends on freq1 absolute power level and UE internal noise. In such cases a sensitivity factor value is defined between 0 and 1. It is important to calculate these correctly to obtain the overall uncertainty.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903. Uncertainties are calculated separately for T1, T2 and T3.

The normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE SS-RSRP reporting accuracy are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE SS-RSRP reporting accuracy. If all the uncertainties were combined root-sum square, the resulting smaller test limits could cause a conformant test system to fail a conformant UE, which would be unacceptable.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

It is concluded to use zero as a value for test tolerances.

Note that there is negligible difference in the way RAN4 calculates the value for Io. Applied Io in the excel sheet is calculated as nominal value in Ch BW at reference point resulting the value -58.89 dBm/100 kHz. TS 38.133 specification value is defined as -58.92 dBm/95.04 MHz. Because test tolerance is not applied and difference is small, it is proposed to use TS 38.133 specification value to be in line with the value in RAN4 specification.

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated, as some would have changed from the original if the Test Tolerances were applied in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (Es/Iot range, RSRP power range, and the Io power range). It is not necessary to calculate all parameters during each test, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all the stimulus requirements are met, then the chosen stimulus offsets are acceptable.

With the uncertainty values and Test Tolerances proposed, the requirements are met.

For this test, the verdict is based on the SS-RSRP values reported by the UE. As stated earlier, the reported values are affected by:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE response (SS-RSRP reports)

Having ensured that the stimulus set by the Test system remains within the side conditions, and knowing its uncertainty, we now need to calculate the range of SS-RSRP values of NR Cell 1 and NR Cell 2 that a conformant UE could report.

# 5. Treatment of related test cases

N/A