3GPP TSG RAN WG5 Meeting #99		 R5-233673

# Incheon, South Korea, 22nd – 26th May 2023

Title:	LS to RAN4 on A-MPR Network Signaling value (NS_02N)

Response to:

Release:	Release-18

Work Item:	LTE_NBIOT_eMTC_NTN_req-UEConTest

Source:	TSG RAN WG5

To:	TSG RAN WG4

Cc:	TSG RAN WG2

Contact person:	Danbo Fu

<Danbo.fu@mediatek.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

In TS 36.102 clause 6.2, RAN4 has defined the additional maximum power reduction(A-MPR) requirements, Network signaling value NS_02N has been introduced in TS 36.102, based on which RAN5 derives specific message content, However RAN5 misses the mapping of NS_02N value to the value IE additionalSpectrumEmission defined in TS 36.331

Taking Band 255 A-MPR Network signaling value NS_02N for category M1 UE as an example,

Table 6.2A.3-1: Additional Maximum Power Reduction (A-MPR) for category M1 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | Table 5.3A-1 | N/A |
| NS_02N | 6.5A.4.4.2 | 255 | Table 5.3A-1 | N/A |
| NS_24 | 6.5A.4.4.3 | 256 | Table 5.3A-1 | ≤ 3.5 |

RAN5 set IE additionalSpectrumEmission as 2N (NS_02N) in the systeminformationblockType2 as part of the cell broadcast message, this exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario

Table 6.2A.3.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for “NS_02N”

| Derivation Path: TS 36.508 [12] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2N |  |  |

But IE AdditionalSpectrumEmission value was defined as Integer in TS 36.331 clause 6.3.4

AdditionalSpectrumEmission information element

-- ASN1START

AdditionalSpectrumEmission:: =		INTEGER (1..32)

AdditionalSpectrumEmission-v10l0:: =	INTEGER (33..288)

-- ASN1STOP

The confirmation about NS_02N value below from RAN4 is necessary.

Table 6.2A.3.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for “NS_02N”

| Derivation Path: TS 36.508 [12] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | Whether 2N could be changed to 2 |  |  |

2	Actions

To TSG RAN WG4

ACTION: 	RAN5 kindly requests RAN4 to

To clarify which INTEGER value should be used for Network signalling value NS_02N in TS 36.102.

3	Dates of next TSG RAN WG5 meetings

TSG RAN WG5 Meeting #99			May 22 – 26, 2023			Incheon, KR

TSG RAN WG5 Meeting #100		       Aug. 21 – 25, 2023			Toulouse, FR