3GPP TSG-RAN5 Meeting #99	R5-233688

# Incheon, Korea, May 22nd – 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0711 | rev | 1 | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of minimum test time for HST scheme A and B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PDSCH test cases for HST SFN scheme A and B are introduced by RAN4. The minimum test time for legacy HST SFN scenario has been analysed and captured in Annex G of 38.521-4. The difference of legacy HST SFN and SFN scheme A and B could be summarized as below: HST SFN; HST SFN scheme A; HST SFN scheme B. RRH simulated; 4; 2; 2. Ds; 700 m. Dmin; 150 m. v; 500 km/h. fd; 870 Hz for 15 kHz SCS test; 1667 Hz for 30 kHz SCS test. TRS; SFN manner; TRP specific manner; TRP specific manner. PDCCH/PDSCH; SFN manner; SFN manner; SFN manner. Doppler precompensation; No; No; Yes. From the above tables we could tell: - The maximum doppler frequency is the same across three scenarios. - HST SFN scheme A and B has fewer working RRHs, which reduces the channel fluctuation. - In HST SFN scheme B, the two RRHs transmit PDCCH/PDSCH with the same doppler frequency, which further reduces the fluctuation. Based on above observations, the minimum test time needed for SFN scheme A and B is expected to be no longer than legacy HST SFN test cases. The exiting MNAS (Minimum number of active subframes) could be reused. The brackets are added to the MNAS value for further check. |
| Summary of change: |  | Adding the minimum test time for test cases for HST SFN scheme A and B. The required minimum number of active frames followed that of HST-SFN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The minimum test time is not decided. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-233053 |  |  |  |  |  |  |  |  |

<Start of Changes>

G.1.5	Minimum Test time

Editor's Note: Simulation method to derive minimum test time for FR2 needs to be evaluated.

If a pass fail decision in clause G.1.4 can be achieved earlier than the minimum test time, then the test shall not be decided, but continued until the minimum test time is elapsed.

The tables below contain the minimum number of slots for FDD and TDD.

By simulations the minimum number of active subframes (carrying DL payload) was derived (MNAS), then adding inactive subframes to the active ones. (for TDD additional subframes contain no DL payload), then rounding up to full thousand.

Simulation method to derive minimum test time:

With a level, corresponding a throughput at the test limit (here 30 % or 70 % of the max. throughput) the preliminary throughput versus time converges towards the final throughput. The allowance of ± 0.2 dB around the above mentioned level is predefined by RAN5 to find the minimum test time. The allowance of ±0.2 dB maps through the function "final throughput versus level" into a throughput corridor. The minimum test time is achieved when the preliminary throughput escapes the corridor the last time. The two functions "final throughput versus level" and "preliminary throughput versus time" are simulation results, which are done individual for each demodulation scenario.

Figure G.1.5-1: Simulation method to derive minimum test time

Table G.1.5-1: Minimum Test time for PDSCH demodulation

| TDD UL-DL pattern |  | Reference Channel |  | Propagation condition |  | Demodulation scenario (doppler speed) |  | Minimum number of active subframes (MNAS) |  | MNAS to MNS Scaling factor (Note 3) |  | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NA |  | R.PDSCH.1-8.1 FDD |  | HST-750 |  | 750 Hz |  | 6000 (Note 1) |  | 1.0526 |  | 6400 |  |
| NA |  | R.PDSCH.1-8.2 FDD |  | HST-972 |  | 972 Hz |  | 6000 (Note 1) |  | 1.0526 |  | 6400 |  |
| NA |  | R.PDSCH.1-8.1 FDD |  | TDLC300-600 |  | 600 Hz |  | 8000 (Note 1) |  | 1.0526 |  | 9000 |  |
| NA |  | R.PDSCH.1-1.1 FDD |  | TDLB100-400 |  | 400 Hz |  | 10000 (Note 1) |  | 1.0526 |  | 11000 |  |
| NA |  | R.PDSCH.1-1.1 HD-FDD |  | TDLB100-400 |  | 400 Hz |  | 10000 (Note 1) |  | 1.3333 |  | 14000 |  |
| NA |  | R.PDSCH.1-1.2 FDD, R.PDSCH.1-2.1 FDD, R.PDSCH.1-5.1 FDD |  | TDLC300-100 |  | 100 Hz |  | 20000 (Note 1) |  | 1.0526 |  | 22000 |  |
| NA |  | R.PDSCH.1-1.2 HD-FDD |  | TDLC300-100 |  | 100 Hz |  | 20000 (Note 1) |  | 1.3333 |  | 27000 |  |
| NA |  | R.PDSCH.1-1.3 FDD, R.PDSCH.1-2.2 FDD, R.PDSCH.1-2.3 FDD, R.PDSCH.1-2.4 FDD, R.PDSCH.1-2.5 FDD, R.PDSCH.1-3.1 FDD, R.PDSCH.1-3.2 FDD, R.PDSCH.1-3.3 FDD, R.PDSCH.1-3.4 FDD, R.PDSCH.1-4.1 FDD, R.PDSCH.1-12.1 FDD, R.PDSCH.2-1.1 FDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.0526 |  | 79000 |  |
| NA |  | R.PDSCH.1-1.3 HD-FDD, R.PDSCH.1-1.4 HD-FDD, R.PDSCH.1-1.5 HD-FDD, R.PDSCH.1-2.1 HD-FDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.3333 |  | 100000 |  |
| NA |  | R.PDSCH.1-7.1 FDD, R.PDSCH.1-7.2 FDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.25 |  | 94000 |  |
| NA |  | R.PDSCH.1-8.3 FDD, R.PDSCH.1-13.1 FDD, R.PDSCH.1-13.2 FDD, R.PDSCH.1-13.3 FDD, R.PDSCH.1-13.4 FDD, R.PDSCH.1-13.5 FDD, R.PDSCH.1-14.1 FDD, R.PDSCH.1-14.2 FDD, R.PDSCH.1-14.3 FDD, R.PDSCH.1-14.4 FDD |  | HST-SFN |  | 870 Hz |  | 30000 (Note 1) |  | 1.0526 |  | 32000 |  |
| NA |  | R.PDSCH.1-8.5 FDD |  | HST-SFN Scheme A HST-SFN Scheme B |  | 870 Hz |  | [30000 (Note 1)] |  | 1.0526 |  | [32000] |  |
| NA |  | R.PDSCH.1-8.4 FDD, R.PDSCH.1-15.1 FDD, R.PDSCH.1-15.2 FDD, R.PDSCH.1-15.3 FDD, R.PDSCH.1-15.4 FDD, R.PDSCH.1-15.5 FDD, R.PDSCH.1-16.1 FDD, R.PDSCH.1-16.2 FDD, R.PDSCH.1-16.3 FDD, R.PDSCH.1-16.4 FDD |  | HST-DPS |  | 870 Hz |  | 30000 (Note 1) |  | 1.0526 |  | 32000 |  |
| FR1.15-1 |  | R.PDSCH.1-1.1 TDD, R.PDSCH.1-1.2 TDD |  | TDLA30-10 |  | 10Hz |  | 75000 (Note 1) |  | 2.8571 |  | 215000 |  |
| FR1.30-1A |  | R.PDSCH.2-1.1 TDD, R.PDSCH.2-1.5 TDD |  | TDLB100-400 |  | 400 Hz |  | 10000 (Note 1) |  | 1.2903 |  | 13000 |  |
| FR1.30-1 |  | R.PDSCH.2-1.2 TDD, R.PDSCH.2-2.1 TDD, R.PDSCH.2-7.1 TDD, R.PDSCH.2-26.1 TDD |  | TDLC300-100 |  | 100 Hz |  | 20000 (Note 1) |  | 1.2903 |  | 26000 |  |
| FR1.30-1 |  | R.PDSCH.2-2.2 TDD, R.PDSCH.2-2.3 TDD, R.PDSCH.2-2.4 TDD, R.PDSCH.2-2.5 TDD, R.PDSCH.2-3.1 TDD, R.PDSCH.2-3.2 TDD, R.PDSCH.2-3.3 TDD, R.PDSCH.2-3.4 TDD, R.PDSCH.2-3.5 TDD, R.PDSCH.2-4.1 TDD, R.PDSCH.2-4.3 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.2903 |  | 97000 |  |
| FR1.30-1 |  | R.PDSCH.2-1.3 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.4815 |  | 112000 |  |
| FR1.30-2 |  | R.PDSCH.2-5.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.2903 |  | 97000 |  |
| FR1.30-2 |  | R.PDSCH.2-17.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 5 |  | 375000 |  |
| FR1.30-3 |  | R.PDSCH.2-6.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.4815 |  | 112000 |  |
| FR1.30-4 |  | R.PDSCH.2-9.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.2903 |  | 97000 |  |
| FR1.30-5 |  | R.PDSCH.2-11.1 TDD |  | TDLB100-400 |  | 400Hz |  | 10000 (Note 1) |  | 1.2903 |  | 13000 |  |
| FR1.30-6 |  | R.PDSCH.2-12.1 TDD |  | TDLB100-400 |  | 400Hz |  | 10000 (Note 1) |  | 1.2903 |  | 13000 |  |
| FR1.30-1 |  | R.PDSCH.2-10.1 TDD |  | HST-1000 |  | 1000 Hz |  | 15000 (Note 1) |  | 1.4815 |  | 23000 |  |
| FR1.30-1 |  | R.PDSCH.2-10.1 TDD |  | HST-1667 |  | 1667 Hz |  | 15000 (Note 1) |  | 1.4815 |  | 23000 |  |
| FR1.30-1 |  | R.PDSCH.2-10.4 TDD, R.PDSCH.2-19.1 TDD, R.PDSCH.2-19.2 TDD, R.PDSCH.2-19.3 TDD, R.PDSCH.2-19.4 TDD, R.PDSCH.2-19.5 TDD, R.PDSCH.2-20.1 TDD, R.PDSCH.2-20.2 TDD, R.PDSCH.2-20.3 TDD, R.PDSCH.2-20.4 TDD, R.PDSCH.2-20.5 TDD, R.PDSCH.2-21.1 TDD |  | HST-SFN |  | 1667 Hz |  | 30000 (Note 1) |  | 1.4815 |  | 45000 |  |
| FR1.30-1 |  | R.PDSCH.2-10.5 TDD, R.PDSCH.2-22.1 TDD, R.PDSCH.2-22.2 TDD, R.PDSCH.2-22.3 TDD, R.PDSCH.2-22.4 TDD, R.PDSCH.2-22.5 TDD, R.PDSCH.2-23.1 TDD, R.PDSCH.2-23.2 TDD, R.PDSCH.2-23.3 TDD, R.PDSCH.2-23.4 TDD, R.PDSCH.2-23.5 TDD, R.PDSCH.2-24.1 TDD |  | HST-DPS |  | 1667 Hz |  | 30000 (Note 1) |  | 1.4815 |  | 45000 |  |
| FR1.30-1 |  | R.PDSCH.2-30.1 TDD |  | HST-SFN Scheme A HST-SFN Scheme B |  | 1667 Hz |  | [30000 (Note 1)] |  | 1.4815 |  | [45000] |  |
| FR2.60-1 |  | R.PDSCH.4-1.1 TDD |  | TDLA30-75 |  | 75 Hz |  | 20000 (Note 2) |  | 1.33 |  | 27000 |  |
| FR2.120-1A |  | R.PDSCH.5-1.1 TDD |  | TDLC60-300 |  | 300 Hz |  | 10000 (Note 2) |  | 1.25 |  | 13000 |  |
| FR2.120-1 |  | R.PDSCH.5-2.1 TDD, R.PDSCH.5-2.2 TDD, R.PDSCH.5-2.3 TDD, R.PDSCH.5-3.1 TDD |  | TDLA30-300 |  | 300 Hz |  | 10000 (Note 2) |  | 1.25 |  | 13000 |  |
| FR2.120-1 |  | R.PDSCH.5-1.2 TDD |  | TDLA30-75 |  | 75 Hz |  | 20000 (Note 2) |  | 1.25 |  | 25000 |  |
| FR2.120-2 |  | R.PDSCH.5-4.1 TDD, R.PDSCH.5-5.1 TDD, R.PDSCH.5-5.2 TDD, R.PDSCH.5-6.1 TDD |  | TDLA30-75 |  | 75 Hz |  | 20000 (Note 2) |  | 1.33 |  | 27000 |  |
| FR2.120-1 |  | R.PDSCH.5-10.1 TDD |  | TDLD30-75 |  | 75 Hz |  | 20000 (Note 2) |  | 1.26 |  | 26000 |  |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots. |  |  |  |  |  |  |  |  |  |  |  |  |  |

< End of Changes >