3GPP TSG-RAN5 Meeting #99	R5-233725

# Incheon, Korea, 22th– 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1606 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 6.2B.4.1.3 configured output power for EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For UEs supporting DPS, the test purpose of TP 1g is to verify NR carrier can reduce 6dB to ensure that total transmitted power would not exceed the core requirements. For PC2 UE, if UE indicates PC3 on NR band, the possible maximum total power would not exceed 26 dBm, then the test purpose of TP 1g can’t be achieved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a Note to clarify that TP 1g for PC2 UE is only applicabel to UEs indicate PC2 on NR band. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TP 1g may not be tested properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-232753r3 |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2B.4.1.3	Configured Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC)

6.2B.4.1.3.1	Test purpose

To verify the UE does not exceed the power bounds defined by PCMAX and PCMAX_H.

6.2B.4.1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC with 1 E-UTRA CC and 1 NR CC within FR1.

6.2B.4.1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2B.4.1.0.1.3.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

6.2B.4.1.3.4	Test description

6.2B.4.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2 and are shown in table 6.2B.4.1.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

0ons table for inter-band EN-DC

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] clause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [5] clause 4.3.1 E-UTRA Test Frequencies as specified in TS 36.508-1 [11] clause 4.3.1 |  |  | Mid range (NOTE 4) |  |  |  |  |
| Test EN-DC bandwidth combination as specified in TS 38.508-1 [5] clause 4.3.1 |  |  | 5MHz for E-UTRA CC1 and Lowest for NR CC1, Highest for E-UTRA CC1 and Highest for NR CC1 |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  | Highest |  |  |  |  |
| NR/E-UTRA Test Parameters for UE supporting DPS |  |  |  |  |  |  |  |
| Test ID (NOTE 3) | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |  |
|  |  | E-UTRA Cell |  |  | NR Cell |  |  |
|  |  | Modulation | RB allocation (NOTE 2) | PLTE | Modulation | RB allocation (NOTE 1) | PNR |
| 1a-1h | N/A | QPSK | Partial_Allocation | {-13, 12, 14, 17, 18, 19, 20, 23} for PC3 UE {-13, 12, 14, 17, 18, 19, 23X, 26} for PC2 UE | DFT-s-OFDM QPSK | Inner_Full | {-13, 12, 14, 17, 18, 19, 23, 23} for PC3 UE {-13, 12, 14, 17, 18, 19, 26X, 265 } for PC2 UE |
| 2a-2c |  | QPSK | Partial_Allocation | {-10, 10, 15} | N/A | N/A | {-10, 10, 15} |
| 3a-3c |  | N/A | N/A | {-10, 10, 15} | DFT-s-OFDM QPSK | Inner_Full | {-10, 10, 15} |
| NR/E-UTRA Test Parameters for UE not supporting DPS |  |  |  |  |  |  |  |
| Test ID (NOTE 3) | Downlink Configuration | EN-DC Uplink Configuration |  |  |  |  |  |
|  |  | E-UTRA Cell |  |  | NR Cell |  |  |
|  |  | Modulation | RB allocation (NOTE 2) | PLTE | Modulation | RB allocation (NOTE 1) | PNR |
| 1a-1d | N/A | QPSK | Partial_Allocation | {-10, 10, 15, 23} for PC3 UE {-10, 10, 15, 26} for PC2 UE | CP-OFDM QPSK | Inner_Full | {-10, 10, 15, 23} for PC3 UE {-10, 10, 15, 26} for PC2 UE |
| 2a-2c |  | QPSK | Partial_Allocation | {-10, 10, 15} | N/A | N/A | {-10, 10, 15} |
| 3a-3c |  | N/A | N/A | {-10, 10, 15} | DFT-s-OFDM QPSK | Inner_Full | {-10, 10, 15} |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 in TS 38.521-1 [8]. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1 in current specification. NOTE 3: The suffix in Test ID identifies the configured power level being tested from the PLTE and PNR lists NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequency. NOTE X: The test point is only applicable to Rel-15 UEs supporting PC2 on NR band. Test point is applicable for UEs Rel-16 and forward, supporting PC2 on both NR band and NR part power class, or supporting PC2 on NR band and not supporting NR part power class. The capability of NR part power class is indicated in TS 38.508-2 Table A.4.3.2B.2.3.1-3a. |  |  |  |  |  |  |  |

Table 6.2B.4.1.3.4.1-2: Void

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.2.1 for SS diagram and A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to Table 6.2B.4.1.3.4.1-1.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.4.1.3.4.3.

7.	For the case of testing overlapping E-UTRA and NR UL transmission scenario when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

6.2B.4.1.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according table 6.2B.4.1.3.4.1-1 on E-UTRA CC and NR CC respectively. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command for the UE to reach PUMAX level.

3.	Measure the mean transmitted power over E-UTRA component carrier and NR component carrier respectively, or/and measure the sum of mean transmitted power over E-UTRA and NR component carriers according to Table 6.2B.4.1.3.5-1 and Table 6.2B.4.1.3.5-2. The period of the measurement shall be at least the continuous duration of one active sub-frame. For TDD, only slots consisting of only UL symbols are under test. For FDD band in inter-band CA with both TDD band and FDD band, only slots overlapping with only UL symbols in TDD are under test.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2B.4.1.3.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2B.4.1.3.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6.1 and TS 38.508-1 [6] clause 4.6.1 with the following exceptions.

Table 6.2B.4.1.3.4.3-1: RRCConnectionReconfiguration: tdm-PatternConfig if E-UTRA on FDD band and UE doesn’t support dynamic power sharing

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| tdm-PatternConfig-r15 CHOICE{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| subframeAssignment-r15 | sa2 |  |  |
| harq-Offset-r15 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2B.4.1.3.4.3-2: SystemInfomationBlockType1: tdd-Config if E-UTRA on TDD band

| Derivation Path: TS 36.508 [11], Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  | Operating on TDD band |  |
| subframeAssignment | sa2 |  |  |
| specialSubframePatterns | ssp7 |  |  |
| } |  |  |  |

Table 6.2B.4.1.3.4.3-3: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | Defined as PLTE in Table 6.2B.4.1.3.4.1-1 |  |  |

Table 6.2B.4.1.3.4.3-4: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| p-NR-FR1 | Defined as PNR in Table 6.2B.4.1.3.4.1-1 |  |  |
| } |  |  |  |

Table 6.2B.4.1.3.4.3-5: RRCConnectionReconfiguration: p-MaxUE-FR1-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 |  | 23 |  | Power Class 3 UE |
|  |  | 26 |  | Power Class 2 UE |
| } |  |  |  |  |

6.2B.4.1.3.5	Test requirement

For UE supporting DPS, the output power measured shall not exceed the values specified in Table 6.2B.4.1.3.5-1.

Table 6.2B.4.1.3.5-1: PCMAX configured UE output power for UE supporting DPS

|  | E-UTRA component carrier | NR component carrier | Total power measured over E-UTRA and NR component carriers |
| --- | --- | --- | --- |
| Test ID 1a | Not measured | Not measured | -10dBm ± (7+TT) |
| Test ID 1b | Not measured | Not measured | 15dBm ± (6+TT) |
| Test ID 1c | Not measured | Not measured | 17 ± (5+TT) |
| Test ID 1d | Not measured | Not measured | 20 + (4+TT)/ - (6+TT) |
| Test ID 1e | Not measured | Not measured | 21 + (3+TT)/ - (5+TT) |
| Test ID 1f | Not measured | Not measured | 22+ (2+TT)/ - (5+TT) |
| Test ID 1g | Not measured | 23 + (2+TT) / 17 – (5+TT) for PC3 UE | 23 + (2+TT) / 21.8 - (5+TT) for PC3 UE |
|  |  | 26 + (2+TT) / 20 - (6+TT) for PC2 UE | 26 + (2+TT) / 24.8 - (3+TT) for PC2 UE |
| Test ID 1h | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] for PC3 UE. | N/A | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] for PC3 UE |
|  | Maximum output power with reduction as defined in Table 6.2.3_1.5-1 of TS 36.521-1 [10] for PC2 UE |  | Maximum output power with reduction as defined in Table 6.2.3_1.5-1 of TS 36.521-1 [10] for PC2 UE |
| Test ID 2a | -10dBm ± (7+TT) | N/A | Not measured |
| Test ID 2b | 10dBm ± (6+TT) | N/A | Not measured |
| Test ID 2c | 15dBm ± (5+TT) | N/A | Not measured |
| Test ID 3a | N/A | -10dBm ± (7+TT) | Not measured |
| Test ID 3b | N/A | 10dBm ± (6+TT) | Not measured |
| Test ID 3c | N/A | 15dBm ± (5+TT) | Not measured |
| NOTE 1: In addition NOTE 2 in Table 6.2.2-1 in TS 36.101 [5] and/or NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] shall apply to the tolerances. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.3.5-3 and Table 6.2B.4.1.3.5-4. |  |  |  |

For UE not supporting DPS, the output power measured shall not exceed the values specified in Table 6.2B.4.1.3.5-2.

Table 6.2B.4.1.3.5-2: PCMAX configured UE output power for UE not supporting DPS

|  | E-UTRA component carrier | NR component carrier | Total power measured over E-UTRA and NR component carriers |
| --- | --- | --- | --- |
| Test ID 1a | -10 dBm ± (7+TT) | -10 dBm ± (7+TT) | Not measured |
| Test ID 1b | 10 dBm ± (6+TT) | 10 dBm ± (6+TT) | Not measured |
| Test ID 1c | 15 dBm ± (5+TT) | 15 dBm ± (5+TT) | Not measured |
| Test ID 1d | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] for PC3 UE | N/A | Not measured |
|  | Maximum output power with reduction as defined in Table 6.2.3_1.5-1 of TS 36.521-1 [10] for PC2 UE |  |  |
| Test ID 2a | -10dBm ± (7+TT) | N/A | Not measured |
| Test ID 2b | 10dBm ± (6+TT) | N/A | Not measured |
| Test ID 2c | 15dBm ± (5+TT) | N/A | Not measured |
| Test ID 3a | N/A | -10dBm ± (7+TT) | Not measured |
| Test ID 3b | N/A | 10dBm ± (6+TT) | Not measured |
| Test ID 3c | N/A | 15dBm ± (5+TT) | Not measured |
| NOTE 1: In addition NOTE 2 in Table 6.2.2-1 in TS 36.101 [5] and/or NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] shall apply to the tolerances. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.3.5-3 and Table 6.2B.4.1.3.5-4. |  |  |  |

Table 6.2B.4.1.3.5-3: Test Tolerance for UE maximum output power (Separate measurements over E-UTRA and NR CCs)

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| E-UTRA | BW ≤ 20MHz | 0.7 dB | 1.0 dB | N/A |
| NR | BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2B.4.1.3.5-4: Test Tolerance for UE maximum output power (Combined measurements of E-UTRA and NR CCs)

| TT for overall output power |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NR |  |  |  |  |  |  |  |  |
|  |  |  | BW ≤ 20MHz |  |  | 20 MHz < BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| E-UTRA | BW ≤ 20MHz | f ≤ 3.0GHz | 0.7 dB | 1.0 dB | 1.0 dB | 0.7 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |

For the UE which supports inter-band EN-DC configuration, ΔTIB,c in 6.2B.4.2 applies where unless otherwise stated, the same ΔTIB,c is applicable to NR band(s) part for DC configurations which have the same NR operating band combination. Unless otherwise stated, ΔTIB,c is set to zero.

<End of Changes>