3GPP TSG RAN WG5 Meeting #2 5G-NR Adhoc	R5-182025

Taoyuan, Taiwan, 9 - 13 April 2018

Source:		Rohde & Schwarz

Title:	Discussion on test point selection for NR Frequency Error in FR1

Agenda Item:		4.1.16

Document for:	Endorsement

# Introduction

This contribution is a supplement to the test point analysis for Frequency error in FR1 [1].

# Discussion

Discussion paper [1] considers the selection of test environment, test frequency, channel bandwidth, subcarrier spacing and uplink configuration. In this discussion paper, we consider the modulation type. For the uplink configuration, we suggest a more straightforward approach.

In LTE, for frequency error only QPSK modulation is tested. As the modulation order has no influence on the frequency error, we follow the same approach in NR as in LTE and suggest QPSK as modulation. According to endorsed proposal in [2], CP-OFDM can be considered as the worst case for the waveform.

Proposal 1: Test CP-OFDM QPSK.

As the frequency error is not impacted by the allocation, we suggest outer_full allocation as a straightforward approach.

Proposal 2: Apply outer_full as RB allocation for the uplink.

For the downlink also a full allocation is proposed as the UE’s ability to synchronize its frequency to the downlink is indirectly tested in the REFSENS test case.

Proposal 3: Apply full allocation for the downlink configuration.

# Conclusion

In this paper, we have discussed additional aspects in the test point selection for the test case frequency error in FR1.

Proposal 1: Test CP-OFDM QPSK.

Proposal 2: Apply outer_full as RB allocation for the uplink.

Proposal 3: Apply full allocation for the downlink configuration.

# References

- R5-181781, Discussion on test point selection for NR Frequency Error in FR1, KDDI, 3GPP TSG RAN5, RAN5 5G-NR Adhoc #2, Taoyuan, Taiwan, April 2018

- R5-181929, Comparison of NR UL Signal CCDFs, Intel Deutschland GmbH, 3GPP TSG RAN5, RAN5 5G-NR Adhoc #2, Taoyuan, Taiwan, April 2018