GPP RAN5#3-5G-NR Adhoc	R5-185747

Jaipur, India, 8 – 12 October, 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0008 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating test case 6.2B.3.1 Additional Maximum Output Power reduction for Intra-band contiguous EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2018-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Updating Additional Maximum Output Power reduction for Intra-band contiguous EN-DC test case and aligning requirements with TS 38.101-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Text update for test case 6.2B.3.1 Additional Maximum Output Power reduction for Intra-band contiguous EN-DC Adding test configuration table for NS_35 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5GS WP will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

[Start of changes]

#### 6.2B.3	UE additional maximum output power reduction for EN-DC

##### 6.2B.3.1	UE Additional Maximum Output Power reduction for Intra-band contiguous EN-DC

Editor’s note: The following aspects are either missing or not yet determined:

- SA message contents in TS 38.508-1[6] subclause 4.6 is FFS.

- UE Power Class test requirements

- Testing with dynamic power sharing is FFS.

6.2B.3.1.1	Test purpose

Additional emission requirements can be signalled by the network with network signalling value indicated by the field additionalSpectrumEmission. To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2B.1.1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2B.3.1.2 	Test applicability

The requirements of this test apply in test case 6.5B.2.1.2 Additional spectrum emission mask for network signalled values NS_04 and NS_35 to all types of NR UE release 15 and forward, supporting intra-band contiguous EN-DC.

6.2B.3.1.3 	Minimum conformance requirements

For EN-DC band combinations with additional requirements the A-MPR allowed are specified in table 6.2B.3.1.3-1 for combinations of network signalling values indicated in E-UTRA and NR cell group(s). Unless otherwise stated the A-MPR allowed below is in addition to the MPR requirements specified in sub-clause 6.2B.2.1.

Table 6.2B.3.1.3-1: Allowed power reduction for intra-band contiguous EN-DC

| DC configuration | Requirement (sub-clause) | E-UTRA network signalling value | NR network signalling value | A-MPR |
| --- | --- | --- | --- | --- |
| DC_(n)71B | 6.5B.2.1.2.3.1 | NS_35 | NS_35 | 6.2B.3.1.3.1 |
| DC_(n)41AA1 | 6.5B.2.1.2.3.2 | NS_04 | NS_04 | 6.2B.3.1.3.2 |
| NOTE 1: Only applies to UEs that support dual UL transmission for this EN-DC combination. NOTE 2: The network signalling value for NR is mapped to configured FBI and AdditionalSpecrumEmission values as specified in [6]. |  |  |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.3.1.

6.2B.3.1.3.1		A-MPR for DC_(n)71B

6.2B.3.1.3.1.0	General

For DC_(n)71B with configured with network signalling values as per Table 6.2B.3.1.1-1 the allowed A-MPR is defined by

-	for UE indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE

where A-MPRDC is the total power reduction allowed (dB),

-	for OFDM:

MA,DC =	10.00 - 11.67*A;		0.00 < A ≤ 0.30

7.10 -  2.00*A;					0.30 < A ≤ 0.80

5.50;								0.80 < A ≤ 1.00

-	for DFT-S-OFDM:

MA,DC =	10.00 - 13.33*A;  	0.00 < A ≤ 0.30

7.00 - 3.33*A;   					0.30 < A ≤ 0.60

5.00;								0.60 < A ≤ 1.00

where

with LCRB and NRB the number of allocated PRB and transmission bandwidth for the respective CG,

-	for UE not indicating support of dynamicPowerSharing

where A-MPR is the total power reduction allowed per CG with

where ÑRB,NR is the transmission bandwidth configuration of the SCG channel for SCS = 15 kHz.

6.2B.3.1.3.2		A-MPR for NS_04

6.2B.3.1.3.2.0	General

When the UE is configured for B41/n41 intra-band contiguous EN-DC and it receives IE NS_04, the UE determines the total allowed maximum output power reduction as specified in this subclause. The A-MPR for EN-DC defined in this section is used instead of MPR defined in 6.2B.2.2, not additively.

The UE determines the Channel Configuration Case and the value of A-MPRIM3 as follows:

If FIM3,low_block,low < 2490.5 MHz

Channel Configuration Case B. A-MPRIM3 defined in subclause 6.2B.3.1.3.2.2.

Else

Channel Configuration Case A. A-MPRIM3 defined in subclause 6.2B.3.1.3.2.1.

where

-	FIM3,low_block,low = (2 * Flow_channel,low_edge) – Fhigh_channel,high_edge

-	Flow_channel,low_edge is the lowermost frequency of lower transmission bandwidth configuration.

-	Fhigh_channel,high_edge is the uppermost frequency of upper transmission bandwidth configuration.

The UE determines the total allowed maximum output power reduction as follows:

For UEs not supporting dynamic power sharing, with backoff applied independently

A-MPRE-UTRA = MAX( A-MPRsingle, E-UTRA, A-MPRIM3 )

A-MPRNR = MAX( A-MPRsingle,NR, A-MPRIM3 )

For UEs supporting dynamic power sharing, with IM# backoff applied equally to E-UTRA and NR

A-MPRE-UTRA = MAX( A-MPRsingle,E-UTRA, A-MPRIM3 )

A-MPRNR = MAX( A-MPRsingle,NR, A-MPRIM3)

where

-	A-MPRsingle, E-UTRA is the A-MPR defined for the E-UTRA transmission in [5]

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in [2]

6.2B.3.1.3.2.1	A-MPRIM3 for NS_04 to meet -13 dBm / 1MHz for 26dBm UE power

A-MPR in this sub-clause is relative to 26 dBm for power class 2. The same A-MPR is used relative to 23 dBm for power class 3. For the UE is configured with channel configurations Case A or Case C (defined in Subclause 6.2B.3.2.3.1), the allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

A-MPRIM3 = MA

Where MA is defined as follows

MA = 	14	;	 0 ≤ B < 0.5

9	;	 0.5 ≤ B < 1.0

7	; 	1.0 ≤ B < 2.0

5	; 	2.0 < B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

For NR

B = (12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

6.2B.3.1.3.2.2	A-MPR for NS_04 to meet -25 dBm / 1MHz for 26 dBm UE power

A-MPR in this sub-clause is relative to 26 dBm. The same A-MPR is used relative to 23 dBm for power class 3. For the UE is configured with channel configurations Case B or Case D, the allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

A-MPRIM3 = MA

Where MA is defined as follows

MA = 	14	; 	0 ≤ B < 1.0

13	; 	1.0 ≤ B < 2.0

12	; 	2.0 ≤ B < 5.0

11	; 	5.0 < B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc,E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

For NR

B = (LCRB_alloc,E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

6.2B.3.1.4	Test description

6.2B.3.1.4.1		Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2B.2.1-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2B.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex TS 36.521-1 [10] Annex C and in Annex C2 for LTE link and NR link respectively.

Table 6.2B.3.1.4.1-0: E-UTRA test configuration table

| E-UTRA Test Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA Channel Bandwidth | E-UTRA Test Frequency (Note 1) | Downlink | Uplink |  |
|  |  | N/A for A-MPR testing. | Modulation | RB allocation |
| 20 MHz | Low range and High range (Note 2) |  | QPSK | 100 |
| NOTE 1: E-UTRA Test Frequency as specified in TS 36.508 [6] subclause 4.3.1 NOTE 2: NR carrier shall be the outermost carrier during test. |  |  |  |  |

Table 6.2B.3.1.4.1-1: Test configuration table (network signalled value "NS_35")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [6] subclause 4.3.1 |  |  |  |  |  | Low range and High range (Note 1) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [6] subclause 4.3.1 |  |  |  |  |  | Lowest and Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest and Highest |  |
| A-MPR test parameters for “NS_35” |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq | ChBw | SCS | N/A for A-MPR testing. | Modulation |  | NR RB allocation |
| 1 | Low | Default | Default |  | DFT-s-OFDM 64 QAM |  | Outer_1RB_Left |
| 2 | High | Default | Default |  | DFT-s-OFDM 64 QAM |  | Outer_1RB_Right |
| 3 | Low | Lowest | Default |  | DFT-s-OFDM 64 QAM |  | Full |
| 4 | Low | Highest | Default |  | DFT-s-OFDM 64 QAM |  | Full |
| 5 | High | Lowest | Default |  | DFT-s-OFDM 64 QAM |  | Full |
| 6 | High | Highest | Default |  | DFT-s-OFDM 64 QAM |  | Full |
| NOTE 1: NR carrier shall be the outermost carrier during test. |  |  |  |  |  |  |  |

Table 6.2B.3.1.4.1-2: NR test configuration table for NS_04

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [6] subclause 4.3.1 |  |  |  |  |  | Low range and High range (Note 1) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [6] subclause 4.3.1 |  |  |  |  |  | Lowest and Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest and Highest |  |
| A-MPR test parameters for “NS_04” |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq | ChBw | SCS | N/A for A-MPR testing. | Modulation |  | NR RB allocation |
| 1 | Low | Default | Default |  | DFT-s-OFDM 64 QAM |  | Outer_1RB_Left |
| 2 | High | Default | Default |  | DFT-s-OFDM 64 QAM |  | Outer_1RB_Right |
| 3 | Low | Lowest | Default |  | DFT-s-OFDM 64 QAM |  | Full |
| 4 | Low | Highest | Default |  | DFT-s-OFDM 64 QAM |  | Full |
| 5 | High | Lowest | Default |  | DFT-s-OFDM 64 QAM |  | Full |
| 6 | High | Highest | Default |  | DFT-s-OFDM 64 QAM |  | Full |
| NOTE 1: NR carrier shall be the outermost carrier during test. |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of section 6.2B.3.1.4.1. As new tables are added to this section, these lines should always follow the tables.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex Cand uplink signals according to TS 36.521-1 [10] Annex H and Annex G for LTE link and NR link respectively.

4.	The UL Reference Measurement channels are set according to TS 36.521-1 [10] Annex A.2 and Annex A for LTE link and NR link respectively. 5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B and Annex Bfor LTE link and NR link respectively.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.3.1.4.3.

6.2B.3.1.4.2		Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format [0_1] for C_RNTI to schedule the UL RMC according to table 6.2B.3.1.4.1-1 on both EN-DC component carriers. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms for the UE to reach PUMAX level.

3.	Measure the mean power over all component carriers for the EN-DC configuration, which shall meet the requirements described in table 6.2B.3.1.5.1-1 thru 6.2B.3.1.5.2-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). [For TDD slots with transient periods are not under test.]

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.5B.2.1.2.4.1-1 and 6.5B.2.1.2.4.1-2, send an RRCConnectionReconfiguration message containing NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 table 4.6.3-89 PUSCH-Config without CP-OFDM condition. When switching to CP-OFDM waveform, send an RRCConnectionReconfiguration message containing NR RRCReconfiguration message with CP-OFDM condition.

6.2B.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] subclause 4.6.1, with the following exceptions for each network signalled value.

6.2B.3.1.4.3.1		Message contents exceptions (network signalled value "NS_04")

1.	Information element additionalSpectrumEmission is set to NS_04. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.1.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_04"

| Derivation Path: TS 38.508-1 [6] clause [TBD], Table [TBD] |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 4 (NS_04) |  |  |

6.2B.3.1.4.3.2		Message contents exceptions (network signalled value "NS_35")

1.	Information element additionalSpectrumEmission is set to NS_35. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.1.4.3.2-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_35"

| Derivation Path: TS 38.508-1 [5] clause [TBD], Table [TBD] |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_35) |  |  |

6.2B.3.1.5 	Test requirement

6.2B.3.1.5.1 	Test requirement for network signalled value “NS_35”

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in table 6.2B.3.1.5.1-1. The allowed A-MPR values specified in table 6.2B.3.1.3-1 are in addition to the allowed MPR requirements specified in clause 6.2B.1.1.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2B.1.1.3-1 apply.

Table 6.2B.3.1.5.1-1: UE Power Class test requirements for network signalled value "NS_35"

| Configuration ID | MPR (dB) | A-MPR (dB) | [ΔTC,c (dB)] | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 2 | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 3 | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| NOTE 1: FFS |  |  |  |  |  |  |  |

6.2B.3.1.5.2	Test requirement for network signalled value “NS_04”

Table 6.2B.3.1.5.2-1: UE Power Class test requirements for network signalled value "NS_04" for UEs not supporting dynamic power sharing with backoff applied independently

| Configuration ID | FIM3,low_block.low | A-MPRIM3(dB) | A-MPRNR (dB) | A-MPRLTE (dB) | TBD | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 2 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 3 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 4 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 5 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 6 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |

Table 6.2B.3.1.5.2-2: UE Power Class test requirements for network signalled value "NS_04" for UEs supporting dynamic power sharing with backoff applied equally to LTE and NR

| Configuration ID | FIM3,low_block.low | A-MPRIM3(dB) | A-MPR (dB) | TBD | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 2 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 3 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 4 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 5 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 6 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |

[End of changes]