3GPP TSG-RAN WG5 Meeting #3-5G-NR Adhoc	R5-185757

# Jaipur, India, 8th - 12th October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 0079 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 5GS MAC Test case 7.1.1.5.3 DRX operation / Short cycle configured / Parameters configured by RRC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  |  | Date: |  | 2018-10-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction to 5GS MAC Test case 7.1.1.5.3 DRX operation / Short cycle configured / Parameters configured by RRC. Updates to include SA Option-2 in same TC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated 7.1.1.5.3.2 Conformance requirements to 3GPP TS 38.321 V15.3.0, updated steps 1 and 2 in Table 7.1.1.5.3.3.2-1: Main behaviour and updated specific message contents to include SA Option-2 behavior |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be compliant to 3GPP TS 38.321 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |

<Start of changes>

##### 7.1.1.5.3	DRX operation / Short cycle configured / Parameters configured by RRC

# 7.1.1.5.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { Short DRX cycle and drx-SlotOffset is configured and [(SFN * 10) + subframe number] modulo drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle) }

then { UE starts the OnDurationTimer after drx-SlotOffset and monitors the PDCCH for OnDurationTimer PDCCH-subframes }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { drxShortCycleTimer is expired and [(SFN * 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset: }

then { UE starts the OnDurationTimer after drx-SlotOffset and monitors the PDCCH for OnDurationTimer PDCCH-subframes }

}

# 7.1.1.5.3.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.321, clause 5.7. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.7]

The MAC entity may be configured by RRC with a DRX functionality that controls the UE’s PDCCH monitoring activity for the MAC entity's C-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, and TPC-SRS-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other subclauses of this specification..When in RRC_CONNECTED, if DRX is configured, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this subclause; otherwise the MAC entity shall monitor the PDCCH continuously.

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX Cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL or DL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX Cycle starts;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When a DRX cycle is configured, the Active Time includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimerDL or drx-RetransmissionTimerUL or ra-ContentionResolutionTimer (as described in subclause 5.1.5) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in subclause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in subclause 5.1.4).

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment:

2>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if a MAC PDU is transmitted in a configured uplink grant:

2>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.

1>	if an drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL.

1>	if a DRX Command MAC CE or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer;

2>	stop drx-InactivityTimer.

1>	if drx-InactivityTimer expires or a DRX Command MAC CE is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer in the first symbol after the expiry of drx-InactivityTimer or in the first symbol after the end of DRX Command MAC CE reception;

3>	use the Short DRX Cycle.

2>	else:

3>	use the Long DRX cycle.

1>	if drx-ShortCycleTimer expires:

2>	use the Long DRX cycle.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer;

2>	use the Long DRX cycle.

1>	if the Short DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Long DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>		start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

1>	if the MAC entity is in Active Time:

2>	monitor the PDCCH;

2>	if the PDCCH indicates a DL transmission

3>	3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL or UL):

3>	start or restart drx-InactivityTimer in the first symbol after the end of the PDCCH reception.

1>	else in current symbol n, if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this subclause:

2>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7].

1>	if CSI masking (csi-Mask) is setup by upper layers:

2>	in current symbol n, if onDurationTimer would not be running considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this subclause:

3>	not report CSI on PUCCH.

1>	else

2>	in current symbol n, if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this subclause:

3>	not report CSI on PUCCH and semi-persistent CSI on PUSCH.

Regardless of whether the MAC entity is monitoring PDCCH or not, the MAC entity transmits HARQ feedback and aperiodic SRS defined in TS 38.214 [7] when such is expected.

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).

# 7.1.1.5.3.3	Test description

# 7.1.1.5.3.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

# 7.1.1.5.3.3.2	Test procedure sequence

For FDD, NormalSLT (current SFN, current sub-frame, current slot, y) = y; For TDD, NormalSLT (current SFN, current slot, y) counts the minimum number of normal slots needed to cover y number of PDCCH-occasions(slots) until next PDCCH-occasion(slot) available, starting from current slot on current Subframe.

Table 7.1.1.5.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | SS transmits NR RRCReconfiguration message to configure specific DRX parameters for PSCell | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits NR RRCReconfigurationComplete message | --> | RRCReconfigurationComplete | - | - |
| 3 | In the first PDCCH occasion, after the drx-SlotOffset when the drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. i.e., on the PDCCH occasion csn1 within the subframe number = (csfn1 + floor([ csn1+ NormalSLT(SFN1, csfn1,csn1, 0)] / numberofslotswithinsubframe)) modulo 10, and system frame number = SFN1 + floor([csfn1 + floor([csn1+ NormalSLT(SFN1, csfn1,csn1, 0)]/ numberofslotswithinsubframe)/10); where [(SFN1 * 10) + csfn1] modulo (ShortDRX-Cycle) = drx-StartOffset modulo (ShortDRX-Cycle); csn1=drx-slotoffset | <-- | MAC PDU | - | - |
| 4 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 3? | --> | HARQ ACK | 1 | P |
| 5 | At least drx-InactivityTimer PDCCH occasions after the transmission of the MAC PDU in Step 3 has been indicated (This means the next DRX cycle or later after Step 1) in the last PDCCH occasion while the drx-onDurationTimer is still running, the SS indicates the transmission a DL MAC PDU on the PDDCH. (Note). i.e., on the PDCCH occasion = [csn2 + NormalSLT(SFN2,csfn2,csn2,drx-onDurationTimer-1)] modulo numberofslotswithinsubframe within the subframe number = (csfn2+ floor([csn2 + NormalSLT(SFN2,csfn2,csn2,drx-onDurationTimer-1)] / numberofslotswithinsubframe)) modulo 10, and system frame number = SFN2 + floor([csfn2 + floor([csn2+ NormalSLT(SFN2,csfn2,drx-onDurationTimer-1)] /numberofslotspersubframe)]/10); where [(SFN2 * 10) + csfn2] modulo (ShortDRX-Cycle) = drx-StartOffset modulo (ShortDRX-Cycle) and csn2=drx-slotoffset. | <-- | MAC PDU | - | - |
| 6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5? | --> | HARQ ACK | 1 | P |
| 7 | UE waits for drx-ShortCycleTimer expire. | - | - | - | - |
| 8 | In the first PDCCH occasion after the drx-SlotOffset when the drx-onDurationTimer of drx-LongCycle is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. i.e., on the PDCCH occasion csn1 within the subframe number = (csfn1 + floor([ csn1+ NormalSLT(SFN1, csfn1,csn1, 0)] / numberofslotswithinsubframe)) modulo 10, and system frame number = SFN1 + floor([csfn1 + floor([csn1+ NormalSLT(SFN1, csfn1,csn1, 0)]/ numberofslotswithinsubframe)/10); where [(SFN1 * 10) + csfn1] modulo (LongDRX-Cycle) = drx-StartOffset; csn1=drx-slotoffset | <-- | MAC PDU | - | - |
| 9 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 8? | --> | HARQ ACK | 2 | P |
| 10 | At least drx-InactivityTimer PDCCH occasions after the transmission of the MAC PDU in Step 8 has been indicated (This means the next DRX cycle or later after Step 5) in the last PDCCH occasion while the drx-onDurationTimer is still running, the SS indicates the transmission a DL MAC PDU on the PDDCH. (Note). i.e., on the PDCCH occasion = [csn2 + NormalSLT(SFN2,csfn2,csn2,drx-onDurationTimer-1)] modulo numberofslotswithinsubframe within the subframe number = (csfn2+ floor([csn2 + NormalSLT(SFN2,csfn2,csn2,drx-onDurationTimer-1)] / numberofslotswithinsubframe)) modulo 10, and system frame number = SFN2 + floor([csfn2 + floor([csn2+ NormalSLT(SFN2,csfn2,drx-onDurationTimer-1)] /numberofslotspersubframe)]/10); where [(SFN2 * 10) + csfn2] modulo (LongDRX-Cycle) = drx-StartOffset and csn2=drx-slotoffset. | <-- | MAC PDU | - | - |
| 11 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 10? | --> | HARQ ACK | 2 | P |
| Note: The drx-InactivityTimer is started in the next PDCCH occasion of the PDCCH occasion where DL new transmission is indicated. |  |  |  |  |  |

# 7.1.1.5.3.3.3	Specific message contents

Table 7.1.1.5.3.3.3-1: RRCConnectionReconfiguration (step1, Table 7.1.1.5.3.3.2-1)

| Derivation Path: 36.508 [7], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.3.3.3-2: RRCReconfiguration (Table 7.1.1.5.3.3.3-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig |  | EN-DC |
| } |  |  |  |
| RRCReconfiguration-v1530-IEs::= SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig |  | FFS |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Editor’s note: Condition for SA needs to be defined in 38.508-1 and FFS will be updated accordingly.

Table 7.1.1.5.3.3.3-3: CellGroupConfig (Table 7.1.1.5.3.3.3-2)

| Derivation Path: 38.508-1 [4], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | [value] |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer | ms20 |  |  |
| drx-InactivityTimer | ms6 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms640 | 4 |  |  |
| } |  |  |  |
| shortDRX SEQUENCE { |  |  |  |
| drx-ShortCycle | ms64 |  |  |
| drx-ShortCycleTimer | 4 |  |  |
| } |  |  |  |
| drx-SlotOffset | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<END of changes>