3GPP TSG-RAN WG5 Meeting #3-5G-NR Adhoc	R5-185760

# Jaipur, India, 8th - 12th October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 0082 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 5GS PDCP Test case 7.1.3.4.1 PDCP handover / Lossless handover / PDCP sequence number maintenance / PDCP status report to convey the information on missing or acknowledged PDCP SDUs at handover / In-order delivery and duplicate elimination in the downlink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  |  | Date: |  | 2018-10-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction to 5GS PDCP Test case 7.1.3.4.1 to update conformance requirements and SA requirements. Merged Option-3 and Option-2 in same TC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated 7.1.3.4.1.2 Conformance requirements to 3GPP TS 38.323 V15.3.0, Updated steps 8,9, 23, 24 in Table 7.1.3.4.1.3.2-1: Main behaviour, Updated 7.1.3.4.1.3.3 Specific message contents. The same TC can now be applicable for both Option-3 and Option-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be compliant to 3GPP TS 38.323, SA Option 2 will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |

<Start>

##### 7.1.3.4.1	PDCP handover / Lossless handover / PDCP sequence number maintenance / PDCP status report to convey the information on missing or acknowledged PDCP SDUs at handover / In-order delivery and duplicate elimination in the downlink

# 7.1.3.4.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with default RB used RLC-AM mode }

ensure that {

when { UE is requested to make a lossless handover by SS }

then { UE creates a PDCP status report to SS }

}

(2)

with { UE in RRC_CONNECTED state with default RB used RLC-AM mode }

ensure that {

when { UE is requested to make a lossless handover by SS }

then { UE discards the corresponding PDCP PDU and PDCP SDU according to the PDCP status report from SS }

}

(3)

with { UE in RRC_CONNECTED state with default RB using RLC-AM }

ensure that {

when { UE is requested to make a lossless handover by SS }

then { UE retransmits the unacknowledged data }

}

(4)

with { UE in RRC_CONNECTED state with default RB using RLC-AM }

ensure that {

when { UE is requested to make a lossless handover by SS }

then { UE achieves in-order delivery and duplicate elimination in the downlink }

}

# 7.1.3.4.1.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.323, clauses 5.1.2, 5.2.2.1, 5.3, 5.4.1, 5.4.2 and 7.1. Unless otherwise stated these are Rel-15 requirements.

[TS 38.323, clause 5.1.2]

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this section. After performing the procedures in this section, the UE shall follow the procedures in subclause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the header compression protocol for uplink and start with an IR state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and SRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer, as specified in subclause 5.2.1;

-	for AM DRBs, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU as specified in the subclause 5.7.4;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the subclause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer, as specified in subclause 5.2.1.

When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

-	process the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, as specified in the subclause 5.2.2.1;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	for SRBs and UM DRBs, if t-Reordering is running:

-	stop and reset t-Reordering;

-	for UM DRBs, deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression;

-	for AM DRBs, perform header decompression for all stored PDCP SDUs if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the header compression protocol for downlink and start with NC state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and SRBs, set RX_NEXT and RX_DELIV to the initial value;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure.

[TS 38.323, clause 5.2.2.1]

In this section, following definitions are used:

-	HFN(State Variable): the HFN part (i.e. the number of most significant bits equal to HFN length) of the State Variable;

-	SN(State Variable): the SN part (i.e. the number of least significant bits equal to PDCP SN length) of the State Variable;

-	RCVD_SN: the PDCP SN of the received PDCP Data PDU, included in the PDU header;

-	RCVD_HFN: the HFN of the received PDCP Data PDU, calculated by the receiving PDCP entity;

-	RCVD_COUNT: the COUNT of the received PDCP Data PDU = [RCVD_HFN, RCVD_SN].

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows:

-	if RCVD_SN < SN(RX_DELIV) – Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) + 1.

-	else if RCVD_SN >= SN(RX_DELIV) + Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) – 1.

-	else:

-	RCVD_HFN = HFN(RX_DELIV);

-	RCVD_COUNT = [RCVD_HFN, RCVD_SN].

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU;

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	discard the PDCP Data PDU;

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

-	store the resulting PDCP SDU in the reception buffer;

-	if RCVD_COUNT >= RX_NEXT:

-	update RX_NEXT to RCVD_COUNT + 1.

-	if outOfOrderDelivery is configured:

-	deliver the resulting PDCP SDU to upper layers.

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering.

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.323, clause 5.3]

When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs.

NOTE:	Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard.

[TS 38.323, clause 5.4.1]

For AM DRBs configured by upper layers to send a PDCP status report in the uplink (statusReportRequired in TS 38.331 [3]), the receiving PDCP entity shall trigger a PDCP status report when:

-	upper layer requests a PDCP entity re-establishment;

-	upper layer requests a PDCP data recovery.

If a PDCP status report is triggered, the receiving PDCP entity shall:

-	compile a PDCP status report as indicated below by:

-	setting the FMC field to RX_DELIV;

-	if RX_DELIV < RX_NEXT:

-	allocating a Bitmap field of length in bits equal to the number of COUNTs from and not including the first missing PDCP SDU up to and including the last out-of-sequence PDCP SDUs, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first;

-	setting in the bitmap field as '0' for all PDCP SDUs that have not been received, and optionally PDCP SDUs for which decompression have failed;

-	setting in the bitmap field as '1' for all PDCP SDUs that have been received;

-	submit the PDCP status report to lower layers as the first PDCP PDU for transmission via the transmitting PDCP entity as specified in subclause 5.2.1..

[TS 38.323, clause 5.4.2]

For AM DRBs, when a PDCP status report is received in the downlink, the transmitting PDCP entity shall:

-	consider for each PDCP SDU, if any, with the bit in the bitmap set to '1', or with the associated COUNT value less than the value of FMC field as successfully delivered, and discard the PDCP SDU as specified in the subclause 5.3.

[TS 38.323, clause 7.1]

This sub clause describes the state variables used in PDCP entities in order to specify the PDCP protocol. The state variables defined in this subclause are normative.

All state variables are non-negative integers, and take values from 0 to [232 – 1].

PDCP Data PDUs are numbered integer sequence numbers (SN) cycling through the field: 0 to [2[pdcp-SN-Size] – 1].

The transmitting PDCP entity shall maintain the following state variables:

a)	TX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU to be transmitted. The initial value is 0.

The receiving PDCP entity shall maintain the following state variables:

a)	RX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU expected to be received. The initial value is 0.

b)	RX_DELIV

This state variable indicates the COUNT value of the first PDCP SDU not delivered to the upper layers, but still waited for. The initial value is 0.

c)	RX_REORD

This state variable indicates the COUNT value following the COUNT value associated with the PDCP Data PDU which triggered t-Reordering.

# 7.1.3.4.1.3	Test description

# 7.1.3.4.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 except the following:

-	2 NR cells (NR Cell 1 and NR Cell 2) are configured with SN terminated SCG bearers in RLC AM mode.

-	The cell power levels are configured as per the Table 7.1.3.4.1.3.1-1.

# Table 7.1.3.4.1.3.1-1: Time instances of cell power level

IF [pc_EN_DC] or [pc_NGEN_DC] and FR1 FDD THEN

|  | Parameter | Unit | EUTRA Cell 1 | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | - |  |  |
|  | SS/PBCH SSS EPRE | dBm/15kHz | - |  |  |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | - |  |  |
|  | SS/PBCH SSS EPRE | dBm/15kHz | - | [-85] | [-79] |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | - |  |  |
|  | SS/PBCH SSS EPRE | dBm/15kHz | - | [-79] | [-85] |  |

IF [pc_EN_DC] or [pc_NGEN_DC] and FR1 TDD THEN

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | FFS | FFS |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | FFS | FFS |  |

IF [pc_EN_DC] or [pc_NGEN_DC] and FR2 THEN

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | FFS | FFS |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | FFS | FFS |  |

# 7.1.3.4.1.3.2	Test procedure sequence

# Table 7.1.3.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS creates 5 PDCP Data PDUs and the TX_NEXT is set to "0". | - | - | - | - |
| - | EXCEPTION: Step 2 and 4 shall be repeated for k=0 to 1(increment=1). | - | - | - | - |
| 2 | The SS sends the PDCP Data PDU#k via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = k. After having sent a PDU, the SS set TX_NEXT = k+1. | <-- | PDCP PDU DATA #k | - | - |
| 3 | The UE sends the PDCP Data PDU#k via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = k. Data is previously received data from PDU #k. (Note 1) | --> | PDCP PDU DATA #k | - | - |
| 4 | Wait for the expiry of t-PollRetransmit. | - | - | - | - |
| - | EXCEPTION: Step 5 and 6 shall be repeated for m=2 to 4 (increment=1). | - | - | - | - |
| 5 | The SS sends the PDCP Data PDU #m via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN =m. After having sent a PDU, the SS set TX_NEXT = m+1. | <-- | PDCP PDU DATA #m | - | - |
| 6 | The UE sends the PDCP Data PDU#m via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = m. Data is previously received data from PDU #m. (Note 2) | --> | PDCP PDU DATA #m | - | - |
| 7 | The SS changes NR Cell 2 parameters according to the row "T1" in table 7.1.3.4.1.3.2.0-1. | - | - | - | - |
| 8 | The SS requests UE to make a handover to NR Cell 2 with the NR RRCReconfiguration message sent on NR Cell 1 with key change. | <-- | RRCReconfiguration | - | - |
| 9 | The UE transmits a NR RRCReconfigurationComplete message. (Note 3) | --> | RRCReconfigurationComplete | - | - |
| 10 | The SS assigns UL grant during the Random Access procedure on NR Cell 2 to allow the UE to send only PDCP status report. | - | - | - | - |
| 11 | Check: Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? | --> | PDCP STATUS REPORT | 1 | P |
| 12 | The SS generates a PDCP status report message and sends it to UE: D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 3. | <-- | PDCP STATUS REPORT | - | - |
| 13 | Configure the SS to allocate Default UL grant to the UE in NR Cell 2. | - | - | - | - |
| 14 | Check: Does the UE send PDU#2? Note: This is to check if UE is discarding PDU#2 based on Status report from SS. | --> | PDCP DATA PDU#2 | 2 | F |
| - | EXCEPTION: Step 15 shall be repeated for m=3 to 4 (increment=1). | - | - | - | - |
| 15 | Check: Does the UE send the PDCP Data PDU #m via RLC-AM RB with the following content to the SS: D/C field = 1 (PDCP Data PDU) and PDCP SN = m? Note: Data is previously received data from PDU #m. | --> | PDCP PDU DATA #m | 3 | P |
| 16 | The SS sends the PDCP Data PDU#5 via RLC-AM RB with the following content to the UE: PDCP Data PDU #5 ( D/C field = 1 (PDCP Data PDU) and PDCP SN=5). | <-- | PDCP DATA PDU#5 | - | - |
| 17 | The UE transmits a PDCP Data PDU via RLC-AM RB with the following content back to the SS: D/C field = 1 (PDCP Data PDU) and PDCP SN=5. Note: Data is previously received packet in PDCP Data PDU#5. (Note 1) | --> | PDCP DATA PDU #5 | - | - |
| 18 | TX_NEXT is set to "6". The SS creates a PDCP Data PDU#6 (not transmitted). | - | - | - | - |
| 19 | The TX_NEXT is set to "7". The SS creates a PDCP Data PDU #7. | - | - | - | - |
| 20 | The SS sends PDCP Data PDU#7 via RLC-AM RB with the following content to the UE: PDCP Data PDU#7; D/C field = 1 (PDCP Data PDU) and PDCP SN=7. | <-- | PDCP DATA PDU #7 | - | - |
| 21 | Check: Does the UE transmit a PDCP DATA PDU#7? | --> | PDCP DATA PDU#7 | 4 | F |
| 22 | The SS changes NR Cell 1 and NR Cell 2 parameters according to row "T2" in Table 7.1.3.4.1.3.1-1. | - | - | - | - |
| 23 | The SS requests UE to make a handover to NR Cell 1 with the NR RRCReconfiguration sent on NR Cell 2 with key change. | <-- | RRCReconfiguration | - | - |
| 24 | The UE transmits a NR RRCReconfigurationComplete message. (Note 3) | --> | RRCReconfigurationComplete | - | - |
| 25 | SS assigns UL grant during the Random Access procedure on NR Cell 1 to allow the UE to send only PDCP status report. | - | - | - | - |
| 26 | The UE sends PDCP Control PDUs via RLC-AM RB with the following content to the SS: D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 6, Bitmap = 0x80. | --> | PDCP STATUS REPORT | - | - |
| 27 | The SS generates a PDCP status report message and sends it to UE: D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 6. | <-- | PDCP STATUS REPORT | - | - |
| 28 | Configure the SS to allocate Default UL grant to the UE in NR Cell 1 | - | - | - | - |
| 29 | The SS sends the PDCP Data PDU#6 via RLC-AM RB with the following content to the UE: PDCP Data PDU#6 ( D/C field = 1 (PDCP Data PDU) and PDCP SN=6). | <-- | PDCP DATA PDU #6 | - | - |
| 30 | Check: Does the UE transmit a PDCP Data PDU via RLC-AM RB with the following content back to the SS? D/C field = 1 (PDCP Data PDU) and PDCP SN=6 Note: Data is previously received packet in PDCP Data PDU#6 | --> | PDCP DATA PDU #6 | 4 | P |
| 31 | Check: Does the UE transmit PDCP Data PDU via RLC-AM RB with the following content back to the SS? D/C field = 1 (PDCP Data PDU) and PDCP SN=7 Note: Data is previously received packet in PDCP Data PDU#7 | --> | PDCP DATA PDU #7 | 4 | P |
| Note 1: The SS acknowledges the received data. Note 2: SS doesn’t send the RLC ACK for this data. Note 3: UE will perform RACH procedure on NR Cell which will happen in parallel to steps 9 and 24 |  |  |  |  |  |  |

# 7.1.3.4.1.3.3	Specific message contents

# Table 7.1.3.4.1.3.3-0: SchedulingRequest-Config (Preamble)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-117 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| sr-TransMax | n64 |  |  |

# Table 7.1.3.4.1.3.3-1: RRCConnectionReconfiguration (steps 8, 23, Table 7.1.3.4.1.3.2-1)

| Derivation Path: 36.508 [7], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup. |  |  |
| } |  |  |  |
| } |  |  |  |
| sk-Counter-r15 | Different counter value used before Handover. |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

# Table 7.1.3.4.1.3.3-2: RRCReconfiguration (steps 8, 23, Table 7.1.3.4.1.3.2-1)

| Derivation Path: 38.508-1 [4], Table: 4.6.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig |  | FFS |
| secondaryCellGroup | CellGroupConfig |  | EN-DC |
| } |  |  |  |
| RRCReconfiguration-v1530-IEs::= SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig |  | FFS |
| masterKeyUpdate ::= SEQUENCE { |  |  | FFS |
| keySetChangeIndicator | True |  | FFS |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Editor’s note: Condition for SA needs to be defined in 38.508-1 and FFS will be updated accordingly.

# Table 7.1.3.4.1.3.3-3: CellGroupConfig (Table 7.1.3.4.1.3.3-2)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF SEQUENCE { | 1 entry |  | EN-DC, FFS |
| RLC-Bearer-Config[1] | RLC-Bearer-Config |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Serving cell Index of NR Cell X | NR cell 2 at step 8 NR cell 1 at step 23 |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon | ServingcellInfocommon | NR cell 2 at step 8 NR cell 1 at step 23 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Editor’s note: Condition for SA needs to be defined in 38.508-1 and FFS will be updated accordingly.

Table 7.1.3.4.1.3.3-4: RLC-Bearer-Config (Table 7.1.3.4.1.3.3-3)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-110 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Bearer-Config ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | LogicalChannelIdentity for NR DRB |  |  |
| servedRadioBearer CHOICE { |  |  |  |
| drb-Identity | 2 FFS | SCG DRB Id | EN-DC, DRB ID FFS for Condition FFS |
| } |  |  |  |
| reestablishRLC[1] | True |  |  |
| } |  |  |  |

Editor’s note: Condition for SA needs to be defined in 38.508-1 and FFS will be updated accordingly.

# Table 7.1.3.4.1.3.3-4: ServingCellConfigCommon (Table 7.1.3.4.1.3.3-3)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| physCellId | PhysCellId |  |  |
| downlinkConfigCommon ::= SEQUENCE { |  |  |  |
| frequencyInfoDL | FrequencyInfoDL | NR cell 2 at step 8 NR cell 1 at step 23 |  |
| } |  |  |  |
| } |  |  |  |

# Table 7.1.3.4.1.3.3-5: RadioBearerConfig (Table 7.1.3.4.1.3.3-1)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { | 1 entry |  | EN-DC |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | 6 | Dedicated EPS bearer Id of SCG DRB |  |
| sdap-Config | Not present |  |  |
| } |  |  |  |
| drb-Identity | 2 | SCG DRB Id |  |
| reestablishPDCP | True |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| securityConfig SEQUENCE { |  |  |  |
| keyToUse | s-KgNB |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { | 1 entry |  | FFS |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | Not present |  |  |
| sdap-Config | FFS |  |  |
| } |  |  |  |
| drb-Identity | FFS |  |  |
| reestablishPDCP | True |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |

Editor’s note: Condition for SA needs to be defined in 38.508-1 and FFS will be updated accordingly.

# Table 7.1.3.4.1.3.3-7: PDCP-Config (Table 7.1.3.4.1.3.3-5)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-74 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| statusReportRequired | True |  |  |
| } |  |  |  |
| } |  |  |  |

<END of changes>