3GPP TSG-RAN WG5 Meeting #3-5G-NR Adhoc	R5-185765

# Jaipur, India, 8th - 12th October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 0087 | rev | -- | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Idle Mode TC - Steering of UE in roaming during registration/security check successful using List Type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  |  | Date: |  | 2018-09-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of 5GS Idle Mode TC 38.523-1 6.3.1.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of new Test Case as part of Idle SA Option 2 WP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SA Option 2 WP implementation will not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

##### 6.3.1.1	Steering of UE in roaming during registration/security check successful using List Type 0

# 6.3.1.1.1	Test Purpose (TP)

(1)

with {UE being in automatic PLMN selection mode, current VPLMN not part of “PLMNs where registration was aborted due to SOR” List and not part of "User Controlled PLMN Selector with Access Technology" list and UE's USIM configured with indication that the UE is to receive Steering of Roaming information due to initial registration in a VPLMN}

ensure that {

when {SOR Transparent container indicates ACK has been requested & List Type indicates secured packet in REGISTRATION ACCEPT and security check is successful}

then {UE sends ACK in REGISTRATION COMPLETE, waits for network release of the NAS signalling connection and selects higher priority PLMN}

}

# 6.3.1.1.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 23.122, clauses C1-184911 , C1-185396. Unless otherwise stated these are Rel-15 requirements.

[TS 23.122, clause C.2]

The VPLMN AMF to the UE: The VPLMN AMF shall transparently send the received steering of roaming information and the indication that the UDM requests an acknowledgement from the UE (if included by the HPLMN UDM), to the UE in the REGISTRATION ACCEPT message;

7)	If the list of preferred PLMN/access technology combinations is received and the security check is successful, then:

a)	if the SOR transparent container contains a secured packet (see 3GPP TS 31.115 [67]), the ME shall upload the secured packet to the USIM using procedures in 3GPP TS 31.111 [41].

NOTE 1:	How the ME handles UICC responses and failures in communication between the ME and UICC is implementation specific and out of scope of this release of the specification.

When the ME receives a USAT REFRESH command qualifier (3GPP TS 31.111 [41]) of type "Steering of Roaming" it performs the procedure for steering of roaming in subclause 4.4.6 but it does not act as if timer T has expired;

b)	otherwise, the ME shall replace the highest priority entries in the "Operator Controlled PLMN Selector with Access Technology" list stored in the ME with the received list of preferred PLMN/access technology combinations. Additionally, if the UDM has not requested an acknowledgement from the UE and if the UE has a list of available and allowable PLMNs in the area and based on this list the UE determines that there is a higher priority PLMN than the currently camped chosen VPLMN and the UE is in automatic network selection mode, then the UE shall either:

i)	release the current N1 NAS signalling connection locally, abort the ongoing registration procedure and then attempt to obtain service on a higher priority PLMN as specified in subclause 4.4.3.3 by acting as if timer T that controls periodic attempts has expired. In this case, steps 8 to 11 are skipped; or

ii)	not release the current N1 NAS signalling connection locally, and complete the registration procedure as specified in 3GPP TS 24.501 [64], and skip steps 8 to 10;

8)	If:

a)	the UE's USIM is configured with indication that the UE is to receive the steering of roaming information due to initial registration in a VPLMN , but neither the list of preferred PLMN/access technology combinations nor the HPLMN indication that 'no change of the "Operator Controlled PLMN Selector with Access Technology" list stored in the UE is needed and thus no list of preferred PLMN/access technology combinations is provided' is received in the REGISTRATION ACCEPT message, or if the steering of roaming information is received but the security check is not successful; and

b)	the current chosen VPLMN is not contained in the list of "PLMNs where registration was aborted due to SOR", not part of "User Controlled PLMN Selector with Access Technology" list , the UE is not in manual mode of operation and the PDU session for emergency services is not pending to be activated;

then the UE shall release the current N1 NAS signalling connection locally, store the PLMN identity in the list of "PLMNs where registration was aborted due to SOR" and attempt to obtain service on a higher priority PLMN as specified in subclause 4.4.3.3 by acting as if timer T that controls periodic attempts has expired, with an exception that the current PLMN is considered as lowest priority, and skip steps 9 to 12;

NOTE 2:	When the UE is in the manual mode of operation or the current chosen VPLMN is part of the "User Controlled PLMN Selector with Access Technology" list, the UE stays on the VPLMN.

9)	The UE to the VPLMN AMF: If the UDM has requested an acknowledgement from the UE and the UE verified that the steering of roaming information  has been provided by the HPLMN in step 6, the UE sends the REGISTRATION COMPLETE message to the serving AMF with a transparent container including the UE acknowledgement;

10)	The AMF to the UDM: The AMF sends a Nudm_SDM_Info request message. If a transparent container with a UE acknowledgement was received in the REGISTRATION COMPLETE message, the AMF includes the transparent container in the Nudm_SDM_Info request message. If the HPLMN decided that the UE is to acknowledge the successful security check of the received steering of roaming information in step 4, the UDM verifies that the acknowledgement is provided by the UE as specified in 3GPP TS 33.501 [66];

11)	If the UE has a list of available PLMNs in the area and based on this list the UE determines that there is a higher priority PLMN than the currently camped chosen VPLMN and the UE is in automatic network selection mode, then the UE shall attempt to obtain service on a higher priority PLMN as specified in subclause 4.4.3.3 by acting as if timer T that controls periodic attempts has expired after the release of the N1 NAS signalling connection. If the N1 NAS signaling connection is not released after implementation dependent time, the UE may locally release the N1 signaling connection; and

12)	The UE deletes the list of "PLMNs where registration was aborted due to SOR".

# 6.3.1.1.3	Test Description

6.3.1.1.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN cells as specified in TS 38.508-1 clause [FFS - Default NAS parameters for simulated NAS cells need to be defined] are configured broadcasting PLMNs as indicated in Table 6.3.1.1.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 6.3.1.1.3.1-1.

Table 6.3.1.1.3.1–1: PLMN identifiers

| Cell | PLMN name | MCC | MNC |
| --- | --- | --- | --- |
| 12 | PLMN2 | 002 | 11 |
| 13 | PLMN3 | 003 | 21 |
| 14 | PLMN4 | 004 | 31 |

-	cell 12 is set to ''Serving cell''[FFS – Need definition of “Serving Cell” in 38.508-1];

-	cell 13 is set to ''Serving cell'';

-       cell 14 is set to “Serving cell'';

UE:-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 38.508-1[FFS – Need to define default value of Elementary Files (EFs) in USIM]) except for those listed in Table 6.3.1.1.3.1-2.

-    The UE is in Switched OFF (state X [FFS- Need to define State ID for switch off state]) according to [38.508-1]

Table 6.3.1.1.3.1–2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFOPLMNwACT | 1 2 3 | PLMN4 PLMN3 PLMN2 Remaining defined entries use default values | NG-RAN NG-RAN NG-RAN |
| EFUST |  | Service n°127 is "available" |  |
| EFSteering_of_UE_in_VPLMN |  | 1 (steering of UE in VPLMN during registration supported) |  |

# 6.3.1.1.4	Test procedure sequence

# Table 6.3.1.1.4-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - |  | - | - | - | - |
| 1 |  | <-- | RRC: SYSTEM INFORMATION (BCCH) | - | - |
| 2 | Power on the UE | - |  | - | - |
| 3-14 | Steps 1 to 12 of the registration procedure described in TS 38.508 subclause 4.5.2 are performed on Cell 14 | - | - | - | - |
| 15 | The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing steering of roaming information indicating list of preferred PLMN/access technology combination provided as a secured packet with acknowledgment requested from the UE for successful reception (Note1) | <-- | RRC: DLInformationTransfer NAS: REGISTRATION ACCEPT | - | - |
| 16 | The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message carrying acknowledgement of successful reception of the steering of roaming information | --> | RRC: ULInformationTransfer NAS: REGISTRATION COMPLETE | 1 | P |
| 17 | The SS transmits an RRCRelease message. | <-- | RRC: RRCRelease | - | - |
| 18 | Check: Does the UE transmits an RRCSetupRequest on Cell 12 within the next 6min? | --> | RRC: RRCSetupRequest | 1 | P |
| 19 | Steps [FFS – Need generic Test procedure for type mobility registration updating procedure] of the generic test procedure in TS 38.508 subclause [FFS] are performed on Cell 12. NOTE: The UE performs a " REGISTRATION REQUEST procedure with type "mobility registration updating” and the RRC connection is released | - | - | - | - |
| Note 1: How the ME handles UICC responses and failures in communication between the ME and UICC is implementation specific and out of scope of this release of the specification |  |  |  |  |  |

# 6.3.1.1.5	Specific message contents

# Table 6.3.1.1.5-1: REGISTRATION ACCEPT for Cell 14 (step 15, Table 6.3.1.1.4-1)

| Derivation Path: 38.508-1 [FFS – Need Registration Accept – SOR Container default] |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR data type | 0 | The SOR transparent container carries steering of roaming information. |  |
| List indication value | 1 | List of preferred PLMN/access technology combinations is provided |  |
| List type | 0 | The list type is a secure packet |  |
| Acknowledgement (ACK) value | 1 | Acknowledgement requested |  |
| PLMN ID 1 | 2 |  |  |
| Access Technology Identifier 1 | NG-RAN |  |  |

# Table 6.3.1.1.5-2: REGISTRATION COMPLETE for Cell 14 (step 16, Table 6.3.1.1.4-1)

| Derivation Path: 38.508-1 [FFS – Need Registration Complete – SOR Container default] |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR data type | 1 | The SOR transparent container carries acknowledgement of successful reception of the steering of roaming information. |  |

<End of Changes>