3GPP TSG-RAN5#3-5G-NR Adhoc	R5-185870

# Jaipur, India, 8th – 12th October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0013 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to EN-DC test case 6.2B.2.2, UE Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  |  | Date: |  | 2018-10-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Test case 6.2B.2.2 is incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added information to empty test case 6.2B.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 6.2B.2.2 will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Beginning of changes>>

##### 6.2B.2.2	UE Maximum Output Power reduction for Intra-Band Non-Contiguous EN-DC

Editor’s note: The following aspects are either missing or not yet determined:

- Initial condition is not complete.

- Minimum requirements is TBD (38.101-3)

- Test requirement is TBD

- Test tolerance is not complete.

- Wgap not defined

6.2B.2.2.1 		Test purpose

Editor’s Note: Explanatory text is needed.

6.2B.2.2.2 	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band EN-DC.

6.2B.2.2.3 	Minimum conformance requirements

TBD

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2B.2.2.

6.2B.2.2.4 	Test description

6.2B.2.2.4.1		Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and test channel bandwidths based on NR operating bands specified in clause 5.3B.1.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2B.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521.1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.2B.2.2.4.1-1: Test configuration table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | TBD |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | TBD |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [6] subclause 4.3.1 |  | TBD |  |
| Test SCS as specified in TS 38.521-1 [8] Table 5.3.5-1 |  | TBD |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for MPR testing | Modulation | RB allocation |
| 1 |  | TBD | TBD |
| 2 |  | TBD | TBD |
| NOTE 1: The specific configuration of each RB allocation is defined in Table [TBD]. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6], Figure A.3.1.1.1 for SS diagram and section A.3.2.1 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] subclause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0 and TS 38.521-1 [8] Annex C.0 for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H and TS 38.521-1 [8] Annex G for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 6.2B.2.2.4.3.

6.2B.2.2.4.2		Test procedure

1.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format [0_1] for C_RNTI to schedule the UL RMC according to Table 6.2.2.4.1-1 and Table 6.2.2.4.1-2 of TS 38.521-1[8] for UE power class 3 and UE power class 2 respectively. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	E-UTRA SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.2.3.3-1 and Table 6.2.3_1.3-1 of TS 36.521-1[10] for UE power class 3 and UE power class 2 respectively. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms for the UE to reach PUMAX level.

4.	Measure the mean power over all component carriers for the EN-DC configuration, which shall meet the requirements described in table 6.2B.2.2.5-1. The period of the measurement shall be at least the continuous duration of [one active sub-frame].

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.5B.2.1.2.4.1-1 and 6.5B.2.1.2.4.1-2, send an RRCConnectionReconfiguration message containing NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 table 4.6.3-89 PUSCH-Config without CP-OFDM condition. When switching to CP-OFDM waveform, send an RRCConnectionReconfiguration message containing NR RRCReconfiguration message with CP-OFDM condition.

6.2B.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] subclause 4.6.1.

6.2B.2.2.5 	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in table 6.2B.2.2.5-1.

Table 6.2B.2.2.5-1: UE Power Class test requirements

TBD

## << End of changes>>