3GPP TSG-RAN5#3-5G-NR Adhoc	R5-185912

# Jaipur, India, 8th – 12th October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 0205 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to clause 4.3.3, physical channel allocations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  |  | Date: |  | 2018-10-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 4.3.3 is incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added new table for downlink physical channels and physical signals - Removed chapter 4.3.3.2.4 and 4.3.3.2.5. Correponding chapters in 36.508 were never filled with information and therefore, it’s not expected to be needed in 38.508-1 either. - Changed default value of physical cell identity to zero (aligned withTable 4.4.2-2) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Clause 4.3.3 will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Beginning of changes>>

### 4.3.3	Physical channel allocations

#### 4.3.3.1	E-UTRA

The same physical channel allocations as in TS 36.508 [2] clause 4.3.3 applies.

#### 4.3.3.2	NR

##### 4.3.3.2.1	Antennas

For FR1 testing, if the UE has two or four Rx antennas, the same downlink signal is applied to each antenna. All UE Rx antennas shall be connected unless otherwise stated in the test case.

##### 4.3.3.2.2	Downlink physical channels and physical signals

Table 4.3.3.2.2-1: Power allocation for OFDM symbols and reference signals

| Parameter | Unit | Value |
| --- | --- | --- |
| SSS transmit power | W | Test specific (Note 1) |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS | dB | 0 |
| EPRE ratio of PBCH to PBCH DMRS | dB | 0 |
| EPRE ratio of PDCCH DMRS to SSS | dB | 0 |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |
| EPRE ratio of PDSCH DMRS to SSS | dB | 0 |
| EPRE ratio of PDSCH to PDSCH DMRS | dB | 0 |
| EPRE ratio of CSI-RS to SSS | dB | 0 |
| EPRE ratio of PTRS to PDSCH | dB | 0 |
| Note 1: Power level chosen to align with cell power level as specified in clause 6.2.2. |  |  |

##### 4.3.3.2.3	Mapping of downlink physical channels and signals to physical resources

Parameters for mapping of downlink physical channels and signals are specified as follows.

Normal Cyclic Prefix

, Physical layer cell identity	= 0 is used as the default physical layer cell identity

SCS, Subcarrier spacing = 15 kHz for FR1 FDD (the same SCS is used for synch and data)

SCS, Subcarrier spacing = 30 kHz for FR1 TDD (with SCS for synch 30 kHz for TDD bands supporting it otherwise using SCS for synch 15 kHz), except for band n34 and n51, where SCS = 15 kHz (the same SCS is used for synch and data)

SCS, Subcarrier spacing = 120 kHz for FR2 TDD (the same SCS is used for synch and data)

For Signalling testing, the default channel bandwidth is specified in clause 6.2.3.1 for each NR band.

For Signalling testing, single SS Tx antenna is used, in FR1, unless specified otherwise in the test case.

For RF testing, the mapping of DL physical channels to resource element is defined in Annex C of TS 38.101-1 [7] and TS 38.101-2 [8] and TS 38.101-3 [9].

## << End of changes>>