3GPP TSG-RAN5 Meeting #3	R5-185914

# Jaipur, India, 8th Oct 2018 - 12th Oct 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0021 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to 7.3B.2.2 - REFSENS for Intra-band Non-Contiguous EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Tech. Netherlands B.V |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2018-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Core spec provides requirements for EN-DC intra-band non-contiguous REFSENS test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test content for 7.3B.2.2 in TS 38.521-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN-DC REFSENS for intra-band non-contiguous EN-DC will not be available in test spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Several unchanged sections skipped>

##### 7.3B.2.2	Reference sensitivity for Intra-band non-contiguous EN-DC

Editor’s note: The following aspects are either missing or not yet determined:

-	The test point selection analysis is incomplete:

-	Working assumption: E-UTRA is not tested during test procedure

-	Test configuration needs further investigation

-	Test tolerance analysis is incomplete

##### 7.3B.2.2.1	Test purpose

To verify the ability of UE that support intra-band non-contiguous EN-DC to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.  A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area near to an e-NodeB or a gNB.

##### 7.3B.2.2.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band contiguous EN-DC.

##### 7.3B.2.2.3	Minimum conformance requirements

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

Editor’s note: FFS how to clarify the issues of 1Tx may also exist for 2Tx mode, for example harmonic, etc.

The normative reference for this requirement is TS 38.101-3 [4] Clause 7.4B.1

##### 7.3B.2.2.4	Test Description

7.3B.2.2.5	Initial Condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.2B.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2.All of these configurations shall be tested with applicable test parameters for each intra-band contiguous EN-DC configuration specified in clause 5.3B.1.2, and are shown in table 7.4B.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.3B.2.2.5-1: Test configuration table [TBD]

| Initial Conditions |  |
| --- | --- |
| Test Environment as specified in TS 38.508-1 [6] subclause 4.1 | TBD |
| Test Frequencies as specified in TS 38.508-1 [6] subclause 4.3.1 | TBD |
| Test CC Combinations setting (NRB_agg) as specified in TS 38.508-1 [6] subclause 4.3.1 | TBD |
| Test SCS for the NR cell as specified in 38.521-1 [8] Table 5.3.5-1 | TBD |
| Test Parameters for EN-DC Configuration |  |
| FFS |  |

1.	Connect the SS to the UE antenna connectors as shown in [TBD].

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0, C.1,C.2,C3.1 and TS 38.521-1 [8] Annex C.0,C.1,C.2,C3.1for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H.0,H.1,H.2,H.3.1 and TS 38.521-1 [8] Annex G.0,G.1,G.2,G.3.1 for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.4B.1.4.3.

7.3B.2.2.6	Test Procedure

1.	TBD

7.3B.2.2.7	Message Contents

Message contents are according to TS 38.508-1 [6] subclause 4.6.1.

##### 7.3B.2.2.8	Test Requirement

For the NR CC, the throughput measurement of the carrier derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex [TBD] with parameters specified in Table [TBD]

<End of changes>