3GPP TSG RAN WG5 #3-5G-NR Adhoc

Jaipur, India, 8th – 12th October 2018

Source:		Rohde & Schwarz

Title:	Discussion on test point selection for EVM in FR2

Document for:	Endorsement

# Introduction

At RAN#78, it was decided that for each NR test case a discussion of test point selection regarding frequency, channel BW, SCS, RB allocation and waveform is required and that the analysis is documented in TR 38.905.

In this contribution the selection of frequency, channel BW, SCS, RB allocation and waveform for the Error Vector Magnitude test case in 38.521-2 is analyzed.

# Error Vector Magnitude

## Discussion

The Error Vector Magnitude (EVM) is a figure of merit for the transmit modulation quality and is defined in section 6.4.2.1 and Annex F Transmit Modulation of TS 38.101-2 [1].

The minimum requirement is specified as follows:

6.4.2.1		Error vector magnitude

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM, the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further equalised using the channel estimates subjected to the EVM equaliser spectrum flatness requirement specified in sub-clauses 6.4.2.4 and 6.4.2.5. For DFT-s-OFDM waveforms, the EVM result is defined after the front-end FFT and IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %. For CP-OFDM waveforms, the EVM result is defined after the front-end FFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %.

The basic EVM measurement interval in the time domain is one preamble sequence for the PRACH and the duration of PUCCH/PUSCH channel, or one hop, if frequency hopping is enabled for PUCCH and PUSCH in the time domain. The EVM measurement interval is reduced by any symbols that contains an allowable power transient as defined in subclause 6.3.3.

The RMS average of the basic EVM measurements for the average EVM case, and for the reference signal EVM case, for the different modulations schemes shall not exceed the values specified in Table 6.4.2.1-1 for the parameters defined in Table 6.4.2.1-2 or Table 6.4.2.1-3 depending on UE power class.  The measurement interval for the EVM determination is 10 subframes. The requirement is verified with the test metric of EVM (Link=TX beam peak direction, Meas=Link angle).

Table 6.4.2.1-1: Minimum requirements for error vector magnitude

| Parameter | Unit | Average EVM level | Reference signal EVM level |
| --- | --- | --- | --- |
| Pi/2 BPSK | % | 30.0 | 30.0 |
| QPSK | % | 17.5 | 17.5 |
| 16 QAM | % | 12.5 | 12.5 |
| 64 QAM | % | 8.0 | 8.0 |

Table 6.4.2.1-2: Parameters for Error Vector Magnitude for power class 1

| Parameter | Unit | Level |
| --- | --- | --- |
| UE EIRP | dBm | 4 |
| UE EIRP for UL 16QAM | dBm | 7 |
| UE EIRP for UL 64QAM | dBm | 11 |
| Operating conditions |  | Normal conditions |

Table 6.4.2.1-3: Parameters for Error Vector Magnitude for power class 2, 3, and 4

| Parameter | Unit | Level |
| --- | --- | --- |
| UE EIRP | dBm | -13 |
| UE EIRP for UL 16QAM | dBm | -10 |
| UE EIRP for UL 64QAM | dBm | -6 |

## Test Environment

In LTE, EVM is tested applying normal condition as test environment. Since for FR2 no significant dependence of the EVM on temperature or voltage is expected, using the same test environment as for LTE is a reasonable approach.

Proposal 1: Define test environment as Normal for EVM in FR2.

## Test Subcarrier Spacing

In this section the selection of the subcarrier spacing to be tested is analyzed. The subcarrier spacing determines the spectrum efficiency as well as the symbol duration. On the one hand, the maximum transmission bandwidth, i.e. largest spectrum efficiency, is achieved with the lowest subcarrier spacing setting a tighter requirement on the modulation performance in the frequency domain. On the other hand, a large subcarrier spacing results in short symbols which need to be well defined in the time domain to achieve a high signal quality. In order to cover both cases, EVM is to be tested with lowest and highest SCS.

Proposal 2: Select lowest and highest SCS for EVM in FR2.

## Test Frequencies

In LTE, EVM is tested for Low range, Mid range, and High range. Due to the large bandwidth of NR bands, the same test coverage as for LTE is needed to ensure that the EVM is not deteriorated by frequency dependent transmission and intermodulation properties of the RF chain of the UE, e.g., those of the PA.

Proposal 3: Test EVM for Low range, Mid range, and High range frequencies in FR2.

## Test Channel Bandwidth

In LTE, EVM is tested for Lowest, 5 MHz and Highest channel bandwidths. This assumption seems to be reasonable to test different behaviors of the UE transmitter related to different frequency selectivity with respect to different channel bandwidth at first sight. However, in order to achieve test time reduction, we believe that mid channel bandwidth can be omitted as it does not pose an extreme case.

Proposal 4: Test EVM for Lowest and Highest channel bandwidth in FR2.

## Test Modulations and waveforms for EVM PUSCH

The minimum requirements are specified for Pi/2-BPSK, QPSK, 16 QAM, and 64 QAM. In order to ensure that the UE meets all these requirements the test has to cover these. Due to the differences in the modulation scheme (with and without precoding) and due to the different CREST factors for DFT-s-OFDM and CP-OFDM both waveforms should be tested.

Proposal 5: Test EVM PUSCH in FR2 for the modulation waveform combinations stated in Table 2.6-1.

Table 2.6-1: Modulation and waveform combinations

DFT-s-OFDM PI/2 BPSK

DFT-s-OFDM QPSK

DFT-s-OFDM 16 QAM

DFT-s-OFDM 64 QAM

CP-OFDM QPSK

CP-OFDM 16 QAM

CP-OFDM 64 QAM

## RB allocation for EVM PUSCH

In LTE, EVM is measured with full allocation and the highest partial allocation where no MPR occurs. The modulations should be tested with outer_full and inner_full allocation to consider the different MPR since EVM depends on the power level.

Proposal 6: Test EVM PUSCH with the modulation, waveform and RB allocation combinations comprised in Table 2.7-1.

Table 2.7-1: Modulation, waveform and RB combinations

| Test ID | Uplink Configuration |  |
| --- | --- | --- |
|  | Modulation | RB allocation (NOTE 1) |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer Full |
| 3 | DFT-s-OFDM QPSK | Inner Full |
| 4 | DFT-s-OFDM QPSK | Outer Full |
| 5 | DFT-s-OFDM 16 QAM | Inner Full |
| 6 | DFT-s-OFDM 16 QAM | Outer Full |
| 7 | DFT-s-OFDM 64 QAM | Inner Full |
| 8 | DFT-s-OFDM 64 QAM | Outer Full |
| 9 | CP-OFDM QPSK | Inner Full |
| 10 | CP-OFDM QPSK | Outer Full |
| 11 | CP-OFDM 16 QAM | Inner Full |
| 12 | CP-OFDM 16 QAM | Outer Full |
| 13 | CP-OFDM 64 QAM | Inner Full |
| 14 | CP-OFDM 64 QAM | Outer Full |

## Test Modulations, waveforms and formats for EVM PUCCH

Depending on the PUCCH format QPSK or PI/2- BPSK are applied as modulation. In LTE, for FDD format 1a and for TDD format 1a / 1b are tested. The same approach should be followed in 5G.

Due to the differences in the modulation scheme (with and without precoding) and due to the different CREST factors for DFT-s-OFDM and CP-OFDM both waveforms should be tested.

Proposal 7: Test EVM PUCCH for DFT-s-OFDM and CP-OFDM with Format 1a / 1b.

## Downlink configuration for EVM PUCCH

In LTE, QPSK is applied as modulation for the downlink and RB allocation is partial. However, the EVM of PUCCH should not depend on the downlink allocation and therefore a specification of the downlink allocation is not required.

Proposal 8: Do not specify a downlink allocation for EVM PUCCH.

## Configuration for EVM PRACH

In LTE, for TDD and FDD specific PRACH preamble formats are applied.

For FDD, the PRACH configuration index is 4 resulting in preamble format 0 with 1.25 kHz subcarrier spacing for the random access preamble, any system frame number and a subframe number equal to 4. Applying the same in 5G requires a PRACH Configuration index of 17.

For LTE TDD, the PRACH configuration index is 53 resulting in preamble format 4 with 7.5 kHz subcarrier spacing. For 5G a subcarrier spacing of 5 kHz is possible by applying preamble format 3. This is achieved with PRACH Configuration Index = 52. Compared to LTE the RS EPRE setting has been changed to take the different subcarrier spacing into account.

Proposal 9: Test EVM PRACH with the preamble format of Table 2.9-1.

Table 2.9-1: EVM PRACH preamble format

| PRACH preamble format |  |  |
| --- | --- | --- |
|  | FDD | TDD |
| PRACH Configuration Index | 17 | 52 |
| RS EPRE setting for test point 1 (dBm/15kHz) | -71 | -65 |
| RS EPRE setting for test point 2 (dBm/15kHz) | -86 | -80 |

# Conclusion

To sum up, the following selection of test points is proposed for EVM in FR2:

Proposal 1: Define test environment as Normal for EVM in FR2.

Proposal 2: Select lowest and highest SCS for EVM in FR2.

Proposal 3: Test EVM for Low range, Mid range, and High range frequencies in FR2.

Proposal 4: Test EVM for Lowest and Highest channel bandwidth in FR2.

Proposal 5: Test EVM PUSCH in FR2 for the modulation waveform combinations stated in Table 2.6-1.

Table 2.6-1: Modulation and waveform combinations

DFT-s-OFDM PI/2 BPSK

DFT-s-OFDM QPSK

DFT-s-OFDM 16 QAM

DFT-s-OFDM 64 QAM

CP-OFDM QPSK

CP-OFDM 16 QAM

CP-OFDM 64 QAM

Proposal 6: Test EVM PUSCH with the modulation, waveform and RB allocation combinations comprised in Table 2.7-1.

Table 2.7-1: Modulation, waveform and RB combinations

| Test ID | Uplink Configuration |  |
| --- | --- | --- |
|  | Modulation | RB allocation (NOTE 1) |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer Full |
| 3 | DFT-s-OFDM QPSK | Inner Full |
| 4 | DFT-s-OFDM QPSK | Outer Full |
| 5 | DFT-s-OFDM 16 QAM | Inner Full |
| 6 | DFT-s-OFDM 16 QAM | Outer Full |
| 7 | DFT-s-OFDM 64 QAM | Inner Full |
| 8 | DFT-s-OFDM 64 QAM | Outer Full |
| 9 | CP-OFDM QPSK | Inner Full |
| 10 | CP-OFDM QPSK | Outer Full |
| 11 | CP-OFDM 16 QAM | Inner Full |
| 12 | CP-OFDM 16 QAM | Outer Full |
| 13 | CP-OFDM 64 QAM | Inner Full |
| 14 | CP-OFDM 64 QAM | Outer Full |

Proposal 7: Test EVM PUCCH for DFT-s-OFDM and CP-OFDM with Format 1a / 1b.

Proposal 8: Do not specify a downlink allocation for EVM PUCCH.

Proposal 9: Test EVM PRACH with the preamble format of Table 2.9-1.

Table 2.9-1: EVM PRACH preamble format

| PRACH preamble format |  |  |
| --- | --- | --- |
|  | FDD | TDD |
| PRACH Configuration Index | 17 | 52 |
| RS EPRE setting for test point 1 (dBm/15kHz) | -71 | -65 |
| RS EPRE setting for test point 2 (dBm/15kHz) | -86 | -80 |

# Number of test points

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for NR FR2.

EVM PUSCH:

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 | 2 | 2 | 14 | 168 |

EVM PUCCH:

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (format and waveform) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 | 2 | 2 | 2 | 24 |

EVM PRACH:

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (RS EPRE) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 | 2 | 2 | 2 | 24 |

# References

- R4-1811910, Big CR for 38.101-2, 3GPP TSG RAN4, September 2018