3GPP TSG RAN WG5 #3-5G-NR Adhoc

Jaipur, India, 8th – 12th October 2018

Source:		Rohde & Schwarz

Title:	Discussion on test point selection for EVM equalizer spectral flatness in FR2

Document for:	Endorsement

# Introduction

At RAN#78, it was decided that for each NR test case a discussion of test point selection regarding frequency, channel BW, SCS, RB allocation and waveform is required and that the analysis is documented in TR 38.905.

In this contribution the selection of frequency, channel BW, SCS, RB allocation and waveform for the EVM equalizer spectrum flatness test case is analyzed.

# Discussion

The EVM equalizer spectrum flatness is a measure for the ripple inside the transmission bandwidth of an UE and is defined in clauses 6.4.2.4 and 6.4.2.5 of TS 38.101-2 [1].

The minimum requirement is specified as follows:

#### 6.4.2.4	EVM equalizer spectrum flatness

The EVM measurement process (as described in Annex F) entails generation of a zero-forcing equalizer. The EVM equalizer spectrum flatness is defined in terms of the maximum peak-to-peak ripple of the equalizer coefficients (dB) across the allocated uplink block. The basic measurement interval is the same as for EVM.

For BPSK modulation waveforms, the minimum requirements are defined in Clause 6.4.2.5.

The peak-to-peak variation of the EVM equalizer coefficients contained within the frequency range of the uplink allocation shall not exceed the maximum ripple specified in Table 6.4.2.4-1 for normal conditions. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirements: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 (Table 6.4.2.4-1) must not be larger than 7 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8 dB (see Figure 6.4.2.4-1).

The requirement is verified with the test metric of EVM SF (Link=TX beam peak direction, Meas=Link angle).

Table 6.4.2.4-1: Minimum requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency range | Maximum ripple (dB) |
| --- | --- |
| FUL_Meas – FUL_Low ≥ X MHz and FUL_High – FUL_Meas ≥ X MHz (Range 1) | 6 (p-p) |
| FUL_Meas – FUL_Low < X MHz or FUL_High – FUL_Meas < X MHz (Range 2) | 9 (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: FUL_Low and FUL_High refer to channel edges NOTE 3: X, in MHz, is equal to 20% of the CC bandwidth |  |

Table 6.4.2.4-2: (Void)

Figure 6.4.2.4-1: The limits for EVM equalizer spectral flatness with the maximum allowed variation of the coefficients indicated under normal conditions

#### 6.4.2.5	EVM spectral flatness for pi/2 BPSK modulation with spectrum shaping

These requirements are defined for pi/2 BPSK modulation. The EVM equalizer coefficients across the allocated uplink block shall be modified to fit inside the mask specified in Table 6.4.2.5-1 for normal conditions, prior to the calculation of EVM. The limiting mask shall be placed to minimize the change in equalizer coefficients in a sum of squares sense.

Table 6.4.2.5-1: Mask for EVM equalizer coefficients for pi/2 BPSK with spectrum shaping, normal conditions

| Frequency range | Parameter | Maximum ripple (dB) |
| --- | --- | --- |
| F_meas – F_center ≤ X MHz or F_center – F_meas ≤ X MHz (Range 1) | X1 | 6 (p-p) |
| F_meas – F_center > X MHz or F_center – F_meas < X MHz (Range 2) | X2 | 14 (p-p) |
| NOTE 1: F_meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: F_center refers to the center frequency of an allocated block of PRBs NOTE 3: X, in MHz, is equal to 25% of the bandwidth of the PRB allocation NOTE 4: See Figure 6.4.2.5-1 for description of X1, X2 and X3 |  |  |

Figure 6.4.2.5-1: The limits for EVM equalizer spectral flatness with the maximum allowed variation. F_center denotes the center frequency of the allocated block of PRBs. F_alloc denotes the bandwidth of the PRB allocation.

This requirement does not apply to other modulation types. The UE shall be allowed to employ spectral shaping for pi/2 BPSK. The shaping filter shall be restricted so that the impulse response of the transmit chain shall meet

│ãt(t,0)│ ≥ │ãt(t, τ)│    ∀τ ≠ 0

20log10│ãt(t,τ)│< -15 dB    1< τ < M - 1,

Where:

│ãt(t,τ)│=IDFT{│ãt(t,f)│ejφ (t,f)} ,

f  is the frequency of the M allocated subcarriers,

ã(t,f) and φ(t,f) are the amplitude and phase response, respectively of the transmit chain

0dB reference is defined as 20log10│ãt(t,0)│

## Test Environment

In LTE, EVM equalizer spectrum flatness is tested under extreme conditions. As there is no minimum conformance requirement for extreme conditions for NR FR2, EVM equalizer spectrum flatness should be tested with normal conditions.

Proposal 1: Define test environment as Normal for EVM equalizer spectrum flatness in FR2.

## Test Subcarrier Spacing

In order to maximize the tested bandwidth the smallest SCS is selected as it provides the highest spectrum efficiency.

Proposal 2: Select smallest SCS for EVM equalizer spectrum flatness in FR2.

## Test Frequencies

In LTE, EVM equalizer spectrum flatness is tested for Low range, Mid range, and High range. Due to the large width of NR bands and the frequency dependent amplitude and phase response of an UE it is reasonable to follow the LTE approach for NR.

Proposal 3: Test EVM equalizer spectrum flatness for Low range, Mid range, and High range frequencies in FR2.

## Test Channel Bandwidth

In LTE, EVM equalizer spectrum flatness is tested for Lowest, 5 MHz and Highest channel bandwidths. Due to the influence of channel bandwidth on the amplitude and phase response of an UE it is reasonable to follow the LTE approach for NR.

Proposal 4: Test EVM equalizer spectrum flatness for Lowest, Mid and Highest channel bandwidth in FR2.

## Test Modulations and waveforms

In LTE EVM equalizer spectrum flatness is tested with QPSK modulation at maximum output power. In NR for FR2 different requirements exist for Pi/2-BPSK (clause 6.4.2.5) and the other modulations (clause 6.4.2.4). For test case 6.4.2.5 the only possible choice is DFT-s-Pi/2-BPSK.

Due to the differences in the modulation scheme (with and without precoding) and due to the different CREST factors for DFT-s-OFDM and CP-OFDM both waveforms should be tested. Select QPSK as a modulation for test case 6.4.2.4 analog to LTE.

Proposal 5: Test EVM equalizer spectrum flatness with CP-OFDM QPSK and DFT-s-OFDM QPSK for test case 6.4.2.4 and with DFT-s-OFDM Pi/2-BPSK for test case 6.4.2.5.

## RB allocation

In LTE, EVM equalizer spectrum flatness is tested with full allocation to test all equalizer coefficients. The same approach is followed for NR.

Proposal 6: Test EVM equalizer spectrum flatness with Outer Full allocation.

# Conclusion

To sum up, the following selection of test points is proposed for EVM equalizer spectrum flatness in FR2:

Proposal 1: Define test environment as Normal for EVM equalizer spectrum flatness in FR2.

Proposal 2: Select smallest SCS for EVM equalizer spectrum flatness in FR2.

Proposal 3: Test EVM equalizer spectrum flatness for Low range, Mid range, and High range frequencies in FR2.

Proposal 4: Test EVM equalizer spectrum flatness for Lowest, Mid and Highest channel bandwidth in FR2.

Proposal 5: Test EVM equalizer spectrum flatness with CP-OFDM QPSK and DFT-s-OFDM QPSK for test case 6.4.2.4 and with DFT-s-OFDM Pi/2-BPSK for test case 6.4.2.5.

Proposal 6: Test EVM equalizer spectrum flatness with Outer Full allocation.

# Number of test points

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for NR FR2.

Test case 6.4.2.4

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 | 3 | 1 | 2 | 18 |

Test case 6.4.2.5

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 | 3 | 1 | 1 | 9 |

# References

- R4-1811910, Big CR for 38.101-2, 3GPP TSG RAN4, September 2018